3GPP TSG-RAN4 Meeting #114bis	R4-2503274

# Wuhan, China, 7th Apr 2025 - 11th Apr 2025

Agenda item:			7.11.1 General aspects

Source:	Eutelsat Group

Title:	Draft CR for Operating bands including priority 2 cases

Document for:	Agreement

# Introduction

This contribution provides draft CRs to 38.101-5 on the band plan implementing current decisions and proposals for the priority 2 cases with uplink band A.

# Discussion

Annex A provides an implementation of the decisions from RAN #114 in [1].

Annex B provides further changes to define the priority 2a and 2b bands for uplink block A. Changes from Annex B are highlighted in yellow relative to Annex A. Annex B also adds a missing note for Region 2 to limit the downlink frequency range to 10700  MHz. – 12700 MHz.

In the current agreement, Note 4 states:

NOTE 4:	For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.

This is a necessary note for the US but is not sufficient to fully specify the bands for Region 2. A more general note is needed to limit the Downlink in region 2 to 12700 MHz:

NOTE :	For Region 2, the downlink is limited to 10700 – 12700 MHz.

It is proposed to add one new band per numerology to cover cases 2a and 2b. This is done using an additional note:

NOTE 6:	For Regions 1 and 3, the uplink is limited to 12750 – 13250 MHz

# Conclusions

Proposal 1: Adopt Annex A as the baseline for section 5 based on the WF in [1] at RAN4 #114.

Proposal 2: Adopt Annex B  for section 5 based on the addition of priority 2a and 2b bands for uplink block A and to fully specify bands 247, 248, 508 and 509 for Region 2.

# References

[1] R4-2502290 “WF for [114]{313] NR_NTN_Ku_Band_General” (Eutelsat Group) RAN4 #114 Feb 2025

Annex A Draft CR for decisions at RAN4 #114 in [1]

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present Release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future Releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

## 5.2	Operating bands

### 5.2.1	General

NTN satellite covers FR1-NTN and FR2-NTN operating bands in the present specification.

### 5.2.2	Operating bands with conducted requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

### 5.2.3	Operating bands with radiated requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1.

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512(Note 1) | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n511(Note 2) | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n510(Note 3) | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

Table 5.2.3-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

## 5.3	UE channel bandwidth

Annex B Draft CR to include priority 2 cases and corrections

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present Release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future Releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

## 5.2	Operating bands

### 5.2.1	General

NTN satellite covers FR1-NTN and FR2-NTN operating bands in the present specification.

### 5.2.2	Operating bands with conducted requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

### 5.2.3	Operating bands with radiated requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1.

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512(Note 1) | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n511(Note 2) | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n510(Note 3) | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504,5 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504,5 MHz | FDD |
| n507 | 127006 MHz - 13250 MHz | 10700 MHz – 127504,5 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 6: For Regions 1 and 3, the uplink is limited to 12750 – 13250 MHz. |  |  |  |

Table 5.2.3-2: Satellite operating bands in FR1-NTN

| Satellite operating band |  | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high |  | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high |  | Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n248 |  | 14000 MHz - 14500 MHz |  | 10700 MHz – 127501,2 MHz |  | FDD |  |  |  |
| n247 |  | 13750 MHz - 14000 MHz |  | 10700 MHz – 127501,2 MHz |  | FDD |  |  |  |
| n246 | 127003 MHz - 13250 MHz | 10700 MHz – 127501,2 MHz | FDD |  |
| NOTE 1: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 2: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 3: For Regions 1 and 3, the uplink is limited to 12750 – 13250 MHz. |  |  |  |  |  |  |  |  |  |

## 5.3	UE channel bandwidth