3GPP TSG RAN WG4 Meeting #114bis	R4-2503288

# WuHan, China, 07 April – 11 April, 2025

Agenda item:			7.17.2.2

Source:	vivo

Title:	Remaining issues on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

- At RAN4 114 meeting, the following agreements are available for this WI:

- Issue 1-1: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement:

  - Option 1 is agreed

- Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 1-3-1: Tentative solution 1: measure one serving CC per band

Agreement:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

- Note: Following the WID, the above agreement applies for the measurement without gap.

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

Support Solution 3 in Rel-19:

- FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only.

  - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC).

  - For FR1+FR2 scenario, PCell is FR1

  - FFS whether NR-DC is included or not

Design for solution 3 (agreed in ad-hoc session)

- Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future.

    - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

      - the 3rd searcher resource sharing is same as the second searcher sharing.

Not support solution 2 in Rel-19.

Issue 1-3-4: Others

Agreement:

  - This issue is closed

- In this contribution we discuss the following open issues:

    - Issue 1-2: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Option 1 (Nokia, vivo): To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

- Option 2 (ZTE, CMCC, Ericsson): The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios.

Consider the practice in the field option 1 is preferred.

Proposal 1: For issue 1-2, option 1 is preferred.

Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

Support Solution 3 in Rel-19:

- FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only.

  - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC).

  - For FR1+FR2 scenario, PCell is FR1

  - FFS whether NR-DC is included or not

Design for solution 3 (agreed in ad-hoc session)

- Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future.

    - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

      - the 3rd searcher resource sharing is same as the second searcher sharing.

Not support solution 2 in Rel-19.

Regarding whether to further consider NW indication on a particular or a few particular SCCs to be measured by using the 3rd searcher, the consideration is that the solution should have consideration on the impact on UE implementation. Currently for measurement on different SCCs, the assumption is each individual SCC shares same searcher resource and the order to perform SCC measurement is up to UE implementation. If the network indication indicates a group of SCC share resource A and the other SCC share resource B, this will bring noticeable change on UE implementation. On the other hand considering a particular deployment scenario where measurement for a particular SC needs be speed up whereas there is no such demands on the rest of SCCs, since the 3rd searcher will be introduced in this WI, it maybe possible to utilize this 3rd searcher to speed up the measurement of a particular SCC.

Proposal 2: For issue 1-3, impact on UE implementation needs be considered to determine whether network indication method should be considered or not.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: For issue 1-2, option 1 is preferred.

Proposal 2: For issue 1-3, impact on UE implementation needs be considered to determine whether  network indication method should be considered or not.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 113, Apple