3GPP TSG RAN WG4 Meeting #114bis	   R4-2503289

# WuHan, China, 07 April – 11 April, 2025

Agenda item:			7.17.2.1

Source:	vivo

Title:	Remaining issues on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

- At RAN4 114 meeting, the following agreements are available:

- Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in ad-hoc session:

- Discuss if additional conditions are needed to activate FBS-based L3 measurements.

  - Do not consider additional conditions beside the agreements from RAN4#113 meeting.

Agreement in RRM session (Monday):

- Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold.

  - Further discuss whether to limit the serving cell to FR2 serving cell.

  - Further discuss whether to add the hysteresis and timeToTrigger parameters.

  - Note: the scenario related to single carrier or CA/DC is discussed in another issue.

- The signaling design is up to RAN2.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement in ad-hoc session:

- UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4.

Further discuss this issue.

Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement in RRM session (Monday):

- No need to add additional processing time in the delay requirement for scenario 5.

- Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement

Agreement:

- Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

In this contribution we discuss the following remaining issues for this topic.

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, following issues are captured in the WF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

For this issue, the open issues are listed in the agreement of RAN4 114 below.

- Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold.

  - Further discuss whether to limit the serving cell to FR2 serving cell.

  - Further discuss whether to add the hysteresis and timeToTrigger parameters.

  - Note: the scenario related to single carrier or CA/DC is discussed in another issue.

For the serving cell, prefer to limit the serving cell to a FR2 serving cell.

Proposal 1: FBS triggering condition of serving cell is worse than a threshold , prefer to limit the serving cell to a FR2 serving cell.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

  - Option 1 (NTT DCM, CMCC, ZTE):

    - UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE.

    - Option 1a (ZTE): The consideration of UE mobility status based triggering condition has been excluded in last meeting, so neither support to take this as additional quit condition.

  - Option 2a: (NTT DCM)

    - Only the UE indication due to overheating can be considered for scenario 1/2/3/4 based on Rel-18 UAI ‘multiRx-PreferenceFR2’.

  - Option 2b: (LGE, MTK)

    - Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused to quit multi-Rx operation for L3 measurement.

  - Option 3: (LGE, HW, vivo, Samsung, Intel)

    - Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

    - Option 3a: (Intel)

      - The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the good serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the good quality criteria are no longer met.

  - Option 4: (ZTE)

    - NW configuration of the threshold RSRP and/or RSRQ triggers the UE monitoring, as long as receiving the threshold, UE starts to monitor the serving cell quality to determine when to activate/deactivate the fast L3 measurement. If not any threshold configured to UE, which implies NW does not expect the UE to perform fast L3 measurement.

  - Option 5: (CATT)

    - RAN4 to consider the serving cell’s signal quality threshold and a validity timer as the quit conditions.

  - Option 6a: (OPPO)

    - UE indication to the NW can be used to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction. The detailed signaling can leave to RAN2 decision.

  - Option 6b: (CT)

    - UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

  - Option 7: (Ericsson)

    - UE shall report deactivating FBS to NW upon deactivation condition being met.

    - RAN4 to study the below FBS deactivation conditions:

      - After UE activates FBS:

        - The condition of activating FBS is invalid again.

          - To avoid too frequent ping-pong between FBS and non-FBS, a minimal during time in FBS/ non-FBS shall be defined.

        - The condition of activating FBS still is valid,

          - L3 measurement with FBS has conducted for a time period, or

            - As an example, the condition of activating FBS still is valid, but the UE is keeping a stationary at cell edge or there’s no qualified neighbour cell to switch. In this case, FBS may be disabled.

          - ﻿Internal overheating is detected.

      - Before UE activates FBS, i.e., before meeting FBS activation condition:

        - Internal overheating is detected.

  - Option 9: (Nokia)

    - UE may exit FBS mode if the condition to enter FBS mode is no longer met.

  - Option 10 (CATT, Nokia)

    - Send LS to RAN2 about the criteria/thresholds for triggering/deactivating Multi-Rx operation on L3 measurements and the corresponding UE behaviours after the whole framework is decided.

- Among these options, option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

- Proposal 2: Option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- Proposals

  - Option 1a: (ZTE, CATT, Ericsson, CT, CMCC)

    - Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

  - Option 1b: (QC)

    - At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS.

  - Option 1c: (MTK)

    - On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either:

      - Single value for reduced BSF (e.g., 6 or 4)

      - Multiple candidate values for BSF (e.g., 6 and 4)

  - Option 2: (vivo)

    - RAN4 to consider the following approaches as solutions:

      - Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

      - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

      - Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

  - Option 3: (HW)

    - The UE capability design shall allow enough flexibility on supporting this feature considering:

      - Different BC

      - Number of Carriers/MOs to measured

      - Intra-frequency/inter-frequency

  - Option 4: (LGE)

    - Introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

- Recommended WF

  - Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). FFS the value(s).

  - Discuss option 3 in regard to the UE capability design:

    - Different BC

    - Number of Carriers/MOs to measured

    - Intra-frequency/inter-frequency

- Proposal 3: For the layer 3 measurement delay reduction, introducing the UE capability of reduced N, candidate 2, 4 and 6 is ok.

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

- Proposals

  - Option 1a: (NTT DCM, Ericsson)

    - For each scenario, the highlighted factors should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

  - Option 1b: (ZTE, OPPO)

    - For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability.

  - Option 1c: (CATT)

    - The reduced Rx BSF can be applied to the following formula according to the trigger conditions to be specified:

      - TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

      - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

      - Tsearch for FR1-FR2 HO and FR2-FR2 HO;

      - Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

      - Tidentify-NR for RRC connection release with redirection to NR.

    - The reduced Rx BSF can apply to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without new specification impact.

  - Option 2: (LGE)

    - For L3 measurement, introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

      - Table. Example of Proposed measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Pmulti-Rx x Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

  - Option 3: (Intel)

    - Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

    - When the criterion is met, the Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms, and Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms; where TSSB_processing = 2 ms, where TPSS/SSS_sync + TSSB_measurement_period is cut in half.

  - Option 4: (Nokia)

    - For L3 measurement delay requirements, cover the impact of Rel-19 BSF reduction through an additional scaling factor NFBS = [reduced BSF]/8, where the reduced BSF depends on UE capability.

    - For HO delay requirements, define the value for N in Tsearch depending on UE capability for BSF reduction.

    - For RRC re-establishment delay, o apply the scaling factor NFBS to the fixed value of Tidentify_intra/inter_NR e.g., NFBS x MAX (1000 ms, 104 x TSMTC, i), where NFBS = [reduced BSF]/8.

    - Capture the applicability of L3 FBS requirements in section 3.6.

  - Option 5: (Samsung)

    - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline.

  - Option 6: (MTK)

    - The following delay requirements/parameters of L3 measurement can be reduced by optimizing beam sweeping factor:

      - For scenarios (1/2/3/4): Mpss/sss_sync and Mmeas_period used in intra/inter-frequency (w/o gap) in FR2

      - For scenario (5): N in Tsearch used in handover delay when target cell is unknown in FR2

      - For scenario (7): Tidentify_intra and Tidentify_inter used to scale RRC Re-establishment/RRC Connection Release

- Recommended WF

  - Discuss this issue using option 1 as starting point.

- It is ok with the recommended WF, i.e., using option 1 as the starting point.

- Proposal 4: For issue 1-2-2, using option 1 as the starting point.

Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement

- Proposals

  - Option 1: (Apple, NTT DCM, CATT, OPPO, Ericsson, Nokia, vivo)

    - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

  - Option 2: (ZTE)

    - Define the transition requirement and follow the traditional L3 measurement transition requirement, i.e. use the longer delay between the two modes.

- Recommended WF

  - If option 1 is agreeable?

- Proposal 5: For the transition between normal L3 measurement and FBS L3 measurement, prefer option 1, do not define transition requirement.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

Proposal 1: FBS triggering condition of serving cell is worse than a threshold , prefer to limit the serving cell to a FR2 serving cell.

- Proposal 2: Option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

- Proposal 3: For the layer 3 measurement delay reduction, introducing the UE capability of reduced N, candidate 2, 4 and 6 is ok.

- Proposal 4: For issue 1-2-2, using option 1 as the starting point.

- Proposal 5: For the transition between normal L3 measurement and FBS L3 measurement, prefer option 1, do not define transition requirement.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4 114, CATT