# 3GPP TSG-RAN WG4 Meeting #114bis  	                               R4-2503294

WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	On RRM requirements for common signal or channel adaptation in R19 NES

Agenda Item:	7.25.2.2

Document for:	Discussion

Introduction

In RAN4 #113 and RAN4 114, the WFs [1], [2] are approved.

Based on all above information, we provide our views on RRM requirements for common signal or channel adaptation in R19 NES.

At RAN1 120 meeting the following agreements are available for SSB adaptation.

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

At RAN2 129 meeting the following agreements related to SSB adaptation are available:

RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Send response LS to RAN4 (Cc: RAN1) to inform RAN2 preference and to ask if RAN4 has any concern.

For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

The agreement above will be also included in the response LS.

Discussion

In this contribution we discuss the following open topics:

- Sub-topic 1-1 SSB adaptation – Scenario

Issue 1-1-2: Scenario clarification- SSB periodicity

- Proposals

  - Option 1: RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements (Apple, Nokia)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

Since based on previous RAN1 reply LS the the periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms and there is no new value compared with legacy SSB burst periodicity, option 1 should be used.

Proposal 1: For SSB periodicity of SSB adaptation, support option 1, i.e., consider legacy SSB burst periodicity as starting point to define the requirements.

- Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-4: L1 requirements impact after/without transition

- Proposals

  - Option 1 (Measurement priority with relation to DRX): For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.  (Samsung)

  - Option 2 (Measurement behaviour at transition): UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation (Samsung)

- Recommended WF:

  - Discuss above proposals.

For L1 requirement, the following agreements have been achieved at RAN4 114 meeting,

Issue 1-2-1: Applicable L1 requirements for SSB adaptation

Agreement:

- No impact of SSB adaptation:

  - RLM

  - BFD

- Define requirements if SSB adaption happens:

  - L1-RSRP for serving cell

  - L1-SINR

  - CBD

- Whether and how to define requirements at transition is discussed in other issues.

Requirements will be defined for L1-RSRP for serving cell, L1-SINR and CBD and their corresponding legacy requirements are copied below:

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3  P  PCBD)  TSSB) |
| DRX cycle > 320ms | Ceil(3  P  PCBD)  TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

Regarding option 1, we prefer the requirement using the same principle as that of legacy, i.e., keep (TDRX,TSSB) when DRX cycle≤320 ms. For option 2, since in RAN1 reply LS it mentions “There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation”, we are ok with option 2.

Proposal 2: For SSB adaptation issue 1-2-4, prefer keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell, L1-SINR requirement. Option 2 is ok.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – general

Agreement:

Deactivated SCell measurement:

- Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements.

  - At least for the case when the SSB periodicity is adapted to be shorter than before

- Option 2: Same as legacy

- Other options are not precluded.

The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

For the L3 requirement impacts, RAN4 gets further RAN2’s reply in LS [R2-2501484] and the content is copied below:

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Regarding OD-SSB case#2, RAN4 has the following agreements in RAN4 114 meeting.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation

Agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

Issue 2-3: Combine OD-SSB and AO-SSB in measurement

Agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

Since in RAN2’s LS it mentions that “RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC, in addition the maximum SSB periodicity is 160ms which is equal to the minimum value of measCycleSCell, hence option 2 should be used.

Proposal 3: For L3 requirements for SSB adaptation, support option 2 “same as legacy” for deactivated SCell measurement.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: For SSB periodicity of SSB adaptation, support option 1, i.e., consider legacy SSB burst periodicity as starting point to define the requirements.

Proposal 2: For SSB adaptation issue 1-2-4, prefer keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell, L1-SINR requirement. Option 2 is ok.

Proposal 3: For L3 requirements for SSB adaptation, support option 2 “same as legacy” for deactivated SCell measurement.

- References

[1] R4-2420242, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4 #113

[2] R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #114