# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503349

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.29.4.2

Source: 	OPPO

Title: 	On other requirements of NR NTN Phase 3

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 part 2 was approved [1].

In this paper, we will provide our views on the remaining issues for other requirements.

Discussion

- Topic #1: SSB periodicity larger than 20 ms ## Issue 1-1: Cell Reselection Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed ## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded. ## Issue 1-3: RRC connection release with redirection to NR (default Trs) Agreement: - In RRC connection release with redirection to NR requirements, no change to the existing Trs. And the requirement does not apply if SMTC configuration or measurement object on this frequency is not provided, from Rel-19. ## Issue 1-4: Handover and Satellite switch with re-sync (default Trs) Agreement: - In handover and satellite switch with re-sync requirements, no change to the existing Trs. And the requirement does not apply if SMTC configuration or measurement object on this frequency is not provided, from Rel-19 ## Issue 1-5: Paging Further discussion: - Further discuss whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - Option 1 (Huawei, vivo): no update is needed - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5 ## Issue 1-6: SMTC configuration Conclusion: No discussion in this meeting.

We also have the observations according to current spec, as follows:

- For RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

- For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

- For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

RAN4 has agreed for RRC connection release with redirection to NR and for Handover and Satellite switch with re-sync, no change to the existing Trs. And the requirement does not apply if SMTC configuration or measurement object on this frequency is not provided, from Rel-19.

However, for handover and Satellite switch with re-sync, no agreements are reached in last meeting for case where the SMTC for the inter-frequency carrier is not provided. Since it is for inter-frequency measurement, we can assume the worst case of the longest periodicity as 160ms. We are ok to update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. By-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. To be honest, there may exist some initial access performance loss if SMTC is not provided.

Proposal 1: For RRC Re-establishment, RAN4 to update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

If proposal 1 was agreed, we should know that network and UE should accept possible performance loss due to longer initial access delay. As a compromised solution, it can also works similarly as other cases that the requirement does not apply when the SMTC is not provided.

Proposal 2: As a compromised solution, the requirement does not apply for case where the SMTC for the inter-frequency carrier is not provided.

RAN4 discussed the impact of SSB periodicity extension (e.g., 320ms) in last meeting. IF RAN1/2 concluded on these, the related measurement requirements may need to be updated, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

However, no consensus is reached for T/Rx beam sweeping. We think DTX/DRX, Dynamic and flexible power sharing could have limited impacts on RRM requirements of NTN. RAN4 can start the work after RAN1/2 reached more concrete conclusion. RAN4 can continue to hold on these discussions in this meeting.

Proposal 3: RAN4 to hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping until any progress in other WGs.

Conclusions

In this paper we provided our views as follows.

Proposal 1: For RRC Re-establishment, RAN4 to update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

Proposal 2: As a compromised solution, the requirement does not apply for case where the SMTC for the inter-frequency carrier is not provided.

Proposal 3: RAN4 to hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping until any progress in other WGs.

Reference

- R4-2502604, WF on RRM requirements for Rel-19 NR NTN phase3, CATT