3GPP TSG-RAN4 Meeting #114bis	R4-2503370

# Wuhan, China, 7th- 11th, Apr, 2025

Source:	Samsung, Verizon

Title:	TP for TR 38.750 CA_n2-n48 with PC2 FDD single UL

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to include CA_n2-n48 according to the request in [1].

PC3 is already in spec.

2. Reference

[1] RP-250806 Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n2-n48

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz)(NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n48A | n28 | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |

NOTE 8:   Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

There is near miss of 2nd harmonic of n2 PC2 UL falling into n48 DL.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n2

Table 5.x.2.1-1: Reference sensitivity exceptions of harmonic

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n48 | 5 | 15 | 25 | 5 | 10.6 | 13.1 | NOTE 6 | UL2/DL1 near-miss |
| NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with 1 Tx. NOTE 8: Applicable to UEs supporting PC2 with 2 Tx. |  |  |  |  |  |  |  |  |  |

### 5.x.3	∆TIB and ∆RIB values (Void)

<End of Text Proposal>