3GPP TSG-RAN WG4 Meeting # 114bis   	R4-2503373

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.12.4

Source: 	CMCC

Title:			  Discussion on RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In last meeting, the impacted features have been identified and most of requirements have been discussed and defined for Air-to-ground network enhancement, the WF has been approved [1]. In this contribution, we further discuss UE capability issues and potential enhancement considering ATG specific CA scenario.

- Discussion

Issue 1-1-2: UE Antenna Type signaling

Agreement:

- To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed

- FFS for the details.

First of all, this UE capability shall indicate whether UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC.

Due to the scheduling is different between two kinds of UE, hence gNB need to know how the feature is supported by UE.

As agreed in RF session, antenna array type for inter-band CA is highly depending on frequency range of operating bands, therefore, the UE feature type shall be per BC.

Based on above analysis, we propose following UE capability:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| CommonRxBeam-r19 Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. | BC | CY | N/A | FR1 only |

Proposal 1: Define the UE capability to indicate common or separate Rx beam as following:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| CommonRxBeam-r19 Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. | BC | CY | N/A | FR1 only |

Issue 1-3-1-2: Whether Interruptions at NR SRS carrier based switching requirements need to be introduced

- FFS

- Proposal 1: The NR SRS carrier based switching interruption requirement can be introduced. (HW)

We understand the motivation of introducing SRS carrier based switching to ATG, that without UL on SCC, PCC could switch to SCC and transmit SRS to help network with channel estimation.

However, from RRM perspective, we didn’t define MTTD requirements for ATG CA. Besides, from RF perspective, whether n39 UL can co-exist with n3 DL is still FFS. Therefore, we propose not to introduce this requirement for ATG.

Proposal 2: Don’t introduce NR SRS carrier based switching interruption requirements.

Issue 1-4-4: Deactivated SCell measurement

Agreement:

- For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

- FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

For the ATG UE antenna type Case 4, since there is only one common beam, UE can’t perform L1 measurement of PCell and L3 neighbour cell measurement of CC with deactivated SCell simultaneously. Therefore, the L1/L3 measurement sharing factor is needed.

Proposal 3: Based on the legacy requirement:

- For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

Agreement:

- For ATG UE antenna types except Case 4

  - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

    - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- FFS for ATG Case#4 UE

For RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, L1-SINR measurement, the scheduling availability was introduced when the UE performs measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1, but doesn’t support following related UE capabilities:

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 Indicates whether the UE supports concurrent SSB based inter-frequency measurement without measurement gap on neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. UE indicates support of this indicates support of interFrequencyMeas-NoGap-r16. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range where the SSB and PDCCH/PDSCH are received. | UE | No | No | Yes |

Furthermore, when intra-band contiguous carrier aggregation in FR1 is configured, due to one set of tansceiver will be applied on this carrier, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, due to UE mandatory capable of two sets of tansceivers, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured. This is also valid for Case 4 UE, which share one antenna panel and one Rx beam but two sets of Rx chains between PCC and SCC. After the antenna receiving the signals from PCC and SCC, UE is able to process them separately.

Proposal 4: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement, following availability also applied for Case#4 UE.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured

Issue 1-3-3: SCell Addition/Activation mechanism

FFS

- Proposal 1: Consider distance triggered SCell addition (CMCC, HW)

  - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.(CMCC)

  - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. (HW)

- Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. (Ericsson)

- Proposal 3: further optimization methods for SCell addition and activation can be considered (CATT)

  - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

Issue 1-4-9: Measurement enhancement for ATG UE with antenna array

FFS

- Proposal 1:

  - If network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement. From requirement point of view, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed.

- Proposal 2:

  - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

As we know, for ATG UE with antenna array, it can’t receive the signal from serving cell and neighbour cells simultaneously due to different Rx beam direction. Therefore, the beam sweeping factor N1 and specific L1/L3 measurement sharing factor has been introduced from R18. Besides, scheduling restriction has been introduced for UE with the antenna array, and additional scheduling restriction also has agreed to be introduced for Case#4 UE.

When the ATG UE capable of antennaArrayType-r18 performs intra-frequency neighbouring cell measurements, the following restrictions apply due to SS-RSRP or SS-SINR measurement: - If deriveSSB-IndexFromCell is enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. - When inter-band carrier aggregation is performed: - - For ATG UE capable of [antenna array capability] on either PCC or SCC, and support [new UE capability of Case 4], the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

The scheduling restriction introduced by antenna array on SCC can be further summarized as following:

- Before SCell activation, when UE performs the neighbour cell measurement on CC with deactivated SCell, the scheduling restriction cased by antenna array will apply to PCell for Case#4 UE, throughput loss will be caused.

- After SCell activation, when UE performs the neighbour cell measurement on SCC (intra-frequency measurement), the scheduling restriction cased by antenna array will apply to SCell for UE with antenna array on SCC (Case#2, Case#4 and Case#5), and additional scheduling restriction will apply to PCell for Case#4 UE, throughput loss will be caused.

At the same time, considering co-located deployment, large cell coverage, LOS propagation condition, which are the ATG specific scenario, we observe there are some cases, that the the measurement for neighbour cells on the carrier frequency of (candidate) SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

For these cases, we further consider measurement improvement methodology.

In inter-band CA scenario, for ATG UE with antenna array on the band of SCC, UE omits the neighbour cell’s measurement on the carrier frequency of SCC. Besides, for Case#4 UE, omits the neighbour cell’s measurement on the CC with deactivated SCell. And Then, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed at all, the measurement delay requirement and scheduling restriction of omni-directional antenna UE can be reused. Throughput can be increased without UE consuming additional power.

This kind of enhancement can be achieved by network configuration, for example, if network identify neighbour cell measurement on SCC is not needed by deployment, or the measurement report from PCell, then network can configure a signal which indicates UE only measure the SCell when performing the intra-frequency measurement/deactivated SCC measurement. Another example is that network configure location based measurement triggering condition, which similar as distance condition in location-based CHO. If the distance between UE and the configured reference point is less than the threshold, the condition is fulfilled, UE can skip the neighbour cell measurement on the SCC or the CC with deactivated SCell.

Observation 1: In following cases, the measurement for neighbour cells on the carrier frequency of (candidate) SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

Proposal 5: UE could skip the neighbour cell measurement of SCC and/or CC with deactivated SCell by network control. For example, network configure a neighbour cell measurement skip command if network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, or network configure location based measurement triggering condition, the neighbour cell measurement on the SCC or the CC with deactivated SCell can be skipped if the condition is fulfilled.

Proposal 6: From requirement point of view, if UE skip the neighbour cell measurement of SCC and/or CC with deactivated SCell, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed, the scheduling restriction caused by antenna array will not be applied.

Whether a new clause is needed for ATG interruption requirement

We believe a new clause for ATG is more clear, and ‘refer to’ method can be used to avoid duplicated paragraph in the spec.

Proposal 7: Create a new clause for ATG interruption requirement.

- Conclusion

In this contribution, we further discuss the UE capability, Case#4 related issue and further enhancement of ATG measurement. The observation and proposals are:

Proposal 1: Define the UE capability to indicate common or separate Rx beam as following:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| CommonRxBeam-r19 Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. | BC | CY | N/A | FR1 only |

Proposal 2: Don’t introduce NR SRS carrier based switching interruption requirements.

Proposal 3: Based on the legacy requirement:

- For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

Proposal 4: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement, following availability also applied for Case#4 UE.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured

Observation 1: In following cases, the measurement for neighbour cells on the carrier frequency of (candidate) SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

Proposal 5: UE could skip the neighbour cell measurement of SCC and/or CC with deactivated SCell by network control. For example, network configure a neighbour cell measurement skip command if network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, or network configure location based measurement triggering condition, the neighbour cell measurement on the SCC or the CC with deactivated SCell can be skipped if the condition is fulfilled.

Proposal 6: From requirement point of view, if UE skip the neighbour cell measurement of SCC and/or CC with deactivated SCell, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed, the scheduling restriction caused by antenna array will not be applied.

Proposal 7: Create a new clause for ATG interruption requirement.

- Reference

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC