3GPP TSG-RAN WG4 Meeting#114bis     	R4-2503378

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.18.3

Source: 	CMCC

Title: 			 Simulation results for BS interference suppressing with MMSE-IRC

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results and observations.

- Simulation results

PUSCH requirements with interference (CP-OFDM/FDD, 5MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 10.53 | 12.13 |
|  |  |  | MMSE-MRC | 12.33 | 15.75 |
|  |  |  | Gain | 1.8 | 3.62 |
|  |  | TDLC300-100 | MMSE-IRC | 11.32 | 12.55 |
|  |  |  | MMSE-MRC | 12.74 | 16 |
|  |  |  | Gain | 1.42 | 3.45 |
| 1T2R (2 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 12.18 | 14.62 |
|  |  |  | MMSE-MRC | 14.51 | 18.35 |
|  |  |  | Gain | 2.23 | 3.73 |
|  |  | TDLC300-100 | MMSE-IRC | 12.92 | 15 |
|  |  |  | MMSE-MRC | 14.6 | 18.38 |
|  |  |  | Gain | 1.68 | 3.38 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.06 | 8.13 |
|  |  |  | MMSE-MRC | 10.49 | 13.96 |
|  |  |  | Gain | 3.43 | 5.83 |
|  |  | TDLC300-100 | MMSE-IRC | 7.47 | 8.52 |
|  |  |  | MMSE-MRC | 10.6 | 14.17 |
|  |  |  | Gain | 3.13 | 5.65 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 3.67 | 4.27 |
|  |  |  | MMSE-MRC | 7.15 | 10.84 |
|  |  |  | Gain | 3.48 | 6.57 |
|  |  | TDLC300-100 | MMSE-IRC | 4.11 | 4.58 |
|  |  |  | MMSE-MRC | 7.28 | 11.04 |
|  |  |  | Gain | 3.17 | 6.46 |

PUSCH requirements with interference (CP-OFDM/FDD, 20MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 11.87 | 12.07 |
|  |  |  | MMSE-MRC | 12.41 | 15.96 |
|  |  |  | Gain | 0.54 | 3.89 |
|  |  | TDLC300-100 | MMSE-IRC | 11.04 | 12.32 |
|  |  |  | MMSE-MRC | 12.98 | 16.16 |
|  |  |  | Gain | 1.94 | 3.84 |
| 1T2R (2 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 12.4 | 14.52 |
|  |  |  | MMSE-MRC | 14.43 | 18.27 |
|  |  |  | Gain | 2.03 | 3.75 |
|  |  | TDLC300-100 | MMSE-IRC | 12.65 | 14.62 |
|  |  |  | MMSE-MRC | 14.83 | 18.69 |
|  |  |  | Gain | 2.18 | 4.07 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.11 | 8.21 |
|  |  |  | MMSE-MRC | 10.23 | 14.19 |
|  |  |  | Gain | 3.12 | 5.98 |
|  |  | TDLC300-100 | MMSE-IRC | 7.39 | 8.43 |
|  |  |  | MMSE-MRC | 10.52 | 14.29 |
|  |  |  | Gain | 3.13 | 5.86 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 3.74 | 4.32 |
|  |  |  | MMSE-MRC | 7.18 | 10.92 |
|  |  |  | Gain | 3.44 | 6.6 |
|  |  | TDLC300-100 | MMSE-IRC | 4.09 | 4.52 |
|  |  |  | MMSE-MRC | 7.41 | 11.09 |
|  |  |  | Gain | 3.32 | 6.57 |

PUSCH requirements with interference (CP-OFDM/TDD, 10MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 11.25 | 13.22 |
|  |  |  | MMSE-MRC | 12.74 | 16.3 |
|  |  |  | Gain | 1.49 | 3.08 |
|  |  | TDLC300-100 | MMSE-IRC | 11.78 | 13.59 |
|  |  |  | MMSE-MRC | 13.08 | 16.38 |
|  |  |  | Gain | 1.3 | 2.79 |
| 1T2R (2 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 12.95 | 15.54 |
|  |  |  | MMSE-MRC | 14.59 | 18.74 |
|  |  |  | Gain | 1.64 | 3.2 |
|  |  | TDLC300-100 | MMSE-IRC | 13.38 | 16.05 |
|  |  |  | MMSE-MRC | 15 | 18.79 |
|  |  |  | Gain | 1.62 | 2.74 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.71 | 9.15 |
|  |  |  | MMSE-MRC | 10.73 | 14.68 |
|  |  |  | Gain | 3.02 | 5.53 |
|  |  | TDLC300-100 | MMSE-IRC | 8.14 | 9.62 |
|  |  |  | MMSE-MRC | 11.01 | 14.79 |
|  |  |  | Gain | 2.87 | 5.17 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 4.38 | 5.24 |
|  |  |  | MMSE-MRC | 7.68 | 11.66 |
|  |  |  | Gain | 3.3 | 6.42 |
|  |  | TDLC300-100 | MMSE-IRC | 4.7 | 5.16 |
|  |  |  | MMSE-MRC | 7.99 | 11.89 |
|  |  |  | Gain | 3.29 | 6.72 |

PUSCH requirements with interference (CP-OFDM/TDD, 100MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 11.27 | 12.87 |
|  |  |  | MMSE-MRC | 12.77 | 16.35 |
|  |  |  | Gain | 1.5 | 3.48 |
|  |  | TDLC300-100 | MMSE-IRC | 11.66 | 13.39 |
|  |  |  | MMSE-MRC | 13.16 | 16.38 |
|  |  |  | Gain | 1.5 | 2.99 |
| 1T2R (2 interference) | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 13.06 | 15.37 |
|  |  |  | MMSE-MRC | 14.64 | 18.74 |
|  |  |  | Gain | 1.58 | 3.37 |
|  |  | TDLC300-100 | MMSE-IRC | 13.4 | 15.83 |
|  |  |  | MMSE-MRC | 14.92 | 18.84 |
|  |  |  | Gain | 1.52 | 3.01 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.78 | 9.21 |
|  |  |  | MMSE-MRC | 10.68 | 14.61 |
|  |  |  | Gain | 2.9 | 5.4 |
|  |  | TDLC300-100 | MMSE-IRC | 8.18 | 9.73 |
|  |  |  | MMSE-MRC | 10.96 | 14.79 |
|  |  |  | Gain | 2.78 | 5.06 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 4.42 | 5.27 |
|  |  |  | MMSE-MRC | 7.72 | 11.76 |
|  |  |  | Gain | 3.3 | 6.49 |
|  |  | TDLC300-100 | MMSE-IRC | 4.7 | 5.51 |
|  |  |  | MMSE-MRC | 7.91 | 11.85 |
|  |  |  | Gain | 3.21 | 6.34 |

- Simulation assumption

PUSCH requirements with interference

Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD, FDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30kHz/10MHz, 100MHz FDD: 15kHz/5MHz, 20MHz |
| Transmit precoding |  | Disabled |
| Antenna configuration |  | 1x2 (1/2 interferes) 1x4 (2 interferes) 1x8 (2 interferes) |
| Propagation condition |  | For serving cell: Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet Option 2: different channel for different MCS: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 For Interference cell: TDLC300-100 |
| MCS |  | 2R: MCS 16 4R: MCS 16 8R: MCS 16 |
| PUSCH configuration | Mapping type | Type A/B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | DM-RS sequence generation | nSCID =0 for all cells Different NID0 for serving and interference cells NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively For DFT-S-OFDM: group hopping and sequence hopping are disabled |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| Test metric |  | SNR@70% max TP |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| INR values for Homogeneous deployment | INRs 5.35 dB in case of 1 interference cells INRs 7.28, -2.52 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 9.82 dB in case of 1 interference cells INRs 12.38, -0.97 dB in case of 2 interference cells |
| Modulation order of interfering PUSCH | 16QAM randomly modulated symbols |
| Interference cell modeling | Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

- Reference

[1] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, Moderator (Nokia)