3GPP TSG-RAN WG4 Meeting #114bis				R4-2503389

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.2.3

Source: 	CMCC

Title: 	Discussion on A-IoT CW RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for A-IoT CW node.

- Discussion

Following list the candidate requirements for A-IoT CW.

| CW RF Requirement | Potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD |
| Frequency error | YES |
| Phase noise | TBD for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD |

- Output power

Based on RAN1 link budget analysis as below, it seems the R2D link is bottleneck for D1T1-B. But following assume CW transmit 23dBm with 10m distant from device and reader transmit 33dBm. now RAN4 define 38dBm per antenna connector for reader. If we assume 2Tx at reader side, then R2D MPL could be enlarged by 8dB, then R2D and D2R link coverage is comparable when assuming CW is 23dBm with 10m distant from device. Enlarging the power of CW to 33dBm could enlarge D2R coverage and this is significent when device has better REFSENSE performance

Table D1T1-96bit-10%BLER-6kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-B | 1-4 | 67.1 | 84.54 | 67.10 | 36.59 |

Observation 1: higher output power of CW will enlarge D2R coverage which is coverage bottleneck for devices with better sensitivity performance.

Besides, the GSM UE transmit with 33dBm, so max 33dBm power is allowed from regulatory point of view.

Proposal 1: the max output power for CW is suggested as 33dBm.

- OFF power requirements

OFF power requirements is still FSS in last meeting. Current scope only contains the HD FDD operation between R2D and CW2D/D2R, CW is OFF when reader is transmitting. One motivation of OFF power is to avoid the interference from closed CW transmitter to device receiver, but such interference is negligible compared with R2D power. Therefore, there is no clear motivation to define OFF power.

Observation 2: there is no clear motivation of why the OFF power is needed.

- Phase noise for outside topology

SI analysis show phase noise could be mitigated through calibration mechanism. For outside topology, this depends on how to perform self-interference cancellation scheme. If there is no accurate CW signal characteristics at reader side, the phase noise may be hard to be canceled and may enlarge the received noise at reader side. But this phase noise can be implicitly impacted into REFSENSE requirement with sensitivity degradation part. RAN4 is suggested to further check whether the sensitivity degradation part in REFSENSE definition could already reflect phase noise performance of CW implicitly.

Proposal 2: further check whether the phase noise of CW could be implicitly reflected into sensitivity degradation part of reader sensitivity requirements definition.

- Unwanted emission requirements

CW has narrow signal bandwidth. In theory, it could have better unwanted emission requirements. NB-IoT BS unwanted emission requirement could be referred as baseline.

Proposal 3: use NB-IoT BS unwanted emission requirements as baseline for CW.

- intermodulation

In last meeting, some companies suggest to discuss the co-located scenario at first since current Tx intermodulation requirement is defined for co-located case. For the outside topology, to avoid the interference from CW to reader receiver, the common assumption is CW distant from reader. Besides, Currently CW colocated with other indoor reader is also outside of the scope since the scope assume no indoor 900MHz BS and UE.

Observation 3: the CW node will not be co-located with other BS or reader according to current scope.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Observation 1: higher output power of CW will enlarge D2R coverage which is coverage bottleneck for devices with better sensitivity performance.

Proposal 1: the max output power for CW is suggested as 33dBm.

Observation 2: there is no clear motivation of defining the OFF power.

Proposal 2: further check whether the phase noise of CW could be implicitly reflected into sensitivity degradation part of reader sensitivity requirements definition.

Proposal 3: use NB-IoT BS unwanted emission requirements as baseline for CW.

Observation 3: the CW node will not be co-located with other BS or reader according to current scope.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.