3GPP TSG-RAN WG4 Meeting # 114-bis                                                          R4-2503394

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.12.2

Source: 	CMCC

Title: 	Discussion on BS RF requirements for ATG

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. During the past meeting we focused on the issue of the TAE requirements, and the revised WID has approved the discussion of the TAE requirements for ATG with DL MIMO. In this paper, we will focus on multi-band operation.

# 2. Discussion

## 2.1. ATG BS capable of multi-band operation

During RAN4#114 meeting, one draft CR has been endorsed, and one part of the draft CR[2] is shown as below.

For multi-band connectors and multi-band RIBs supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands. The RF requirements for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered by the present release of this specification.

We have noticed that the highlighted part of the specification: “The RF requirements for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered by the present release of this specification.” The reason is that for the multi-band feature, there is a requirement to process two or more carriers in common active RF components simultaneously. However, this is not feasible for CA band combination involving TDD and FDD bands. Considering that the only inter-band CA combination currently supported by ATG is CA_n3-n39, ATG cannot support multi-band operation at this stage.

Proposal 1: ATG BS could not support multi-band operation with CA_n3-n39.

# 3. Conclusions

In this contribution, ATG BS requirements are discussed with the following proposals:

Proposal 1: ATG BS could not support multi-band operation with CA_n3-n39.

# 4. References

[1] RP-242385, Revised WID on Enhancements for Air-to-ground network for NR

[2] R4-2502280, draft CR to TS 38.104: the introduction of Rel-19 ATG BS supporting CA