3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503427

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM core requirements

Source: 	CATT

Agenda item:	7.12.4

Document for:	Discussion

- Introduction

In last RAN4 meeting, RRM requirements for Rel-19 ATG enhancement were discussed. The conclusions for ATG RRM requirements for CA were captured in the approved WF, and there were still some issues for FFS [1]. In this paper, we further discuss the potential RRM impact for Rel-19 ATG RRM requirements and give our views.

- Discussion

  - 2.1 Signalling characteristics

- Whether Interruptions at NR SRS carrier based switching requirements need to be introduced

In the last meeting, the agreement on whether interruptions at NR SRS antenna port switching requirements need to be introduced is copied as below [1], and whether interruptions at NR SRS carrier based switching requirements need to be introduced were still FFS.

FFS - Proposal 1: The NR SRS carrier based switching interruption requirement can be introduced. (HW)

Regarding whether interruptions at NR SRS carrier based switching requirements need to be introduced, most companies think that RAN4 doesn’t need to introduce NR SRS carrier based switching related requirements as only DL CA is considered for ATG CA. However, there were still company believe that interruptions at NR SRS carrier based switching requirements need to be considered even though only DL CA is supported for ATG CA.

As we know, the essence of SRS carrier switching mechanism is to allow UE to send SRS between different uplink carriers, enabling the BS to obtain uplink information of multiple carriers. From our understanding, SRS carrier switching is allowed to exist independently of UL CA configuration according to TS 38.331, that is, even if UL CA is not configured, the BS can still configure UE to send SRS on multiple uplink carriers. We also can understand that although SRS is commonly used for UL channel estimation, due to the heterogeneity of UL and DL channels in TDD scenarios, the measurement results of multiple UL carriers can also be used to assist and optimize the resource allocation of DL CA.

Observation 1: SRS carrier switching is allowed to exist independently of UL CA configuration, that is, even if UL CA is not configured, the BS can still configure UE to send SRS on multiple uplink carriers.

Observation 2: Although SRS is commonly used for UL channel estimation, due to the heterogeneity of UL and DL channels in TDD scenarios, the measurement results of multiple UL carriers can also be used to assist and optimize the resource allocation of DL CA.

However, the existing supported example band combination is only CA_n3A-n39A for DL inter-band CA, which is the combination of FDD band and TDD band. If the measurement results of multiple UL carriers mainly can be used for TDD scenarios, then only the measurement results on multiple UL carriers in the TDD related UL band  will be useful to DL CA.

Proposal 1:  The NR SRS carrier based switching interruption requirement can be introduced.

- Only the measurement results on multiple UL carriers in the TDD related UL band will be useful to DL CA.

- SCell Addition/Activation mechanism

In the last meeting, RAN4 discussed SCell Addition/Activation mechanism but still didn’t reach the consensus. The candidate options are copied as follows [1]:

FFS - Proposal 1: Consider distance triggered SCell addition (CMCC, HW) - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.(CMCC) - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. (HW) - Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. (Ericsson) - Proposal 3: further optimization methods for SCell addition and activation can be considered (CATT) - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and/or activation can be considered, as the coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell. At least for Case 4, omitting the measurement of SCell addition and/or activation eliminates the need to apply scheduling restrictions to neighboring cell measurements on candidate SCCs, and it will achieve throughput gains.

Generally, we are fine with proposal 1 for Case 4. However, we think that option1-1 will also involve RAN2’s work. From RAN4’s perspective, another method is to define distance triggered SCell addition and/or activation requirements with some distance related conditions only in RAN4 spec, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted.

Observation 3: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell.

Observation 4: At least for Case 4, omitting the measurement of SCell addition and/or activation eliminates the need to apply scheduling restrictions to neighboring cell measurements on candidate SCCs, and it will achieve throughput gains.

Proposal 2: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, distance triggered SCell addition can be considered for Case 4.

Proposal 3: Another method is to define distance triggered SCell addition and/or activation requirements with some distance related conditions only in RAN4 spec, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted.

  - 2.2 Measurement procedure

- Deactivated SCell measurement

In the last meeting, RAN4 discussed the requirement for deactivated SCell measurement and agreed that •	 L1/L3 measurement sharing factor is not needed for ATG UE antenna types except Case 4. Whether L3/L1 measurement sharing factor is needed for ATG UE antenna type Case 4 was still FFS. The agreement is copied as follows [1]:

Issue 1-4-4: Deactivated SCell measurement Agreement: - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed - FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

For the ATG UE antenna type Case 4, since there is only one common Rx beam on the bands of PCC and SCC, even though UE doesn’t need to perform L1 measurement on the deactivated SCell, UE still need to perform L1 measurement on the PCell. Therefore, whether to introduce L1/L3 measurement sharing factor for L3 deactivated SCell measurement depends on whether to introduce L1/L3 measurement sharing factor between L3 deactivated SCell measurement and L1 measurement on the PCell.

In our view, although PCell and SCell are co-located in ATG, L3 deactivated SCell measurement defined in existing spec is for neighbor cell, while L1 measurement requirements on PCell defined in existing spec is for serving cell, so the L1/L3 measurement sharing factor is needed.

Observation 5: Even though UE doesn’t need to perform L1 measurement on the deactivated SCell, UE still need to perform L1 measurement on the PCell.

Observation 6: Whether to introduce L1/L3 measurement sharing factor for L3 deactivated SCell measurement depends on whether to introduce L1/L3 measurement sharing factor between L3 deactivated SCell measurement and L1 measurement on the PCell.

Observation 7: L3 deactivated SCell measurement defined in existing spec is for neighbor cell, while L1 measurement requirements on PCell defined in existing spec is for serving cell.

Proposal 4: For the ATG UE antenna type Case 4, the L1/L3 measurement sharing factor is needed.

- Conclusion

This document discussed the issues for R19 ATG RRM core requirements and presented following observations and proposals:

Observation 1: SRS carrier switching is allowed to exist independently of UL CA configuration, that is, even if UL CA is not configured, the BS can still configure UE to send SRS on multiple uplink carriers.

Observation 2: Although SRS is commonly used for UL channel estimation, due to the heterogeneity of UL and DL channels in TDD scenarios, the measurement results of multiple UL carriers can also be used to assist and optimize the resource allocation of DL CA.

Proposal 1:  The NR SRS carrier based switching interruption requirement can be introduced.

- Only the measurement results on multiple UL carriers in the TDD related UL band will be useful to DL CA.

Observation 3: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell.

Observation 4: At least for Case 4, omitting the measurement of SCell addition and/or activation eliminates the need to apply scheduling restrictions to neighboring cell measurements on candidate SCCs, and it will achieve throughput gains.

Proposal 2: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, distance triggered SCell addition can be considered for Case 4.

Proposal 3: Another method is to define distance triggered SCell addition and/or activation requirements with some distance related conditions only in RAN4 spec, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted.

Observation 5: Even though UE doesn’t need to perform L1 measurement on the deactivated SCell, UE still need to perform L1 measurement on the PCell.

Observation 6: Whether to introduce L1/L3 measurement sharing factor for L3 deactivated SCell measurement depends on whether to introduce L1/L3 measurement sharing factor between L3 deactivated SCell measurement and L1 measurement on the PCell.

Observation 7: L3 deactivated SCell measurement defined in existing spec is for neighbor cell, while L1 measurement requirements on PCell defined in existing spec is for serving cell.

Proposal 4: For the ATG UE antenna type Case 4, the L1/L3 measurement sharing factor is needed.

- Reference

[1]	R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC