3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503435

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	7.27.2.3

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part2 was approved and some agreements were reached [1], but there still many issues left [2]. This document will further discuss the remaining RRM issues about Conditional LTM/Subsequent Conditional LTM, and give our understanding and proposals.

- Discussion

Sub-topic 1-1: General aspects on Conditional LTM

- The applicable scenarios for conditional LTM

In the last meeting, the applicable scenarios for conditional LTM was discussed in RAN4, and agreement is copied as follows [1]:

< Agreement > - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

For the case which CLTM is triggered by L1 measurement, whether to consider deactivated SCell and the cells with same frequency were still FFS. In R18, this scenario has already been supported by RAN2, but RAN4 has not defined the relevant requirements due to time limitation. In R19 CLMT, considering that network configuration should not be restricted, we believe that relevant requirements can be defined only in R19 CLTM if time is allowed.  Considering the workload, we can also compromise not to define requirements.

Proposal 1: The requirements for deactivated SCell and the cells with same frequency can be defined only in R19 CLTM if time is allowed.

Sub-topic 1-2: (Subsequent) Conditional LTM delay

- The components of (Subsequent) conditional LTM delay

In the last meeting, RAN4 discussed the components of conditional LTM delay, there is not much controversy about the overall conditional LTM delay (DCLTM), but the main point of controversy lies in what components are included in TCLTM_interrupt. The agreement is copied as follows [1]:

< Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

In detail, the legacy L3 CHO delay (DCHO) was defined as follows in TS 38.133 [3]:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time. Therefore, for Rel-19 conditional LTM, we believe that the legacy DCHO can be a baseline, so the conditional LTM delay can be:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

For the interruption time in conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline, since TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms in R18 LTM, so the whole CLTM delay can be as follows:

DCLTM = TRRC + TEvent_DU + Tmeasure+ TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU+ TCLTM_execution.

Proposal 2: The legacy delay requirments for L3 CHO and R18 LTM can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure+ TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU+ TCLTM_execution.

In the last meeting, one controversial part is which components are included in the interruption time for CLTM. For option 1, we think it is an optimization for reducing interruption delay. In R18 LTM, UE stops source cell data Tx/Rx after receiving cell switch command. However, most of the interruption time was spent waiting for SSB of the target cell, so option 1 remove TLTM-RRC-processing + Tfirst-RS + TRS-proc from the interruption time. Option 3 simply uses TLTM-interrupt defined in R18 LTM to completely replace Tinterrupt in legacy CHO delay. In our view, the optimization like option 1 can be introduced in the subsequent release, and companies need to further clarify the necessity of optimization for reducing interruption delay in CLTM.

Proposal 3: For interruption time, the optimization like option 1 can be introduced in the subsequent release, and companies need to further clarify the necessity of optimization for reducing interruption delay in CLTM.

For subsequent conditional LTM delay, it should be discussed based on conditional LTM delay, we believe that at least TRRC will not be considered in subsequent conditional LTM delay.

Proposal 4: At least TRRC will not be considered in subsequent conditional LTM delay.

- TRRC

In the last meeting, the Way forward for TRRC is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay. - Proposal 2: Too early to agree whether TRRC is needed.

Since RAN4 has agreed the starting point of conditional LTM delay under some assumption, which is the end of the last TTI containing the RRC command for conditional LTM [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM], so TRRC has been included and the legacy defination of TRRC can be reused.

Proposal 5: TRRC is the RRC procedure delay [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM].

- Tmeasure

In the last meeting, the Way forward on the components of Tmeasure is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - Proposal 1a: - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on CSI-RS based L3-RSRP measurement. - Proposal 1b: - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure. - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

Firstly, the general description in proposal 1 is ok for us, and it is the same with the existing definition. However, the detailed measurement period need to be clarified, and it is related to whether L3 based measurement is considered. In the last meeting, RAN4 has agreed to define the requirements for L3 measurement based conditional LTM in FR1. The agreement is copied as below [1]:

Issue 1-1-2: Measurement requirements on conditional LTM < Agreement > - For FR1, define the requirements for L3 measurement based conditional LTM. - For FR2, not define the requirements for L3 measurement based conditional LTM.

According to the above, Tmeasure can refer to proposal 1a.

Proposal 6: The following general description in proposal 1 is ok for us:

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

Proposal 7: Tmeasure can refer to proposal 1a.

- TCLTM_execution / TLTM_execution

The Way forward on the components of TCLTM_execution / TLTM_execution in the last meeting is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Proposal 1a: TLTM_execution can be up to 10ms. - Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay.

We are fine with the description in proposal 1 to reuse the legacy definition in existing CHO delay requirements [3].

Proposal 8: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

Sub-topic 1-3: Early UL sync

- UE based TA measurement

In the last meeting, RAN4 further discussed whether to define the requirements for UE based TA measurement and the Way Forward are copied as follows [1]:

< Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

Although RAN2 also supported UE based TA measurement mechanism in R18 LTM, and RAN4 did not define the related requirements for it. However, compared with R18 LTM, we believe that the UE based TA measurement mechanism in CLTM is more suitable for UE, because UE can start measuring at any time and have more autonomy. When the condition of switching are met and the UE measures the TA in advance, the UE can use the corresponding TA when accessing, and the network can save resources without transmitting PDCCH order. However, we also can understand the concerns from companies, so we suggest to focus on how to define the feasible requirements for UE based TA measurement for conditional intra-CU LTM.

Proposal 9: RAN4 to focus on how to define the feasible requirements for UE based TA measurement for conditional intra-CU LTM.

- Conclusion

This document discussed the issues for conditional Intra-CU LTM and presented following proposals:

Proposal 1: The requirements for deactivated SCell and the cells with same frequency can be defined only in R19 CLTM if time is allowed.

Proposal 2: The legacy delay requirments for L3 CHO and R18 LTM can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure+ TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU+ TCLTM_execution.

Proposal 3: For interruption time, the optimization like option 1 can be introduced in the subsequent release, and companies need to further clarify the necessity of optimization for reducing interruption delay in CLTM.

Proposal 4: At least TRRC will not be considered in subsequent conditional LTM delay.

Proposal 5: TRRC is the RRC procedure delay [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM].

Proposal 6: The following general description in proposal 1 is ok for us:

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

Proposal 7: Tmeasure can refer to proposal 1a.

Proposal 8: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

Proposal 9: RAN4 to focus on how to define the feasible requirements for UE based TA measurement for conditional intra-CU LTM.

- Reference

[1]     R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

[2]     R4-2500541, Topic summary for [114][227] NR_Mob_Ph4_Part2, China Telecom

[3]     3GPP TS 38.133, NR Requirements for support of radio resource management