3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503436

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.29.4.1

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 was approved and some agreements were reached, but there still are a few issues left [1]. This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase 3 and give our understanding and proposals.

- Discussion

  - General consideration

- 2.1.1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

SDT requirements

For CG-SDT, RAN4 has agreed to remove TA validation based on the RSRP change criterion for (e)RedCap UE with FR1-NTN and to refer to the requirements on TA validation for the PUR requirements for IoT-NTN. Whether to send RAN2 LS were still FFS. The agreement in the last meeting is copied as follows [1]:

Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

In our view, from RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement only focus on RAN4’s requirements.

The SDT was supported in R17 NTN in RAN2, which was not limited to UE types, and the specific content is no different from SDT defined in TN. According to the RAN2’s spec, if the field of cg-SDT-TA-ValidationConfig not configured, then the UE does not perform RSRP based TA validation. Therefore, from a network perspective, there is no need to restrict configuration, and RAN4 can only make our decision from the perspective of RRM requirements. Therefore, we think that triggering RAN2's discussion on this issue is meaningless and RAN4 does not need to send LS to RAN2.

Observation 1: From RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement for TA validation in CG-SDT requirements only focus on RAN4’s requirements.

Proposal 1: For TA validation in CG-SDT requirements for (e)RedCap NTN, RAN4 does not need to send LS to RAN2 and the agreement reached in the last meeting only focus on RAN4’s requirements.

- 2.1.2 Features to be considered for (e)RedCap UE with FR1-NTN

SMTCs and 'in parallel' related

      - a. Feature 25-1(parallelSMTC)

In the last meeting, RAN4 discussed the UE capability of parallelSMTC (Feature 25-1 in R17) for (e)RedCap UE with FR1-NTN. RAN4 firstly mainly focused on the discussion on the baseline for UE capability, which means that it is mandatory for UE to support, and there were two options for baseline. The Way Forward in the last meeting is copied as follows [1]:

<Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

In our view, there is no need to differentiate 1Rx and 2Rx RedCap UEs for this UE capability.

For Option 2, UE is madatory to support 2 SMTC and it is the same as legacy in NTN. In this case, UE always supports simultaneously parallel measurement of multiple SMTCs, but it requires that multiple SMTCs do not overlap in the time domain and meet the minimum proximity distance. In this case, UE can measure all SMTCs in TDM without extending the overall measurement delay due to multiple SMTCs, and it is also regarded as in parallel measurement. The measurement delay will not be extended due to the number of SMTCs that partially overlap with each other if multiple SMTCs meet the minimum proximity distance.

For Option 1, UE is madatory to support 1 SMTC. In this case, UE cannot measure multiple SMTCs in parallel at the same time, only one SMTC can be measured. If the UE is configured with multiple SMTCs (such as 2 SMTC), unlike legacy, even if the two SMTCs do not overlap, UE can only measure 1 SMTC and there is no measurement requirements for other SMTCs. In addition, if Option 1 is supported by RAN4 finally, we think that UE will also need to optionally support the 2 parallel SMTCs are TDM with proximity distance.

Observation 2: For Option 2, if multiple SMTCs do not overlap in the time domain and meet the minimum proximity distance, UE can measure all SMTCs in TDM without extending the overall measurement delay due to multiple SMTCs.

Observation 3: For Option 1, if the UE is configured with multiple SMTCs (such as 2 SMTC), even if the two SMTCs do not overlap, UE can only measure 1 SMTC and there is no measurement requirements for other SMTCs.

In NTN scenario, since time difference between serving cell and neighbour cell will be too long, an SMTC may not be able to cover their measurements simultaneously. Therefore, from network perspective, we prefer Option 2 to ensure that at least 2 SMTCs can be measured simultaneously. However, we also understand the concern from UE sides, and the proponents of Option 1 are encouraged to further clarify which specific reduced capabilities of RedCap UE have made it unable to handle more than two SMTCs simultaneously or just for a little power saving gains, etc.

Proposal 2: There is no need to differentiate 1Rx and 2Rx RedCap UEs for this UE capability.

Proposal 3: If we don't see significant power saving gains or other benefits, prefer Option 2 to ensure that at least 2 SMTCs can be measured simultaneously:

- RedCap UE is madatory to support 2 parallel SMTC.

- ‘Parallel’ means the SMTCs are TDM with proximity distance.

- The measurement delay will not be extended due to the number of SMTCs that partially overlap with each other if multiple SMTCs meet the minimum proximity distance.

Proposal 4: If Option 1 is supported by RAN4, the UE will also need to optionally support the 2 parallel SMTCs are TDM with proximity distance.

      - b. The proximity distance between two SMTCs

In the last meeting, RAN4 discussed the proximity distance between two SMTCs for (e)RedCap UE with FR1-NTN. RAN4 has agreed to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN and the distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, it was FFS and the agreement is copied as follows [1]:

Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

For option 1, RAN4 will not consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, which means the legacy collision condition can be reused as following:

- Two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms.

For option 2, when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, RAN4 will consider relaxation x ms for proximity compared to legacy 4ms.

In our view, we don’t see any technical reasons to reuse the legacy collision condition and prefer option 1. The proponents of Option 2 are encouraged to further clarify which specific reduced capabilities of RedCap UE will lead to this relaxation.

Proposal 5: If we don't see significant power saving gains or other benefits, RAN4 doesn’t need to relax for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier and to reuse the legacy collision condition.

      - c. The number of cells/SSBs/supported satellites

In the last meeting, the Way Forward for the number of cells/SSBs/supported satellites are copied as below [1]:

<Way Forward>: - For the number of supported satellites, - Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE. - Proposal 2: - RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite. - The total number of supported satellites for measurement in a frequency layer is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer. - For the number of cells and SSB, - Proposal 1: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e., - Proposal 1a: For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Firstly, for Feature 25-7(maxNumber-LEO-SatellitesPerCarrier-r17), which indicates the number of target LEO satellites the UE can monitor per carrier including serving satellite, we think that the legacy UE capability of maxNumber-LEO-SatellitesPerCarrier-r17 can be reused for (e)RedCap UEs and it will be optional with capability signalling.

Regarding the number of cells and number of SSB for (e)RedCap UEs with TN, since both are the same as normal UEs with TN, so the legacy requirement for normal UE with NTN also can be reused for (e)RedCap UEs with NTN.

Proposal 6: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs, and it will be optional with capability signalling.

Proposal 7: For the number of cells and SSB, the following legacy requirement also can be reused for (e)RedCap UEs:

- For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

NCD-SSB

In RAN4#112 meeting, RAN4 has already reached the following agreements [1]:

<Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

In our view, even though NCD-SSB can bring benefits to RedCap UE in NTN, but considering that the NTN bandwidth is too small since the configurable bandwidth of NTN is only 20MHz, so the benefit of NCD-SSB is not obvious, so we support to still keep the above agreements.

Another point we suggest to clarifying is the meaning of ‘no need to define NCD-SSB specific measurement requirements’ in the agreement. To facilitate the writing of CR, RAN4 to only use ‘CD-SSB’ to define the requirements or use ‘SSB’ should be discussed. For this issue, the WF in the last meeting is copied as follows [3]:

<Way Forward>: - Proposal 1: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB. - Proposal 2: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

For the terminology of SSB used in the spec, we support to use the terminology of SSB without differentiating CD-SSB and NCD-SSB in the spec, which seems to be uncontroversial.

For whether to add additional requirement applicability about CD-SSB/NCD-SSB, we think that the types of SSB need to be clarified. Considering that the network configuration can’t be restricted, so it is better to add a clarification of applicability in a separate clause to reflect the related agreement on this aspect (such as clause 3 in TS 38.133[4]), and the wording can be: the requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB. If there is no clarification on the types of SSB, the applicability of the requirements is not very clear since we have no requirements for NCD-SSB.

Proposal 8: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

Other features

In the last meeting, other features were not discussed, and the Way Forward is copied as below [1]:

<Way Forward>: - For SMTC adjustment in idle/inactive, - Option 1: Keep mandatory for RedCap UE. - Option 2: Keep optional for RedCap UE. - For Location-based measurement trigger, - Option 1: Keep optional for RedCap UE. - Option 2: RAN4 not to further discuss applicability of location-based measurement report trigger.

For SMTC adjustment in idle/inactive, we think it is quite important feature in RRM measurements for NTN, if it is not supported for UE, the UE will not be able to adjust SMTC time window to measure neighbour cell due to the time difference between serving cell and neighbour cell will change. Therefore, we think that SMTC adjustment in idle/inactive should be mandatory in NTN regardless of the UE type and it also doesn’t need very high complexity for RedCap UE.

Proposal 9: SMTC adjustment in idle/inactive should be supported by (e)RedCap UEs with NTN.

For Location-based measurement trigger, the related UE capability was defined by RAN2 not by RAN4, and there is no requirement related to this feature in RAN4 spec, so we support Option 2 to not further discuss the applicability of location-based measurement report trigger.

Observation 4: For Location-based measurement trigger, the related UE capability was defined by RAN2 not by RAN4, and there is no requirement related to this feature in RAN4 spec.

Proposal 10: RAN4 not to further discuss applicability of location-based measurement report trigger.

  - Reduction in the number of UE Rx branches

    - Mobility requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

RAN4 has agreed to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands in RAN4#113 meeting, and this issue is about how to define the enhanced cell reselection measurement requirements. The Way Forward in the last meeting is copied as follows [1]:

<Way Forward>: - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN. - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused. - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE, - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples - Same delay requirements for 1Rx and 2Rx UEs. - Proposal 1c: - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements - For 1 Rx (e)RedCap NTN UE, RAN4 to - Option 1: Reuse the existing enhanced cell reselection measurement requirements - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined. - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

From our point of view, we agree to keep this enhanced feature as an optional feature. For the specific measurement requirements, we prefer option 1a, that is, RAN4 to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle, the requirement defined for normal UEs with FR1-NTN can be reused. If RedCap UE is configured with eDRX_IDLE cycle, then the power saving may be more important for RedCap UE, RAN4 may not need the enhanced cell reselection measurement requirement.

Proposal 11: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional for (e)RedCap UE with FR1-NTN.

- RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle, the requirement defined for normal UEs with FR1-NTN can be reused.

Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

In the last meeting, RAN4 agreed to define measurement requirements in cell re-selection for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. This issue is about how to define the related requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN. The Way Forward is copied as below [1]:

<Way Forward>: - Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline. - Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle. - Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

In our view, Proposal 1 is not clear enough. For Proposal 2, since RAN4 agreed to introduce eDRX configuration, so the eDRX related impact need to be considered for all of cell re-selection measurement requirements including inter-RAT E-UTRAN TN carrier cell re-selection. So we support the following:

- For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN.

- For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for (e)RedCap UE with TN.

For Proposal 3, it is not contradictory with the above, and it just clarify the margin of RSRP/RSRQ in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell (or an already identified inter-RAT E-UTRA cell) meets the reselection criteria. So we support that RAN4 to reuse the existing margin of RSRP/RSRQ defined for normal UEs with FR1-NTN for 1 Rx RedCap and 2 Rx RedCap.

Proposal 12: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

- For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN.

- For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for (e)RedCap UE with TN.

Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

Proposal 13: RAN4 to reuse the existing margin of RSRP/RSRQ defined for normal UEs with NTN for 1 Rx and 2 Rx (e)RedCap UE.

- the reselection criteria are met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

Regarding how to define SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN, RAN4 has reached on the following agreements in RAN4#113 meeting [2]:

RAN4#113: #### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN < Agreement >: - For 1Rx (e)RedCap UEs with FR1-NTN bands, - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

The Way Forward for SDT in RAN4#114 meeting is copied as follows [1]:

<Way Forward>: - Proposal 1: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as: - For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Since RAN4 has agreed to remove TA validation based on the RSRP change criterion for CG-SDT and to refer to the requirements on TA validation for the PUR requirements for IoT-NTN, the threshold of sdt-RSRP-Threshold-r17 can be ignored only from RAN4’s perspective if the UE is configured with cg-SDT-TA-ValidationConfig.

Therefore, the agreement reached in RAN4#113 can be removed.

Proposal 14: For SDT requirements on RedCap NTN (including CG-SDT and RA-SDT), the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

In addition, except for the TA validation and corresponding offset of threshold for CG-SDT and RA-SDT requirements, we think that the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

- For NTA in CG-SDT, it refers to the value specified in subclause 7.1C.2.

- For requirements for SDT based on 2-step RA and 4-step RA, they refer to 6.2C.2.3 and 6.2C.2.2 respectively.

Proposal 15: Except for the TA validation and corresponding offset of threshold for CG-SDT and RA-SDT requirements, we think that the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

- For NTA in CG-SDT, it refers to the value specified in clause 7.1C.2.

- For requirements for SDT based on 2-step RA and 4-step RA, they refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

  - The impact of HD-FDD

    - General HD-FDD related impact

General HD-FDD related impact

RAN4 didn’t discuss this issue in the last meeting, and the Way Forward is copied as below [1]:

<Way Forward>: - Proposal 1: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario. - Proposal 1a: To following the TN, the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs. - Proposal 1c: - Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE during L1-RSRP measurement - Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Proposal 1d: RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions. - Proposal 2: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side. - Proposal 3: For HD RedCap and eRedCap UEs, RAN4 to adopt a high level statement that the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Proposal 4: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

For this issue, RAN1 has almost completed the collision handling rule for overlapping DL and UL in HD-FDD for RedCap UE with NTN, which may impact scheduling availability in RRM requirement, and the current progress is listed here for information:

RAN1#116: Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered. RAN1#117: Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. RAN1#118: Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied RAN1#118bis: Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS RAN1#119: Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL RAN1#120: Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

RAN4 has agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point [3]. According to the RedCap WI, we think that at least the following  RRM requirements are affected by HD-FDD for (e)RedCap UE with NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

Proposal 16: The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE in NTN scenario, and at least the following requirements will be affected by HD-FDD:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- 2.3.2 The specific impact of HD-FDD on NR Handover

The impact on NR Handover due to HD-FDD

For the impact on NR Handover due to HD-FDD, we think that the legacy applicability can be reused for RACH-based/RACH-less HO, Time/location-based CHO and Satellite switching with re-synchronization. In addition, since there is no Tsearch in CHO with L3 measurement in existing requirements [2], so we propose the following:

Proposal 17: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Proposal 18: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

  - Others

UE capabilities for (e)RedCap UE with FR1-NTN bands

In the last meeting, the WF on UE capabilities for (e)RedCap UE with FR1-NTN bands is copied as below [1]:

<Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us. It may be determined later.

In our view, the advantage of Proposal 1 is simplicity, convenience, and greater rationality, as currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified. Afterwards, we can define corresponding signaling for certain UE capabilities that need to be modified or added.

Observation 5: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified.

The company that support Proposal 2 is mainly concerned that some of the requirements defined for (e)RedCap UE in TN networks may not be supported or defined in NTN scenarios, and some of the requirements defined for ordinary UE in NTN networks may no longer be supported for (e)RedCap UE. Therefore, they want to define a new set of UE capabilities for (e)RedCap UE with FR1-NTN. However, this is all from the RAN4 perspective of defining requirements.

However, from the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded. In addition, most of the features of (e)RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole, as there may be duplication and waste.

Observation 6: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 7: Most of the features of (e) RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 19: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

CR skeleton

In the last meeting, RAN4 disccussed the specification structure for (e)RedCap UEs with FR1-NTN and agreed to use new sections and section number with new suffix X in the specification to capture requirements. It is worth mentioning that in order to reduce the size of the spec, if the requirements for (e)RedCap UEs with FR1-NTN can reuse the legacy, utilizing the reference method can be reused. The specific clauses that can utilize the reference method be can be discussed directly in the draft CRs.

According to the work plan and the progress, CR split should be done in this meeting. After examine the agreements and ongoing open issues so far, the following CR skeleton and notes can be a starting point for drafting the CRs:

Proposal 20: The initial CR work split for (e)RedCap UEs with FR1-NTN can be as follow.

| Clauses | Notes | Company |
| --- | --- | --- |
| 3.6.Y Applicability of requirements for RedCap UEs with NTN |  |  |
| 4.1 Cell Selection | No need to have new clauses, only clarify that the offset is also applicable to RedCap UEs with NTN |  |
| 4.2X Cell Re-selection for RedCap UEs with NTN |  |  |
| 5.1X Cell Re-selection for RedCap UEs with NTN | Refer to Clause 4.2X |  |
| 5.5X Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN |  |  |
| 5.7X Random access based Small Data Transmissions (RA-SDT) for RedCap |  |  |
| 6.1X Handover for RedCap UEs with NTN | Include handover，conditional handover and satellite switching with re-synchronization |  |
| 6.2X RRC Connection Mobility Control for RedCap UEs with NTN |  |  |
| 7.1X UE transmit timing for RedCap UEs with NTN | Refer to Clause 7.1C |  |
| 7.2X UE timer accuracy for RedCap UEs with NTN | Refer to Clause 7.2C |  |
| 7.3X Timing advance for RedCap UEs with NTN | Refer to Clause 7.3C |  |
| 8.1X Radio Link Monitoring for RedCap UEs with NTN |  |  |
| 8.5X Link Recovery Procedures for RedCap UEs with NTN |  |  |
| 8.6X Active BWP switch delay for RedCap UEs with NTN | Refer to Clause 8.6C |  |
| 8.10X Active TCI state switching delay for RedCap UEs with NTN | Refer to Clause 8.10C |  |
| 8.13X UE-specific CBW change for RedCap UEs with NTN | Refer to Clause 8.13C |  |
| 8.14X Pathloss reference signal switching delay for RedCap UEs with NTN | Refer to Clause 8.14C |  |
| 9.1X General measurement requirement for RedCap UEs with NTN |  |  |
| 9.2X NR intra-frequency measurements for RedCap UEs with NTN |  |  |
| 9.3X NR inter-frequency measurements for RedCap UEs with NTN |  |  |
| 9.5X L1-RSRP measurements for Reporting for RedCap UEs with NTN |  |  |
| 9.9X NR measurements for positioning for RedCap UEs with NTN |  |  |

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Observation 1: From RAN2’s signaling perspective, they will not to differtiate RedCap UEs and non-RedCap UEs, the agreement for TA validation in CG-SDT requirements only focus on RAN4’s requirements.

Proposal 1: For TA validation in CG-SDT requirements for (e)RedCap NTN, RAN4 does not need to send LS to RAN2 and the agreement reached in the last meeting only focus on RAN4’s requirements.

Observation 2: For Option 2, if multiple SMTCs do not overlap in the time domain and meet the minimum proximity distance, UE can measure all SMTCs in TDM without extending the overall measurement delay due to multiple SMTCs.

Observation 3: For Option 1, if the UE is configured with multiple SMTCs (such as 2 SMTC), even if the two SMTCs do not overlap, UE can only measure 1 SMTC and there is no measurement requirements for other SMTCs.

Proposal 2: There is no need to differentiate 1Rx and 2Rx RedCap UEs for this UE capability.

Proposal 3: If we don't see significant power saving gains or other benefits, prefer Option 2 to ensure that at least 2 SMTCs can be measured simultaneously:

- RedCap UE is madatory to support 2 parallel SMTC.

- ‘Parallel’ means the SMTCs are TDM with proximity distance.

- The measurement delay will not be extended due to the number of SMTCs that partially overlap with each other if multiple SMTCs meet the minimum proximity distance.

Proposal 4: If Option 1 is supported by RAN4, the UE will also need to optionally support the 2 parallel SMTCs are TDM with proximity distance.

Proposal 5: If we don't see significant power saving gains or other benefits, RAN4 doesn’t need to relax for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier and to reuse the legacy collision condition.

Proposal 6: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs, and it will be optional with capability signalling.

Proposal 7: For the number of cells and SSB, the following legacy requirement also can be reused for (e)RedCap UEs:

- For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Proposal 8: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

Proposal 9: SMTC adjustment in idle/inactive should be supported by (e)RedCap UEs with NTN.

Observation 4: For Location-based measurement trigger, the related UE capability was defined by RAN2 not by RAN4, and there is no requirement related to this feature in RAN4 spec.

Proposal 10: RAN4 not to further discuss applicability of location-based measurement report trigger.

Proposal 11: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional for (e)RedCap UE with FR1-NTN.

- RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle, the requirement defined for normal UEs with FR1-NTN can be reused.

Proposal 12: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

- For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN.

- For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for (e)RedCap UE with TN.

Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

Proposal 13: RAN4 to reuse the existing margin of RSRP/RSRQ defined for normal UEs with NTN for 1 Rx and 2 Rx (e)RedCap UE.

- the reselection criteria are met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

Proposal 14: For SDT requirements on RedCap NTN (including CG-SDT and RA-SDT), the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Proposal 15: Except for the TA validation and corresponding offset of threshold for CG-SDT and RA-SDT requirements, we think that the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

- For NTA in CG-SDT, it refers to the value specified in clause 7.1C.2.

- For requirements for SDT based on 2-step RA and 4-step RA, they refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

Proposal 16: The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE in NTN scenario, and at least the following requirements will be affected by HD-FDD:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

Proposal 17: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Proposal 18: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

Observation 5: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified.

Observation 6: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 7: Most of the features of (e) RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 19: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

Proposal 20: The initial CR work split for (e)RedCap UEs with FR1-NTN can be as follow.

| Clauses | Notes | Company |
| --- | --- | --- |
| 3.6.Y Applicability of requirements for RedCap UEs with NTN |  |  |
| 4.1 Cell Selection | No need to have new clauses, only clarify that the offset is also applicable to RedCap UEs with NTN |  |
| 4.2X Cell Re-selection for RedCap UEs with NTN |  |  |
| 5.1X Cell Re-selection for RedCap UEs with NTN | Refer to Clause 4.2X |  |
| 5.5X Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN |  |  |
| 5.7X Random access based Small Data Transmissions (RA-SDT) for RedCap |  |  |
| 6.1X Handover for RedCap UEs with NTN | Include handover，conditional handover and satellite switching with re-synchronization |  |
| 6.2X RRC Connection Mobility Control for RedCap UEs with NTN |  |  |
| 7.1X UE transmit timing for RedCap UEs with NTN | Refer to Clause 7.1C |  |
| 7.2X UE timer accuracy for RedCap UEs with NTN | Refer to Clause 7.2C |  |
| 7.3X Timing advance for RedCap UEs with NTN | Refer to Clause 7.3C |  |
| 8.1X Radio Link Monitoring for RedCap UEs with NTN |  |  |
| 8.5X Link Recovery Procedures for RedCap UEs with NTN |  |  |
| 8.6X Active BWP switch delay for RedCap UEs with NTN | Refer to Clause 8.6C |  |
| 8.10X Active TCI state switching delay for RedCap UEs with NTN | Refer to Clause 8.10C |  |
| 8.13X UE-specific CBW change for RedCap UEs with NTN | Refer to Clause 8.13C |  |
| 8.14X Pathloss reference signal switching delay for RedCap UEs with NTN | Refer to Clause 8.14C |  |
| 9.1X General measurement requirement for RedCap UEs with NTN |  |  |
| 9.2X NR intra-frequency measurements for RedCap UEs with NTN |  |  |
| 9.3X NR inter-frequency measurements for RedCap UEs with NTN |  |  |
| 9.5X L1-RSRP measurements for Reporting for RedCap UEs with NTN |  |  |
| 9.9X NR measurements for positioning for RedCap UEs with NTN |  |  |

- Reference

- R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- 3GPP TS 38.133, NR Requirements for support of radio resource management