3GPP TSG-RAN WG4 Meeting #114bis	                                                                                    R4-2503466

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	CATT

Title: 	Discussion on SBFD general issues

Agenda item:	7.23.1

Document for:	Approval

- Introduction

This contribution provides further discussion on the open issues for SBFD parameters according to the WF [1] in last RAN4 meeting. Channel BW, subband configuration and guard band size are discussed.

- Discussion

  - DU/UL subband declaration

Some of the agreements are strange that it’s the first time for base station to limit the declaration number and mandatory to test all of the cases. In our understanding, the declaration approach is invented to leave the flexibility for operation and manufacturer not to set any restrictions. For SBFD feature, it’s so new that large scale deployment is not available that the specification should have sufficient flexibility to make sure that choices are in the hands of operators to use the feature’s gain. So if the agreements can’t be reversed, at least the flexibility left on the table should be kept as much as possible.

For the declaration, current limitation for WA class is too strict. We think the limitation should not be applied for MR and LA because the implementation is not as difficult as WA class.

Proposal 1: No limitation for DL/UL sub-band declaration for MR class.

  - DL/UL subband configurations

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

In RAN4#114, there are no new agreements on issues related to the configuration of DL/UL subbands, and we can continue the analysis based on the WF [2] of RAN4#113.

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

From real deployment perspective, allowing more UL subband sizes would bring more flexibility to the implementation, so the maximum 3 specific UL subband sizes can be allowed.

Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

For the limitation of the WA class, our understanding is that the UL SB size of DUD and DU can be different because the size of DUD and DU SB naturally differ when the whole BS is the same. So we have the following proposal,

Proposal 3: DUD and DU pattern can have different UL SB size declaration.

The restriction should not be applied for MR because the implementation is not as difficult as WA class, so we have the following proposals,

Proposal 4: The restriction of DL/UL subband configurations is not applied to FR1 MR BS.

Proposal 5:  For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction is not applied to FR1 MR BS.

  - Restriction on guardband size

- Agreement:

  - Restriction on guardband size for SBFD:

    - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband)

    - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications.

    - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband:

    - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

Based on RAN4#112 agreement, the following options for restriction on guardband size are identified, while no further agreement achieved in RAN4#113 and RAN4#114:

From a standard point of view, in order to guarantee the benefits of introducing SBFD, there should be a limit on the maximum number of RBs for the guardband.

Proposal 6: There should be a limit on the maximum number of RBs for the guardband.

  - Other restriction on DL sub-band size

No further agreement achieved for restriction on DL subband size in RAN#114, so  we can continue the analysis based on the WF [2] of RAN4#113.

Way Forward:

- FFS the following restriction on DL subband size:

  - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

At least one DL subband size should be no less than the DL initial BWP size. In this case, SBFD operations can also be performed when system information is broadcasted.

Proposal 7: For SBFD, at least one DL subband size should be no less than the DL initial BWP size.

- Conclusion

This contribution provides our further analysis for SBFD general issues. We have the following proposals.

Proposal 1: No limitation for DL/UL sub-band declaration for MR class.

Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

Proposal 3: DUD and DU pattern can have different UL SB size declaration.

Proposal 4: The restriction of DL/UL subband configurations is not applied to FR1 MR BS.

Proposal 5:  For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction is not applied to FR1 MR BS.

Proposal 6: There should be a limit on the maximum number of RBs for the guardband.

Proposal 7: For SBFD, at least one DL subband size should be no less than the DL initial BWP size.

- References

[1] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114

[2] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113

[3] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112