3GPP TSG RAN WG4 Meeting #114bis		R4-2503473

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Clarification on the usage of 7MHz channel bandwidth

Agenda Item:	7.8.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#107, the WID on 7MHz channel bandwidth was revised with necessary updates [1]. And during the discussions in the first RAN4 meeting, several issues were raised out which may have impacts on the usage of 7MHz channel bandwidth:

1) DL initial BWP size may impact accessibility of a legacy UE [2]:

- FFS on if the maximum initial BWP size shall then be restricted to 5 MHz.

2) Addition of 7MHz channel bandwidth to a band should not alter the minimum channel bandwidth of this band, i.e., the minimum channel bandwidth of an applicable band should be either 3MHz or 5MHz [3]:

… RAN4 has discussed release independence of the 7 MHz channel bandwidth and have decided that there is no reason from a RAN4 perspective that 7 MHz cannot be release independent from Release 15 for a band with a minimum channel bandwidth of 3MHz or 5MHz. …

In our views, it is important to clarify these restrictions in RAN4 specs on the usage of 7MHz channel bandwidth.

# 2 Discussion

As identified in our previous paper, there is one scenario where a legacy UE may not access to a cell with a 7MHz channel bandwidth [4]:

- SIB1 carrier is 7MHz;

- Initial DL BWP size is 6MHz (larger than 5MHz but less than 7MHz),

In such a scenario, a legacy UE not supporting 7MHz channel bandwidth cannot access to this BS since the UE cannot find a channel bandwidth which is no larger than 7MHz but no less than 6MHz. Considering that 7MHz channel bandwidth is an optional feature, it is necessary to clarify in RAN4 specifications that a cell with SIB1 carrier as 7MHz and an initial DL BWP size between 5MHz and 7MHz is not accessible to a UE not supporting 7MHz channel bandwidth.

Proposal 1: RAN4 to clarify in specifications that a cell with SIB1 carrier as 7MHz and an initial DL BWP size between 5MHz and 7MHz is not accessible to a UE not supporting 7MHz channel bandwidth.

The minimum channel bandwidth of one band determines the multiplexing pattern between SSB and CORESET, including the numbers of PRB and symbols. As specified in TS 38.213 [5], only a limited number of minimum channel bandwidths are supported with the combination of SCSs of SSB and PDCCH, e.g., 3MHz, 5MHz, 10MHz etc. The following table illustrates an example for a band with SCS combination as {15k, 15k} and minimum channel bandwidth as 3MHz or 5MHz:

It is quite straightforward that for a band with a minimum channel bandwidth as 10MHz, the addition of 7MHz channel bandwidth support to this band will alter the minimum channel bandwidth from 10MHz to 7MHz, which has not been specified in RAN1 specs yet. And it is RAN4’s responsibility to specify and maintain the supported channel bandwidths for each band, hence, it is important to clarify in RAN4 specifications that the addition of 7MHz channel bandwidth to a band shall not alter the minimum channel bandwidth of the band.

Proposal 2: RAN4 to clarify in specifications that the addition of 7MHz channel bandwidth to a band shall not alter the minimum channel bandwidth of the band.

# 3 Conclusion

In this contribution, we have the following proposals on clarification on the usage of 7MHz channel bandwidth:

Proposal 1: RAN4 to clarify in specifications that a cell with SIB1 carrier as 7MHz and an initial DL BWP size between 5MHz and 7MHz is not accessible to a UE not supporting 7MHz channel bandwidth.

Proposal 2: RAN4 to clarify in specifications that the addition of 7MHz channel bandwidth to a band shall not alter the minimum channel bandwidth of the band.

# Reference

[1] RP-250373, “Revised WID on NR FR1 7 MHz Channel Bandwidth”, T-Mobile USA, China Telecom

[2] R4-2502875, “WF on requirements for 7MHz channel bandwidth”, Ericsson

[3] R4-2503017, “LS on Signalling for 7 MHz Channel Bandwidth”, T-Mobile USA

[4] R4-2501059, “Discussion on release independence for 7MHz channel bandwidth”, CATT

[5] TS 38.213 v18.5.0