3GPP TSG RAN WG4 Meeting #114bis		R4-2503474

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on UE RF requirements for 7MHz channel bandwidth

Agenda Item:	7.8.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

According to the workplan, one of the tasks in this meeting is to reach agreement on UE REFSENS requirement for band n5 and n26:

In this contribution, we derive and propose the corresponding values for REFSENS for band n5 and n26 respectively.

# 2 Discussion

For REFSENS requirement, it is well-known to derive based on the following equation:

Sensitivity = -174dBm + 10*log(RxBW) + NF + SNR +IM – diversity gain + additional relaxation

Where the following parameters are used for refarming NR bands below 6GHz

- NF: 9dB

- SNR: -1dB

- IM: 2.5dB

- Diversity gain: 3dB for 2Rx

- RxBW: corresponding to transmission bandwidth configuration

- Additional relaxation: band-specific parameter, such as Tx desens etc.

For band n5, all parameters except RxBW can be re-used and RxBW is 35 PRBs. Hence REFSENS is calculated as -96.53 dBm.

Proposal 1: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n5.

Proposal 2: Specify REFSENS for 7MHz channel bandwidth for band n5 as -96.53dBm.

Similarly, the same principle can be applied to band n26, and REFSENS is computed as -96.09dBm.

Proposal 3: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n26.

Proposal 4: Specify REFSENS for 7MHz channel bandwidth for band n26 as -96.09dBm.

# 3 Conclusion

In this contribution, we have the following proposals for UE RF requirements for 7MHz channel bandwidth for band n5 and n26:

Proposal 1: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n5.

Proposal 2: Specify REFSENS for 7MHz channel bandwidth for band n5 as -96.53dBm.

Proposal 3: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n26.

Proposal 4: Specify REFSENS for 7MHz channel bandwidth for band n26 as -96.09dBm.

# Reference

[1] R4-2500695, “Work plan on 7MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA