3GPP TSG RAN WG4 Meeting #114bis		R4-2503475

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on BS RF requirements for 7MHz channel bandwidth

Agenda Item:	7.8.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114, no much agreements were reached on BS RF requirements for 7MHz channel bandwidth [7]. In this contribution, we provide our proposals possible changes required for supporting 7MHz channel bandwidth in BS specifications.

# 2 Discussion

For REFSENS, the situation is different from that at US side, which means that REFSENS is not scaled by channel bandwidth. And in the current TS 38.104, 5/10/15MHz channel bandwidths have the same REFSENS for different BS types and classes. Hence it is reasonable to set the same level as that for 5/10/15MHz.

Proposal 1: RAN4 to specify REFSENS as the same as that for channel bandwidths 5/10/15MHz.

The below tables illustrate the changes accordingly.

BS type 1-C and 1-H

For wide area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -103.6 |
|  |  | G-FR1-A1-21 (Note 6) | -103.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |

For middle range BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -98.6 |
|  |  | G-FR1-A1-21 (Note 6) | -98.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
|  |  | G-FR1-A1-10 (Note 3) | -96.7 (Note 2) |

For local area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -95.6 |
|  |  | G-FR1-A1-21 (Note 6) | -95.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
|  |  | G-FR1-A1-10 (Note 3) | -93.7 (Note 2) |

BS type 1-O

For WA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -101.7 - ΔOTAREFSENS |

For MR BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -96.7 - ΔOTAREFSENS |

For LA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -93.7 - ΔOTAREFSENS |

For Rx dynamic range, the current specs indicates that the interfering signal mean power is scaled with channel bandwidth for conducted requirement, which means a scaling factor of 1.5dB applied for 7MHz channel bandwidth on top of that for 5MHz channel bandwidth, i.e., -81dBm for WA, -76dBm for MR and -73dBm for LA.

Proposal 2: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81dBm for WA, -76dBm for MR and -73dBm for LA.

The following tables show the required changes.

BS Type 1-C and 1-H:

For WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -73.6 | -84.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.7 | -81 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |

For MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (Note 2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -68.6 | -79.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -65.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.7 | -76 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |

For LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -65.6 | -76.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -62.7 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.7 | -73 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |

And similarly, for radiated requirements, the interfering signal mean power could also be scaled as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

Proposal 3: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

The below tables illustrate the required changes.

WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -82.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -81- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -79.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |

MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -76- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -74.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.4- ΔOTAREFSENS |  |  |

LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -74.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -73- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -71.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.4- ΔOTAREFSENS |  |  |

# 3 Conclusion

In this contribution, we have the following proposals on BS RF requirements for 7MHz channel bandwidth:

Proposal 1: RAN4 to specify REFSENS as the same as that for channel bandwidths 5/10/15MHz.

Proposal 2: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81dBm for WA, -76dBm for MR and -73dBm for LA.

Proposal 3: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

# Reference

[1] R4-2502870, “WF on requirements for 7MHz channel bandwidth”, Ericsson