3GPP TSG RAN WG4 Meeting #114bis		R4-2503484

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on REFSENS for LP-WUS

Agenda Item:	7.26.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114, an agreement was reached on the values of NF+IM for deriving REFSENS for FR1 bands [1]:

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB …

In this contribution we propose REFSENS for FR1 according to this agreement.

# 2 Discussion

The following equation is often used for deriving REFSENS:

REFENS (dBm) = -174dBm + 10*log(RxBW) + NF + SNR +IM – diversity gain

Where

- RxBW corresponds to transmission bandwidth configuration

- NF is noise figure

- SNR corresponds to detection performance of 1% MDR

- IM: implementation margin

- Diversity gain: 0dB since only 1Rx is assumed for LP-WUR

As seen in our previous paper, two SNRs, i.e., -3.3dB and -7dB could be used for OOK-based and OFDM-based receiver respectively, which means the values of (NF+SNR+IM) are 14.7dB and 6.5dB respectively. Hence REFSENS can be computed for different SCS with the above inputs.

- SCS = 15kHz, NRB = 11:

RxBW equals to 15kHz *12 * 11 = 1980kHz, and REFSENS = -174 + 10*log(1980k) + 14.7 or 6.5, i.e., -96.3dBm or -104.5dBm.

- SCS = 30kHz, NRB = 11

RxBW equals to 30kHz *12 * 11 = 3960kHz, and REFSENS = -174 + 10*log(3960k) + 14.7 or 6.5, i.e., -93.3dBm or -101.5dBm.

The below table summarizes REFSENS values for FR1:

| SCS(kHz)\REFSENS (dBm) | OOK-based receiver | OFDM-based receiver |
| --- | --- | --- |
| 15 | -96.3 | -104.5 |
| 30 | -93.3 | -101.5 |

Proposal: RAN4 to specify REFSENS for FR1 LP-WUR as the following table:

| SCS(kHz)\REFSENS (dBm) | OOK-based receiver | OFDM-based receiver |
| --- | --- | --- |
| 15 | -96.3 | -104.5 |
| 30 | -93.3 | -101.5 |

# 3 Conclusion

In this contribution, we have the following proposal on REFSENS for FR1 LP-WUR:

Proposal: RAN4 to specify REFSENS for FR1 LP-WUR as the following table:

| SCS(kHz)\REFSENS (dBm) | OOK-based receiver | OFDM-based receiver |
| --- | --- | --- |
| 15 | -96.3 | -104.5 |
| 30 | -93.3 | -101.5 |

# Reference

[1] R4-2503028, “WF on UE RF requirements for LP-WUS”, vivo

[2] R4-2501052, “Further discussion on REFSENS for LP-WUS”, CATT