3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503510

Wuhan, China, 7th – 11th April 2025

Source:	Qualcomm Incorporated

Title:	3 Tx NR specifications

Agenda item:	7.22.2

Document for:	Approval

# 1.	Introduction

In RAN4#114 there was further discussion on 3 TX and the unresolved items were captured in a WF [1]. In this paper we present further views on the remaining unresolved issues.

# 2. 	Discussion

In RAN4#114 there was discussion on the following:


<Sub-topic 1-1> Power class

<Way forward>: PC for 3Tx requirement

- For PC3 and PC2, specify requirements for FWA and handheld UE

- FFS on PC1.5 with 3Tx, will be decided in the next meeting

  - Option 1: Consider it in the next release

  - Option 2: Specify requirements in Rel-19

The 3Tx WI was started in RAN1#116 and to date the following 1-layer, 2-layer and 3-layer TPMIs have been defined.

In RAN4#112 it was agreed to support 1-layer and 3-layer transmit precoder matrices from the above list [2].

it was agreed that the 3Tx architecture cannot support PC1.5 using full power mode0 in Rel-19 as RAN4 assumes that the highest output power from a single PA is restricted to PC2 due to technology constraints.

Observation 1: Not possible to configure a UE for PC1.5 in mode 0 using a single PA based on current technology.

Another way of achieving higher output power is by using TxD where multi-Tx transmission is allowed. However, for 3Tx TxD is precluded as captured in the main session report [2] illustrated below.

Issue 1-1: PC for 3Tx requirement

Agreement:

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

Observation 2: Not possible for a UE to support higher power using TxD as TxD is precluded for 3 Tx.

In the last meeting there was discussion as to whether a UE can support PC1.5. It was suggested that by using the 3-layer identity matrix PC1.5 could be supported with the 3X26 dBm PA architecture. However, if a UE declares support for a certain power class it is insufficient for it to only support that power class using a subset of the supported transmit precoder matrices (TPM). It must support this power class using all supported TMPs including the single port DCI_format 0_0.

Observation 3: A UE must support a given power class for all supported transmit precoder matrices.

Observation 4: A UE must also support a given power class using the single port DCI format 0_0

As the UE cannot support PC1.5 in the single port DCI_format 0_0 the UE cannot declare this power class unless it has a single PA that can deliver PC1.5. As discussed earlier in RAN4 it has been deemed not possible to support PC1.5 using a single PA based on current PA technology.

Proposal 1: As 3Tx UE cannot output 29dBm for all supported transmit precoder matrices as well as DCI format 0_0 it cannot declare support for PC1.5.

In RAN4#114 the following was agreed:

<Way forward>: PC2 for 3Tx

- Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results

- Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

As mentioned above TxD is not allowed for the 3Tx, hence it is not possible to adopt the 2Tx PC2 MPR definition for 3Tx. Therefore, only the 1Tx MPR should be adopted.

Observation 5: As TxD is not allowed for 3Tx it is not possible to adopt the 2Tx PC2 MPR definition for 3Tx

Proposal 2: For 3Tx adopt the same MPR as the MPR for PC2 with 1Tx

# Conclusion

In this paper we further present our views on power class support for 3 Tx and MPR table creation and make the following observations and proposals:

Observation 1: Not possible to configure a UE for PC1.5 in mode 0 using a single PA based on current technology.

Observation 2: Not possible for a UE to support higher power using TxD as TxD is precluded for 3 Tx.

Observation 3: A UE must support a given power class for all supported transmit precoder matrices.

Observation 4: A UE must also support a given power class using the single port DCI format 0_0

Proposal 1: As 3Tx UE cannot output 29dBm for all supported transmit precoder matrices as well as DCI format 0_0 it cannot declare support for PC1.5.

Observation 5: As TxD is not allowed for 3Tx it is not possible to adopt the 2Tx PC2 MPR definition for 3Tx

Proposal 2: For 3Tx adopt the same MPR as the MPR for PC2 with 1Tx

# References

[1] R4-2503037, WF on UE RF requirements for MIMO enhancement, February 17th ‒ February 11th , 2025

[2] R4-2414310, WF on UE RF requirements for MIMO enhancement, August 19th – August 23rd, 2024