3GPP TSG RAN WG4 Meeting #114bis		 R4-2503565

# April 7 – 11, 2025, Wuhan, China

Source:	EchoStar, Boost Mobile Network

Title:	Views on open issues of n252/B252 UE RF requirements

Agenda item:			6.13.1, 6.14.1

Document for:	Discussion and Decision

# 1. Introduction

Taking into account the agreed Way Forward [1] from RAN4#114 meeting and based on further discussion with companies, we identify that the following are open issues for n252/B252 UE RF requirements:

- REFSENS for n252

- Out-of-band blocking requirement for n252/B252

- SS raster for n252

- AMPR for NTN-IoT for B252

- How to specify regulatory requirements for GNSS frequency range

This contribution indicates the EchoStar/Boost preference for the above open issues.

# 2. Discussion

## 2.1 UE REFSENS for n252

In the last meeting, several companies [2][3] proposed values for n252 REFSENS. The following two options were identified. Company proposing a relaxer value (option1) indicated that since n252 DL frequency range (i.e., 2180-2200 MHz) is within n256 DL frequency range (i.e., 2170-2200 MHz), and co-band duplexer for n252 and n256 is likely used, the Rx REFSENS performance of n252 is expected to be the same as n256 [2]. On the other hand, company proposing a tighter/more sensitive value (option 2) offered an opinion that a dedicated duplexer can be assumed [3].

Option 1: similar REFSENS as n256 (Mediatek [2])

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
| n252 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

Option 2: reuse Band23 design REFSENS (CATT [3])

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |  |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |  |
| n252 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |  |

We understand that REFSENS value depends on the assumption of the duplexer filter. As an operator, we prefer for RAN4 to adopt a tighter minimum requirement of REFSENS values.

Proposal 1:	RAN4 to discuss the above REFSENS values for n252 and decide in this meeting.

## 2.2 Out-of-band blocking for n252/B252

Several companies also provided their views in term of out-of-band blocking requirement in the last meeting [2][3][4][5][6]. Similar as of the REFSENS topic, the propose requirement is based on the assumption of what kind of the duplexer is applied.

The following summarizes different options identified from the last meeting:

Option 1: 	OOB blocking requirement based on duplexer filter cobanding n252 and n256 (MediaTek [2], CATT[3], ZTE [4])

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: Band n252 lower frequency ranges are modified to enable specific implementationsvoid NOTE 4: void |  |  |  |  |  |

Option 2: 	OOB blocking requirement based on duplexer cobanding bands n252, n256 and NR band n1/n65 (Huawei [5], Qualcomm[6])

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Option 3: OOB blocking requirement based on dedicated duplexer filter (similar to LTE Band 23) (CATT[3])

A wider frequency range may create worse blocking performance due to receiver de-sense. This effect may become more severe when HPUE is considered.

Proposal 2: 	RAN4 to discuss the above options and decide in this meeting.

## 2.3 SS Raster for n252

From previous discussions, it is the understanding that case A of SS block pattern will be supported/specified for n252. The open issue is whether we need to specify case B or C.
EchoStar confirms that since n256 channel bandwidth is larger than n252, then what is defined for n256 (case A) shall be applicable for n252. At this stage, case B or C is not needed.

Proposal 3:  	RAN4 to agree to specify Case A for n252.

## 2.4 AMPR for B252

In the last meeting, Mediatek in [7] provided AMPR simulation result B252 for the case of 200kHz channel bandwidth in B252 with UE-to-UE coexistence spurious requirement of -30dBm/MHz and -40 dBm/MHz.

From table 5, 6 in [7] as excerpted below, for 180kHZ CBW, 0 (zero) AMPR is observed for B252 for both UE-to-UE coexistence spurious requirement of -30dBm/MHz and -40 dBm/MHz.

Proposal 4: 	RAN4 to agree no A-MPR needs to be specified for B252 Cat. NB1/NB2 UEs for both UE-to-UE coexistence spurious requirements of -30dBm/MHz and -40 dBm/MHz.

Proposal 5: 	RAN4 to agree to capture proposal 4 in the TR 36.764

Furthermore, no A-MPR has been specified for B106 to meet the UE-to-UE coexistence spurious requirement of -30 dBm/MHz to protect n5. Based on that, no A-MPR is required for M1 UEs to meet the -30 dBm/MHz limit.

Proposal 6: 	RAN4 to agree that no A-MPR needs to be specified for B252 Cat.M1 IoT NTN UEs for both UE-to-UE coexistence spurious requirements of -30dBm/MHz and -40 dBm/MHz

Proposal 7: 	RAN4 to agree to capture proposal 6 in the TR 36.764

## 2.4 How to specify regulatory requirements for GNSS frequency range

In the last RAN4#114 meeting, the following way forward [1] was agreed to meet FCC requirements related to the protection of GNSS frequency range: 1559 – 1610 MHz (FCC 47 CFR § 25.216).

The following are two options on how to specify the protection for GNSS frequency:

Option 1: 	Associate n252 with the existing NS_02N that specifies the spurious emission requirement for GNSS frequency. 
This option provide operational flexibility, to include or not include NS_02N depending on region. In this option, the network may need to include several NS values (NS_02N for GNSS protection and NS_06N or NS_07N for n2/n25 protection). Considering frequency separation between n252 and GNSS frequency range, there is no additional AMPR to n252.

===== Change based on Option1 =====

================================

Option 2: 	To include the GNSS frequency as protected frequency range in the section/table Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence as protected frequency range, with the understanding that this requirement will be mandatory supported by the UE.

===== Change based on Option2 ======

================================

Proposal 8: 	RAN4 to discuss the above options and decide in this meeting.

# 3. Summary and proposal

This document addressed the open issues for n252/B252 UE RF requirements. EchoStar/Boost Mobile views and preference for each issues were clarified and the following were proposed:

Proposal 1:	RAN4 to discuss the following options of REFSENS values for n252 and decide in this meeting.

Option 1: similar REFSENS as n256

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
| n252 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

Option 2: 0.5dB tighter REFSENS

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |  |
| n252 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |  |

Proposal 2: 	RAN4 to discuss the following options out-of-band blocking requirement for n252/B252 and decide in this meeting.

Option 1: OOB blocking requirement based on duplexer filter cobanding n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: Band n252 lower frequency ranges are modified to enable specific implementationsvoid NOTE 4: void |  |  |  |  |  |

Option 2: OOB blocking requirement based on duplexer cobanding bands n252, n256 and NR band n1/n65

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Option 3: OOB blocking requirement based on dedicated duplexer filter (similar to LTE Band 23)

Proposal 3:  	RAN4 to agree to specify Case A for n252.  (At this stage, case B or C is not needed)

Proposal 4: 	RAN4 to agree no A-MPR needs to be specified for n252 NB1/NB2 UEs for both UE-to-UE coexistence spurious requirements of -30dBm/MHz and -40 dBm/MHz.

Proposal 5: 	RAN4 to agree to capture proposal 4 in the TR 36.764

Proposal 6: 	RAN4 to agree that no A-MPR needs to be specified for for n252 M1 IoT NTN UEs

Proposal 7: 	RAN4 to agree to capture proposal 6 in the TR 36.764

Proposal 8: 	RAN4 to discuss the following options to specify protection for GNSS frequency and decide in this meeting.

Option 1: 	Associating n252 with NS_02N (where spurious emission requirement is already specified)

Option 2: 	To include the GNSS frequency as protected frequency range in Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

# Reference

- R4-2502850, “WF on NR and IoT NTN bands”, Moderator (Aalyria)

- R4-2501321, “Discussion on NR-NTN n252 UE RF requirements”, Mediatek

- R4-2501002, “Discussion on UE RF requirements for the NTN new S-band”, CATT

- R4-2500602, “Discussion on system parameters and UE RF requirements for NR-NTN S-band”, ZTE

- R4-2501455, “Discussion on the NTN S band definition”, Huawei

- R4-2502005, “UE-to-UE coexistence and OOB blocking for n252”, Qualcomm

- R4-2501330, “Discussion on UE-to-UE coexistence of n252 UL interference with n2 DL and n25 DL”, Mediatek