3GPP TSG-RAN WG4 Meeting #114bis                           						         R4-2503583

# Wuhan, China, 7th – 12th April, 2025

Source: 	Apple

Title: 	On signalling support for type 4 UE

Agenda Item:	7.4.1.2

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings and the latest agreements were captured in the WF [1]. The following issues remain open for further study.

### <Issue 2-2-7: BS signalling framework>

### <Issue 2-2-7: BS signalling framework>

This contribution will give further analysis on these open issues and presents our views.

# Discussion

The following BS signalling aspect has been concluded as open issue for further study.

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated (Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

- Option 2 (Nokia)

- UE is Type 1 (2/CC), Type 1 (4/CC) or Type 4 depending on present Rel-19 BS signaling element value.

- When Rel-19 BS signaling is absent, UE follows Rel-18 BS signaling, i.e. is Type 4.

- Option 3 (Qualcomm)

1）When maxMIMO-layers ＞4

- UE is Type 1

2）When maxMIMO-layers = 4

- Evaluate following options

  - Type 1 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

  - Type 4 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

3）When maxMIMO-layers = 2, UE follows Rel-18

- Option 4 (ZTE)

1）If maxMIMO-layer > 4, Rel-19 Type 1 is assumed.

2）If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify Type 4 or Type 1. If maxMIMO-layer = 4 and NW signaling is not provided, Type 1 is assumed.

- Option 5 (Huawei)

1）When maxMIMO-layers ＞4

- UE is Type 1

2) When maxMIMO-layers = 4

- When BS signaling is absent, Type 4 is considered. (in accordance to the way Type 2 was specified in Rel-18) UE is Type 1.

- When BS signalling is present, Type 1 is considered. (in accordance to the way Type 1 was specified in Rel-18).

3) When maxMIMO-layers = 2, UE follows Rel-18

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated (Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

- Option 2 (Nokia)

- UE is Type 1 (2/CC), Type 1 (4/CC) or Type 4 depending on present Rel-19 BS signaling element value.

- When Rel-19 BS signaling is absent, UE follows Rel-18 BS signaling, i.e. is Type 4.

- Option 3 (Qualcomm)

1）When maxMIMO-layers ＞4

- UE is Type 1

2）When maxMIMO-layers = 4

- Evaluate following options

  - Type 1 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

  - Type 4 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

3）When maxMIMO-layers = 2, UE follows Rel-18

- Option 4 (ZTE)

1）If maxMIMO-layer > 4, Rel-19 Type 1 is assumed.

2）If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify Type 4 or Type 1. If maxMIMO-layer = 4 and NW signaling is not provided, Type 1 is assumed.

- Option 5 (Huawei)

1）When maxMIMO-layers ＞4

- UE is Type 1

2) When maxMIMO-layers = 4

- When BS signaling is absent, Type 4 is considered. (in accordance to the way Type 2 was specified in Rel-18) UE is Type 1.

- When BS signalling is present, Type 1 is considered. (in accordance to the way Type 1 was specified in Rel-18).

3) When maxMIMO-layers = 2, UE follows Rel-18

### <Issue 2-2-3: UE behavior or maxMIMO-layers>4/CC>

Agreement:

  - When maxMIMO-layers > 4, Type 1(collocated). No new BS signaling is needed.

### <Issue 2-2-3: UE behavior or maxMIMO-layers>4/CC>

Agreement:

  - When maxMIMO-layers > 4, Type 1(collocated). No new BS signaling is needed.

It seems common understanding that when maxMIMO-layer>4, UE is type 1 and no BS signaling is needed. This point was also concluded in issue 2-2-3 in [1]. In this case, the design is aligned with Rel-18 design.

Proposal 1: when maxMIMO-layers>4, UE is type 1. No new BS signalling is needed.

The remaining case to be further addressed is when maxMIMO-layers =4 and when maxMIMO-layers <=2.

InRel-18, a default UE behaviour in non-collocated condition is assumed and introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for the case of maxMIMO-layer value equal or less than 2 in. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

When maxMIMO-layers =4, the signalling for type 4 UE needs to well address the following 2 cases additionally. In this regard, Rel-18 signalling design (absent - present way) cannot well meet the purpose. And the suggested way [3] of “absent, present with value=0 (non-collocated), present with value=1 (collocated)” seems a good direction to go. Following this way, there seems no need to define a default UE behavior for type 4 as done in Rel-18 for type 2 since every status is clearly commanded by network signaling.

- Rel-19 type 4 capable UE behavior operate under Rel-18 network

As indicated in [3], assuming a Rel-19 UE indicates both Type-4 and Type-2, then Rel-18 NW only understand Type-2 and use maxMIMO-layers=4 to control UE work under collocated if following above design, but Rel-19 UE would see "new Rel-19 NW signaling =absent + maxMIMO-layers=4", then this UE misthink Non-collocated Type-4. Discrepancy between UE and gNB happens.

- Only type 2 capable Rel-19 UE behavior under Rel-19 network.

Since type 2 capability is reported, network just handle such UE by following Rel-18 design. No issue in our view.

Proposal 2: New Rel-19 BS signalling is needed for the case of maxMIMO-layer=4 and it can be defined as “present with value=0 (collocated), present with value=1 (non-collocated)”

Proposal 3: No need to define default UE behaviour for type 4 UE when maxMIMO-layer=4.

When maxmIMO-layer<=2, there are 3 cases,

Case 1: the UE only support type 2 but not type 4.

Case 2: the UE is type 4, but network is Rel-18 network. The network can only understand Rel-18 capability.

Case 3: the UE is type 4 and network is Rel-19 network. The UE is only

For case 1, the UE just follow type 2 design in Rel-18.

For case 2, the network uses Rel-18 network signalling to instruct UE retuning its RF to correct status. The issue is that there is no default UE behaviour for type 4 UE and more RF chains are supported than type 2 UE. The UE should configure at least 4RF chains to collocated scenario when the R18 network indication is sent and configure at lease 2 RF chains to non-collocated scenario when the Rel-18 network signalling is absent.

For case 3: The network can just deem the case when maxmIMO-layer=2 as a fall-back mode of the case when maxMIMO-layers=4 and network indication is present with 0.

Proposal 4: For maxMIMO-layer<=2, network use Rel-18 signalling to instruct the UE to correct status. FFS whether a default UE behaviour is needed or not for type 4 UE in legacy network and Rel-19 network.

Another issue to be considered is whether to consider switching between type 1 and type 4. Though it is not captured in the WF it will have impact on signaling design. Our view is that we think we need anything specifically for this as type 4 UE is intended for FWA use case. The RF configuration can only happen at intial connection and no need to expand the discussion to dynamically switching between collocated and non-collocated. Defining NW signaling elements for said “initial connection” roughly aligned with Rel-18 ones (accounting the differences for to 4L etc) and UE capabilities to indicate support of R19 non-collocated should suffice. We didn’t have anything for switching in Rel-18 either.

Proposal 5: Do not consider switching between type 1 and type 4.

# Summary

This contribution presented our consideration on the open issues for type 4 UE. The following observations and proposals are concluded.

Proposal 1: when maxMIMO-layers>4, UE is type 1. No new BS signalling is needed.

Proposal 2: New Rel-19 BS signalling is needed for the case of maxMIMO-layer=4 and it can be defined in the way of “absent, present with value=0 (collocated), present with value=1 (non-collocated)”

Proposal 3: No need to define default UE behaviour for type 4 UE when maxMIMO-layer=4.

Proposal 4: For maxMIMO-layer<=2, network use Rel-18 signalling to instruct the UE to correct status. FFS whether a default UE behaviour is needed or not for type 4 UE in legacy network and Rel-19 network.

Proposal 5: Do not consider switching between type 1 and type 4.

# References

[1] R4-2502924, WF on UE requirements for non-collocated scenario, KDDI, RAN4#114