3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503594

Wuhan, China, 7th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

In the previous meeting, following agreement was captured in WF[2].

Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement(RAN4#112bis): - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Agreement(RAN4#113): - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Agreement(RAN4#114): - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any.

In previous meeting, RAN4 had further clarification on the scope of RRM impact for SSB adaptation. If there is no further progress in other WG, possible scenario can be as follows:

- CONNECTED mode

- SSB adaptation for SCell

- SSB adaptation for NCD-SSB

- SSB adaptation in time-domain only for SSB periodicity

- Frequency location is same before and after SSB adaptation

- SSB index is same before and after SSB adaptation

L1 requirements for SSB adaptation

Following is the agreement for L1 measurement achevied in last two meetings.

Issue 1-2-1: L1 requirements at transition Agreement(RAN4#113): - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement(RAN4#113): - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement(RAN4#114): - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement(RAN4#114): - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

In the last two meeting, RAN4 achieved progress on L1 measurement for SSB adaptation, which includes RRM impacts for L1 measurement and transistion requirements. L1 measurement can be discussed, if new issue is identified, otherwise other issue such as L3 measurement should be focused.

Proposal 1: RAN4 to focus on L3 measurement requirement issue rather than L1 measurement requirement

L3 requirements for SSB adaptation

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. In RAN2#129, reply LS[3] RAN2 thanks RAN4 for the LS in R4-2416911 on SSB adaptation. Regarding the question of SMTC impacts due to SSB adaptation, RAN2 made the following agreements: Agreements on SSB adaptation - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Deactivated SCell L3 requirements for SSB adaptation

In the previous meeting, reusing the same requirements of OD-SSB Case#2 for deactivated SCell L3 requirements for SSB adaptation has been discussed. Before down select one of the option, Option 1 needs to be clarified first.

In deactivated SCell L3 measurement for OD-SSB Case#2, there can be two measurements method. The one is OD-SSB, and the other is combination of AO-SSB and OD-SSB. We think Option 1 only applies when only using OD-SSB. Furthermore, as stated in the agreement, it assumes OD-SSB periodicity is shorter than AO-SSB. Furthermore, in our understanding, SSB dapatation includes both transition of shorter to longer, and longer to shorter. If Option 1 is applied, no requirement is applied when the SSB periodicity is longer than before.

Hence, we think requirements for OD-SSB Case#2 cannot be directly reused. Some aspects needs to be clarified.

Proposal 2: Before down select deactivated SCell L3 requirements for SSB adaptation, Option 1 (reusing OD-SSB Case#2) needs to be clarified due to following aspects:

- Only OD-SSB based requriements can be applied (no combination of AO-SSB and OD-SSB)

- No clear requirments when the SSB periodicity is longer than before

L3 measurement requirements for SSB adaptation

In reply LS[3], RAN2 decide to keep SMTC based L3 RRM framework, and to introduce additional SMTC configuration according to SSB adaptation. Although the detail is not agreed yet, we think NW can indicate SMTC periodicity as well as SSB periodicity, which means that some how they can be aligned. In our understanding, subsequent to transition period, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity.

Proposal 3: Subsequent to transition period, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity.

For impacted RRM requirements, intra-frequency identification, intra-freqeucny with/without gap needs to be specified. Particulary regarding intra-frequency without gap requirement, there can be an issue with measurement gap. In general, intra-frequency measurement with gap is applied to enable UE to perform intra-frequency measurement when the SSB to be measured is not within the active bandwidth part. To measure within measurement gap, current requirement take maximum of MGRP and SMTC periodicity. However, when SSB adaptation is indicated, the periodicity of SSB or SMTC can be reduced, which may lead to partially overlapped with measurement gap. In consequence, since a UE may not find SSB to be measured outside of measurement gap, the UE may not leverage the frequent adapted SSB in this situation.

Figure 1 Intra-frequency measurement with gap for SSB adpatation

Proposal 4: Specifcy intra-frequency identification and intra-freqeuncy measurement w/wo gap requirement

- For intra-frequency with gap requirement, specify the requirement when intra-frequency measurement becomes partially overlapped with measurement gap

L3 transition requirements for SSB adaptation

For the L3 transition requirements for SSB adaptation, we prefer to reuse legacy requirement, since there are no reason not to have different requirement. As captured in 9.1.6 as below, usually ‘requirements with the longer delay apply’ is used.

9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within NCSG or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

Proposal 5: For L3 measurement period, requirements with longer delay is applied corresponding to the SSB before and after adaptation

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Proposal 1: RAN4 to focus on L3 measurement requirement issue rather than L1 measurement requirement

Proposal 2: Before down select deactivated SCell L3 requirements for SSB adaptation, Option 1 (reusing OD-SSB Case#2) needs to be clarified due to following aspects:

- Only OD-SSB based requriements can be applied (no combination of AO-SSB and OD-SSB)

- No clear requirments when the SSB periodicity is longer than before

Proposal 3: Subsequent to transition period, requirements apply based on the SMTC periodicity after transition for SMTC adaptation of periodicity

Proposal 4: Specifcy intra-frequency identification and intra-freqeuncy measurement w/wo gap requirement

- For intra-frequency with gap requirement, specify the requirement when intra-frequency measurement becomes partially overlapped with measurement gap

Proposal 5: For L3 measurement period, requirements with longer delay is applied corresponding to the SSB before and after adaptation

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei

[3] R2-2501484, “Reply LS on SSB adaptation”, Apple