3GPP TSG-RAN4 Meeting #114bis	R4-2503597

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title: 	TP to TR 38.787 on SL intra-band NC CA A-MPR

Agenda Item:	7.2.1

Document for:	Approval

- Introduction

This contribution is a text proposal to capture PSFCH and S-SSB A-MPR simulation results for SL intra-band non-contiguous CA.

It is based on the approved TP (R4-2502154) in last RAN4#114 meeting.

- Text proposal

<Start of text proposal>

6.2.3	UE additional maximum output power reduction

For the A-MPR analysis to comply the US regulation, RAN4 agreed to study and specipy the related PC2 A-MPR requirements with NS_52 after final FCC regulation for the additional emission limits is released in US.

For the A-MPR analysis to comply the EU reguration, RAN4 need to study and specify the PC3/PC2 A-MPR requirements with NS_33 based on the merged the interested companies results.

The required EU regulations for SL CA UE with 10MHz CBW are listed as below

- Additional Spectrum mask: When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.2.3-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

- Additional emission requirements:

Table 6.2.3-2: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5945 | -30 | 1 |  |
| Frequency range | 5835 |  | - | 5855 | -30 | 1 |  |
| NOTE 1: Void. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Void. |  |  |  |  |  |  |  |

- Additional requirements to protect CEN DSRC:

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

For A-MPR with NS_33, only 10+10MHz channel bandwidth combination is evaluated.

6.2.3.1	A-MPR for PSSCH/PSCCH transmission

6.2.3.1.1	Simulation results from LG Electronics (R4-2502153/R4-2503595)

Figure 6.2.3.1.1-0 shows A-SEM and A-SE for 7 carrier frequencies, 5860, 5870, 5880, 5890, 5900, 5910, and 5920MHz.

Figure 6.2.3.1.1-0: A-SEM and A-SE for "NS_33"

Table 6.2.3.1.1-1 and Figure 6.2.3.1.1-1 show all aggregated 10+10 configuration of SL intra-band NC CA in which EU regulation can apply with ‘NS_33’.

Table 6.2.3.1.1-1: All aggregated 10+10 configuration of SL intra-band NC CA for "NS_33"

| Aggregated 10+10 Conf.# | description | Gap [MHz] | Fc(CC1) [MHz] | Fc(CC2) [MHz] | Remarks (A-SEM & A-SE aspect) |
| --- | --- | --- | --- | --- | --- |
| 1 | S0_10_G10_10 | 10 | 5860 | 5880 | = Scen#11 |
| 2 | S10_10_G10_10 | 10 | 5870 | 5890 | = Scen#10 |
| 3 | S20_10_G10_10 | 10 | 5880 | 5900 |  |
| 4 | S0_10_G20_10 | 20 | 5860 | 5890 | = Scen#13 |
| 5 | S10_10_G20_10 | 20 | 5870 | 5900 | = Scen#12 |
| 6 | S0_10_G30_10 | 30 | 5860 | 5900 | = Scen#14 |
| 7 | S10_10_G30_10 | 30 | 5870 | 5910 |  |
| 8 | S0_10_G40_10 | 40 | 5860 | 5910 | = Scen#15 |
| 9 | S0_10_G50_10 | 50 | 5860 | 5920 |  |
| 10 | S30_10_G10_10 | 10 | 5890 | 5910 |  |
| 11 | S40_10_G10_10 | 10 | 5900 | 5920 |  |
| 12 | S20_10_G20_10 | 20 | 5880 | 5910 |  |
| 13 | S30_10_G20_10 | 20 | 5890 | 5920 |  |
| 14 | S20_10_G30_10 | 30 | 5880 | 5920 |  |
| 15 | S10_10_G40_10 | 40 | 5870 | 5920 |  |

Figure 6.2.3.1.1-1: All aggregated 10+10 configuration of SL intra-band NC CA for "NS_33"

For A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 6.2.3.1.1-2 are considered.

Table 6.2.3.1.1-2: SL non-contiguous CA AMPR evaluation scenarios

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | B | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- | --- |
| Conf#1, Conf#4 ~Conf#9 in 6.2.3.1.1-1 | 1 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30A | 15 |
|  | 2 | 10RB30 | 10RB12 | 3.6 | SEfreq_-30A | 15 |
|  | 3 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30A | 15 |
|  | 4 | 12RB30 | 12RB10 | 4.32 | SEfreq_-30A | 15 |
|  | 5 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30A | 15 |
|  | 6 | 15RB30 | 15RB7 | 5.4 | SEfreq_-30A | 15 |
|  | 7 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30A | 15 |
|  | 8 | 20RB30 | 20RB2 | 7.2 | SEfreq_-30A | 15 |
|  | 9 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30A | 15 |
|  | 10 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30A | 15 |
|  | 11 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30A | 15 |
|  | 12 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30A | 15 |
|  | 13 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30A | 15 |
|  | 14 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30A | 15 |
|  | 15 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30A | 15 |
|  | 16 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30A | 15 |
|  | 17 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30A | 15 |
|  | 18 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30A | 15 |
|  | 19 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30A | 15 |
|  | 20 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30A | 15 |
| Conf#2, Conf#3 in 6.2.3.1.1-1 | 1 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30A | 15 |
|  | 2 | 10RB30 | 10RB12 | 3.6 | SEMfreq_-13A | 15 |
|  | 3 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30A | 15 |
|  | 4 | 12RB30 | 12RB10 | 4.32 | SEMfreq_-13A | 15 |
|  | 5 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30A | 15 |
|  | 6 | 15RB30 | 15RB7 | 5.4 | SEMfreq_-13A | 15 |
|  | 7 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30A | 15 |
|  | 8 | 20RB30 | 20RB2 | 7.2 | SEMfreq_-13A | 15 |
|  | 9 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30A | 15 |
|  | 10 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30A | 15 |
|  | 11 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30A | 15 |
|  | 12 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30A | 15 |
|  | 13 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30A | 15 |
|  | 14 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30A | 15 |
|  | 15 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30A | 15 |
|  | 16 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30A | 15 |
|  | 17 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30A | 15 |
|  | 18 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30A | 15 |
|  | 19 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30A | 15 |
|  | 20 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30A | 15 |

Here, SEMfreq_-13A and SEfreq_-30A in Table 6.2.3.1.1-1 are addressed with similar approach as NR.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to SE -30 dBm / MHz.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to SE -30 dBm / MHz.

-      SEM-37/100kHz,low = Threshold frequency where lower additional spectral emission mask below the lower channel corresponds to  -37dBm/100kHz, as specified in Clause 6.5E.2.3.1 of TS38.101-1.

-	SEM-37/100kHz,high = Threshold frequency where upper additional spectral emission mask above the upper channel corresponds to  -37dBm/100kHz, as specified in Clause 6.5E.2.3.1 of TS38.101-1.

-	Flow = 5855 MHz

-	Fhigh = 5925 MHz

SEMfreq_-13 = AND (FIM3,low_block,low > SEM-13,low,  FIM3,high_block,high < SEM-13,high )

SEfreq_-30  = OR(FIM3,low_block,low ≤ SEM-13,low ,  SEM-13,high ≤ FIM3,high_block,high )

SEMfreq_-13A = AND (FIM3,low_block,low > SEM-37,low , FIM3,high_block,high < SEM-37,high )

SEfreq_-30A = OR(FIM3,low_block,low ≤ MAX(SEM-13,low, Flow ),  MIN(SEM-13,high, Fhigh) ≤ FIM3,high_block,high )

Figure 6.2.3.1.1-2, Figure 6.2.3.1.1-3, Figure 6.2.3.1.1-4, and Figure 6.2.3.1.1-5 show the example of Frequency for FIM3,low_block,low & FIM3,high_block,high, such as, SEMfreq_-13/SEfreq_-30A, SEfreq_-30/SEfreq_-30A, SEMfreq_-13/SEMfreq-13A, SEfreq_-30/SEfreq_-30A respectively. Here, carrier frequency combinations of {5860, 5880MHz} and {5870, 5890} are assumed.

Figure 6.2.3.1.1-2: Example of SEMfreq_-13/SEfreq_-30A with {5860, 5880}MHz

Figure 6.2.3.1.1-3: Example of SEfreq_-30/SEfreq_-30A with {5860, 5880}MHz

Figure 6.2.3.1.1-4: Example of SEMfreq_-13/SEMfreq-13A with {5870, 5890}MHz

Figure 6.2.3.1.1-5: Example of SEfreq_-30/SEfreq_-30A with {5870, 5890}MHz

The A-MPR requirements for NS_33 should be specified to meet -37dBm/100kHz and -30dBm/1MHz.

- PC3 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-13 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.1.1-4, Table 6.2.3.1.1-5, and Table 6.2.3.1.1-6 show the AMPR simulation results.

Table 6.2.3.1.1-4: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 16.0 | 14.5 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 16.4 | 14.1 | 16.0 | 13.6 | 16.0 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '64QAM' | 16.0 | 14.5 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.4 |
|  | '256QAM' | 16.5 | 14.1 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.3 | 13.6 | 10.4 | 13.7 | 10.4 | 13.2 | 10.9 | 12.7 | 11.8 |
|  | '16QAM' | 14.1 | 11.4 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 12.3 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 12.3 |
|  | '256QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 11.8 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '16QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 9.0 |
|  | '64QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '256QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.4 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.4 | 10.4 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.4 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '16QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '64QAM' | 14.1 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '256QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.5 | 16.0 | 14.0 | 15.0 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '16QAM' | 16.0 | 14.5 | 16.0 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '64QAM' | 16.0 | 14.5 | 16.0 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '256QAM' | 16.0 | 14.5 | 15.5 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 12.2 |
|  | '16QAM' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 11.8 |
|  | '64QAM' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 11.8 |
|  | '256QAM' | 14.1 | 11.3 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.8 | 12.7 | 11.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.1 | 14.5 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '16QAM' | 14.2 | 14.0 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '64QAM' | 14.2 | 14.5 | 13.7 | 13.6 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '256QAM' | 14.1 | 14.5 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.4 |
|  | '64QAM' | 11.3 | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.4 |
|  | '256QAM' | 11.3 | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.4 | 9.4 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.5 | 14.1 | 15.5 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.5 | 11.4 |
|  | '16QAM' | 15.5 | 14.1 | 15.5 | 13.7 | 15.0 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '64QAM' | 15.5 | 14.1 | 15.1 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.1 | 11.4 |
|  | '256QAM' | 15.5 | 14.1 | 15.1 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.1 | 11.9 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '16QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '256QAM' | 13.6 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | '16QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | '64QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.8 |
|  | '256QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.9 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.9 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 12.8 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 13.3 | 15.0 | 12.3 | 14.5 | 11.4 |
|  | '64QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 13.3 | 15.0 | 12.3 | 14.6 | 11.4 |
|  | '256QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 12.8 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.4 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '16QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.6 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '64QAM' | 14.1 | 11.3 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '256QAM' | 14.1 | 10.9 | 13.6 | 10.9 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 11.8 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 15.5 | 14.5 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '64QAM' | 15.5 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '256QAM' | 15.5 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '16QAM' | 13.6 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '256QAM' | 14.1 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |

Table 6.2.3.1.1-5: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 17.7 | 15.3 | 17.7 | 14.4 | 17.2 | 14.0 | 16.2 | 13.1 | 15.8 | 12.2 |
|  | '16QAM' | 17.7 | 15.3 | 17.7 | 14.9 | 17.2 | 14.0 | 16.7 | 13.1 | 15.8 | 12.2 |
|  | '64QAM' | 17.7 | 15.3 | 17.7 | 14.4 | 17.2 | 14.0 | 16.7 | 13.1 | 15.8 | 12.2 |
|  | '256QAM' | 17.7 | 15.3 | 17.2 | 14.4 | 17.2 | 14.0 | 16.3 | 13.1 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '16QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '64QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '256QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.4 | 12.1 | 14.0 | 13.1 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '16QAM' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '64QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '256QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '16QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
|  | '64QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '16QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '64QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '256QAM' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.3 | 10.7 |
|  | '16QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '64QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 11.2 | 11.7 | 11.2 | 11.2 | 11.2 | 10.7 | 10.7 | 10.7 | 10.8 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.7 | 15.3 | 16.7 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | '16QAM' | 17.7 | 15.3 | 17.2 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | '64QAM' | 17.7 | 15.3 | 16.7 | 14.8 | 16.7 | 13.9 | 16.3 | 13.0 | 15.8 | 12.6 |
|  | '256QAM' | 17.7 | 15.3 | 17.2 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '16QAM' | 15.4 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '64QAM' | 14.9 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '256QAM' | 15.4 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '16QAM' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '64QAM' | 14.9 | 15.3 | 14.4 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '256QAM' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '16QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '64QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.7 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.8 | 14.9 | 16.8 | 14.5 | 16.3 | 14.0 | 15.8 | 13.1 | 15.8 | 12.2 |
|  | '16QAM' | 17.2 | 14.9 | 16.8 | 14.5 | 16.3 | 14.0 | 15.8 | 13.1 | 15.3 | 12.2 |
|  | '64QAM' | 17.3 | 14.9 | 16.3 | 14.5 | 16.3 | 14.0 | 16.3 | 13.1 | 15.3 | 12.2 |
|  | '256QAM' | 17.3 | 14.9 | 16.8 | 14.5 | 16.8 | 14.0 | 15.8 | 13.1 | 15.3 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.2 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |
|  | '16QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.0 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '64QAM' | 14.9 | 12.2 | 14.4 | 11.2 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '256QAM' | 14.9 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 13.5 | 13.1 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '16QAM' | 14.9 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '64QAM' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '256QAM' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.1 | 13.1 | 12.6 | 12.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 11.2 | 10.7 | 10.7 | 10.3 | 10.8 |
|  | '16QAM' | 12.1 | 11.7 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.3 | 10.3 |
|  | '64QAM' | 12.1 | 12.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.8 | 10.8 |
|  | '256QAM' | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.7 | 15.0 | 16.8 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.6 |
|  | '16QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.2 |
|  | '64QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.2 |
|  | '256QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 14.1 | 16.3 | 13.1 | 15.8 | 12.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '16QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '64QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '256QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.2 | 15.3 | 17.2 | 14.4 | 16.7 | 14.0 | 15.8 | 13.1 | 15.8 | 12.6 |
|  | '16QAM' | 17.2 | 15.3 | 16.8 | 14.4 | 16.7 | 14.0 | 16.2 | 13.1 | 15.8 | 12.6 |
|  | '64QAM' | 17.2 | 15.3 | 16.8 | 14.8 | 16.7 | 14.0 | 16.2 | 13.1 | 15.3 | 12.6 |
|  | '256QAM' | 16.8 | 15.3 | 16.8 | 14.4 | 16.7 | 14.0 | 15.8 | 13.1 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.9 | 11.7 | 14.4 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '16QAM' | 14.9 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 14.0 | 13.1 |
|  | '64QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.0 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |
|  | '256QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |

Table 6.2.3.1.1-6: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 13.7 | 9.0 | 14.1 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '16QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '64QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.1 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '16QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '64QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '256QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '16QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 6.6 | 4.8 | 6.2 | 5.2 |
|  | '64QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 7.1 | 4.8 | 6.2 | 5.2 |
|  | '256QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.7 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '16QAM' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '64QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '256QAM' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.2 | 5.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.7 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.6 | 9.4 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | '16QAM' | 14.6 | 9.4 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | '64QAM' | 14.6 | 9.0 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 14.6 | 9.0 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.9 |
|  | '16QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 9.0 | 11.8 | 9.9 | 11.8 | 10.8 |
|  | '64QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.9 |
|  | '256QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.5 | 9.4 | 8.0 | 8.0 | 7.6 | 7.1 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | '16QAM' | 9.0 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 7.1 | 6.2 | 6.1 |
|  | '64QAM' | 9.5 | 9.5 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.1 | 6.1 |
|  | '256QAM' | 9.5 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 12.7 | 9.0 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '64QAM' | 12.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '256QAM' | 12.7 | 9.4 | 12.7 | 8.0 | 13.2 | 7.5 | 12.8 | 7.1 | 12.7 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '16QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '64QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '256QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.0 | 9.4 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | '16QAM' | 9.0 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.6 | 6.1 |
|  | '64QAM' | 9.0 | 9.4 | 8.5 | 8.0 | 7.6 | 7.1 | 6.6 | 6.6 | 6.1 | 6.1 |
|  | '256QAM' | 9.5 | 9.5 | 8.1 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 13.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 13.2 | 9.4 | 13.6 | 8.0 | 13.6 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '64QAM' | 13.2 | 9.5 | 13.7 | 8.0 | 13.2 | 7.5 | 13.2 | 7.1 | 12.7 | 6.1 |
|  | '256QAM' | 13.7 | 9.4 | 13.6 | 8.0 | 13.2 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.3 | 6.1 | 11.8 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.4 |
|  | '16QAM' | 12.7 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.9 |
|  | '64QAM' | 12.2 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.4 |
|  | '256QAM' | 12.2 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.8 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 13.7 | 9.0 | 13.7 | 8.0 | 13.6 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 13.7 | 9.0 | 13.7 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '64QAM' | 13.7 | 8.9 | 13.6 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 13.7 | 9.5 | 13.6 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
|  | '16QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
|  | '64QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.8 |
|  | '256QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.8 |

From Table 6.2.3.1.1-4, Table 6.2.3.1.1-5, and Table 6.2.3.1.1-6, AMPR is almost same for all modulation order. Same AMPR can be defined regardless of modulation order. And, the simulation results can be categorized with 2 cases because the result of each case is very similar.

  - Case 1 : S0_10_G10_10, S0_10_G20_10, S0_10_G30_10, S0_10_G40_10, S0_10_G50_10

  - Case 2 : S10_10_G10_10, S20_10_G10_10, S10_10_G20_10, S10_10_G30_10

Here,

Case 1 corresponds to that any Carrier Frequency in 2CC combination is either 5860MHz or 5920MHz, as follows.

  - {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920}

Case 2 corresponds to that any Carrier Frequency in 2CC combination is other than either 5860MHz or 5920MHz, as follows.

  - {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910}

Or,

If any Carrier Frequency in 2CC combination is either 5860MHz or 5920MHz

Case 1

Else

Case 2

Table 6.2.3.1.1-7, Table 6.2.3.1.1-8, and Table 6.2.3.1.1-9 show the maximum value of simulation results for SL non-contiguous CA.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 6.2.3.1.1-7 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 1x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 14.5 | 16.5 |
|  | 5.04 ≤ B < 10.08 | 13.2 | 16.0 |
|  | 10.08 ≤ B |  | 14.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 13.3 | 14.5 |
|  | 5.04 ≤ B < 10.08 | 10.0 | 13.2 |
|  | 10.08 ≤ B |  | 11.3 |

Table 6.2.3.1.1-8 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 2x20dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 15.3 | 17.7 |
|  | 5.04 ≤ B < 10.08 | 14.0 | 17.2 |
|  | 10.08 ≤ B |  | 15.4 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 14.1 | 15.3 |
|  | 5.04 ≤ B < 10.08 | 10.8 | 14.0 |
|  | 10.08 ≤ B |  | 12.2 |

Table 6.2.3.1.1-9 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 2x20dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 9.0 | 14.6 |
|  | 5.04 ≤ B < 10.08 | 7.5 | 14.1 |
|  | 10.08 ≤ B |  | 13.2 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 8.1 | 9.5 |
|  | 5.04 ≤ B < 10.08 | 5.7 | 7.6 |
|  | 10.08 ≤ B |  | 6.1 |

The AMPR can be proposed as Table 6.2.3.1.1-10, Table 6.2.3.1.1-11, and Table 6.2.3.1.1-12 based on the simulation results when considering implementation margin.

Table 6.2.3.1.1-10 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 17.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.0 | ≤ 19.0 |
|  | 10.08 ≤ B |  | ≤ 17.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 16.5 | ≤ 17.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 16.0 |
|  | 10.08 ≤ B |  | ≤ 15.0 |

Table 6.2.3.1.1-11 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.5 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 17.0 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.5 | ≤ 16.5 |
|  | 10.08 ≤ B |  | ≤ 15.5 |

Table 6.2.3.1.1-12 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 13.5 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 12.5 | ≤ 17.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 13.0 | ≤ 14.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 10.5 | ≤ 12.5 |
|  | 10.08 ≤ B |  | ≤ 11.5 |

- PC2 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-1 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.1.1-14, Table 6.2.3.1.1-15, and Table 6.2.3.1.1-16 show the AMPR simulation results for the SL non-contiguous CA.

Table 6.2.3.1.1-14: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 19.4 | 16.5 | 18.9 | 15.6 | 18.9 | 15.2 | 17.9 | 14.3 | 17.0 | 13.8 |
|  | '16QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.5 | 13.8 |
|  | '64QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.5 | 13.8 |
|  | '256QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.0 | 12.9 | 16.6 | 12.9 | 16.1 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '16QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 16.1 | 12.4 | 15.7 | 13.3 | 15.2 | 14.3 |
|  | '256QAM' | 17.0 | 12.9 | 16.6 | 12.9 | 16.1 | 12.4 | 15.6 | 13.3 | 15.2 | 14.3 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '16QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '64QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '256QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '16QAM' | 13.3 | 12.5 | 12.9 | 12.4 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '256QAM' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 11.5 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '16QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '64QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '256QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 11.5 |
|  | '16QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '256QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.5 | 18.9 | 16.0 | 18.4 | 15.1 | 17.9 | 14.2 | 17.5 | 13.8 |
|  | '16QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 18.4 | 15.6 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | '64QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 17.9 | 15.6 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | '256QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 17.9 | 15.1 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '16QAM' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 13.4 | 16.1 | 12.9 | 16.1 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '256QAM' | 16.6 | 13.4 | 16.1 | 12.4 | 15.7 | 12.4 | 15.7 | 13.3 | 15.2 | 14.3 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.2 | 14.2 | 13.8 | 13.8 |
|  | '16QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.2 | 14.2 | 13.8 | 13.8 |
|  | '64QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.6 | 14.3 | 14.2 | 13.8 | 13.8 |
|  | '256QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.7 | 14.2 | 13.8 | 13.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '16QAM' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '256QAM' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 11.5 | 12.0 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.0 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '16QAM' | 18.5 | 16.6 | 18.4 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '64QAM' | 18.5 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '256QAM' | 18.5 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.5 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 15.2 | 14.3 |
|  | '16QAM' | 16.1 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 14.7 | 14.3 |
|  | '64QAM' | 16.5 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 14.7 | 14.3 |
|  | '256QAM' | 16.5 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.2 | 12.9 | 15.2 | 14.3 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | '16QAM' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.9 |
|  | '64QAM' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | '256QAM' | 16.2 | 16.6 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '16QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 12.0 |
|  | '64QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 11.5 |
|  | '256QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 11.5 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.2 | 18.9 | 15.8 | 18.5 | 15.3 | 17.9 | 14.3 | 17.0 | 13.4 |
|  | '16QAM' | 18.9 | 16.2 | 18.9 | 15.8 | 18.5 | 15.3 | 18.0 | 14.3 | 17.0 | 13.9 |
|  | '64QAM' | 19.4 | 16.2 | 18.9 | 15.7 | 18.4 | 14.8 | 17.9 | 14.3 | 17.0 | 13.9 |
|  | '256QAM' | 18.9 | 16.2 | 18.9 | 15.7 | 18.0 | 14.8 | 18.0 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.0 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '16QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 16.1 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 13.4 | 16.1 | 12.9 | 15.6 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '256QAM' | 16.5 | 13.4 | 16.1 | 12.9 | 16.1 | 12.4 | 15.2 | 13.4 | 15.2 | 14.3 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.5 | 18.4 | 15.6 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '16QAM' | 18.9 | 16.5 | 18.4 | 15.6 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '64QAM' | 18.9 | 16.5 | 18.4 | 15.6 | 18.4 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '256QAM' | 18.5 | 16.5 | 18.4 | 16.1 | 17.9 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '16QAM' | 16.5 | 12.9 | 16.1 | 12.4 | 15.2 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '64QAM' | 16.6 | 12.9 | 16.1 | 12.4 | 16.1 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '256QAM' | 16.6 | 12.9 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |

Table 6.2.3.1.1-15: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 19.7 | 16.8 | 19.2 | 16.4 | 19.2 | 15.5 | 18.7 | 14.6 | 17.8 | 14.1 |
|  | '16QAM' | 19.7 | 16.8 | 19.2 | 16.4 | 19.2 | 15.5 | 18.2 | 14.6 | 18.2 | 14.1 |
|  | '64QAM' | 20.2 | 17.3 | 19.7 | 16.4 | 19.2 | 15.5 | 18.7 | 14.6 | 17.8 | 14.1 |
|  | '256QAM' | 20.2 | 17.3 | 19.2 | 16.4 | 19.2 | 15.5 | 18.3 | 14.6 | 17.8 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 16.0 | 14.6 |
|  | '64QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.4 | 13.7 | 15.5 | 15.0 |
|  | '256QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.5 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '16QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '64QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 14.6 | 10.0 | 14.1 | 11.8 |
|  | '256QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 14.6 | 10.0 | 14.1 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.7 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '16QAM' | 16.9 | 15.6 | 16.4 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '64QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '256QAM' | 16.5 | 15.6 | 16.0 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.7 | 17.2 | 19.2 | 16.3 | 18.7 | 15.9 | 18.2 | 15.0 | 17.8 | 14.1 |
|  | '16QAM' | 19.7 | 17.3 | 19.2 | 16.3 | 18.7 | 15.4 | 18.2 | 15.0 | 17.3 | 14.1 |
|  | '64QAM' | 19.7 | 17.3 | 19.2 | 16.3 | 19.2 | 15.9 | 18.2 | 15.0 | 17.8 | 14.1 |
|  | '256QAM' | 19.2 | 17.2 | 19.2 | 16.3 | 19.2 | 15.9 | 18.2 | 15.0 | 17.3 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '256QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | '16QAM' | 16.9 | 17.2 | 16.5 | 16.3 | 15.5 | 15.9 | 15.0 | 14.6 | 14.1 | 14.1 |
|  | '64QAM' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | '256QAM' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.7 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.3 | 16.9 | 18.3 | 16.5 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | '16QAM' | 18.8 | 16.9 | 18.3 | 16.4 | 18.3 | 15.5 | 17.8 | 14.6 | 17.3 | 14.2 |
|  | '64QAM' | 19.2 | 16.9 | 18.8 | 16.5 | 18.7 | 15.5 | 17.8 | 14.6 | 17.3 | 14.2 |
|  | '256QAM' | 18.8 | 16.9 | 18.3 | 16.4 | 18.3 | 15.5 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.9 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 15.5 | 13.7 | 15.1 | 14.6 |
|  | '16QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.4 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 14.6 | 14.6 | 14.1 | 14.2 |
|  | '16QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 15.0 | 14.6 | 14.1 | 14.2 |
|  | '64QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 14.6 | 14.6 | 14.1 | 14.2 |
|  | '256QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.6 | 15.0 | 14.6 | 14.1 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.6 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.7 | 16.5 | 19.2 | 16.1 | 18.8 | 15.6 | 18.3 | 14.7 | 17.8 | 14.2 |
|  | '16QAM' | 19.7 | 17.0 | 19.2 | 16.1 | 19.2 | 15.6 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | '64QAM' | 19.7 | 16.5 | 19.2 | 16.1 | 19.2 | 15.6 | 18.3 | 14.7 | 17.3 | 14.2 |
|  | '256QAM' | 19.7 | 17.0 | 19.2 | 16.0 | 19.2 | 15.6 | 18.3 | 14.7 | 17.8 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.8 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
|  | '64QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 17.3 | 13.7 | 16.8 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.2 | 16.8 | 18.8 | 16.4 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | '16QAM' | 19.2 | 16.9 | 18.8 | 16.4 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | '64QAM' | 19.2 | 17.3 | 18.7 | 16.4 | 18.7 | 15.5 | 18.2 | 15.1 | 17.3 | 14.2 |
|  | '256QAM' | 19.2 | 16.8 | 18.7 | 16.4 | 19.2 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '16QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |

Table 6.2.3.1.1-16: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 15.6 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 16.1 | 11.5 | 15.6 | 10.5 | 15.6 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.7 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 16.1 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.8 |
|  | '16QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.4 |
|  | '64QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.9 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '16QAM' | 11.5 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '64QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.6 | 6.3 | 7.7 | 6.7 |
|  | '256QAM' | 11.9 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.8 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '16QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '64QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.6 | 6.3 | 7.7 | 6.7 |
|  | '256QAM' | 11.5 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.8 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '16QAM' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '64QAM' | 16.6 | 11.4 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '256QAM' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.6 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.7 | 7.7 | 14.2 | 9.1 | 14.3 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '16QAM' | 14.7 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '64QAM' | 14.7 | 7.7 | 14.2 | 9.1 | 14.2 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '256QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 14.2 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '16QAM' | 12.0 | 11.5 | 10.5 | 10.5 | 9.1 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '64QAM' | 11.5 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '256QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.5 | 8.2 | 8.6 | 7.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.1 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '16QAM' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.1 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '64QAM' | 14.7 | 11.5 | 14.7 | 10.5 | 14.7 | 9.5 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '256QAM' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.6 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.8 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.9 | 11.4 | 12.9 | 12.4 |
|  | '16QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.8 | 11.4 | 12.9 | 12.4 |
|  | '64QAM' | 13.8 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.9 | 11.4 | 12.9 | 12.4 |
|  | '256QAM' | 13.8 | 7.7 | 13.3 | 8.6 | 13.3 | 10.0 | 12.9 | 11.5 | 12.9 | 12.4 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '16QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '64QAM' | 12.0 | 11.5 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '256QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.6 | 8.6 | 7.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.6 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.1 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.5 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.6 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '16QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '64QAM' | 14.2 | 7.6 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.7 | 11.9 | 15.6 | 10.5 | 15.2 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 15.6 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.6 | 11.4 | 15.6 | 10.5 | 15.6 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 15.7 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 14.2 | 8.6 | 13.8 | 10.5 | 13.3 | 11.4 | 13.3 | 12.4 |
|  | '16QAM' | 14.2 | 7.2 | 14.2 | 8.6 | 13.8 | 10.5 | 13.3 | 11.4 | 13.3 | 12.4 |
|  | '64QAM' | 14.2 | 7.7 | 14.3 | 8.6 | 13.8 | 10.5 | 13.3 | 11.5 | 12.9 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 14.3 | 8.6 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.4 |

Table 6.2.3.1.1-17, Table 6.2.3.1.1-18, and Table 6.2.3.1.1-19 show the maximum value of simulation results.

Table 6.2.3.1.1-17 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 1x26dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 16.5 | 19.4 |
|  | 5.04 ≤ B < 10.08 | 15.2 | 18.9 |
|  | 10.08 ≤ B |  | 17.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 15.3 | 16.6 |
|  | 5.04 ≤ B < 10.08 | 12.0 | 15.6 |
|  | 10.08 ≤ B |  | 13.4 |

Table 6.2.3.1.1-18 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 2x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 17.3 | 20.2 |
|  | 5.04 ≤ B < 10.08 | 15.5 | 19.2 |
|  | 10.08 ≤ B |  | 17.3 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 15.6 | 17.2 |
|  | 5.04 ≤ B < 10.08 | 12.3 | 15.9 |
|  | 10.08 ≤ B |  | 13.7 |

Table 6.2.3.1.1-19 : PSSCH/PSCCH AMPR simulation results for SL non-ontiguous CA with 2x23dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 11.5 | 16.6 |
|  | 5.04 ≤ B < 10.08 | 9.6 | 16.1 |
|  | 10.08 ≤ B |  | 14.7 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 10.6 | 12.0 |
|  | 5.04 ≤ B < 10.08 | 7.2 | 9.6 |
|  | 10.08 ≤ B |  | 7.7 |

The A-MPR can be proposed as Table 6.2.3.1.1-20, Table 6.2.3.1.1-21, and Table 6.2.3.1.1-22 based on the simulation results when considering implementation margin.

Table 6.2.3.1.1-20 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 19.0 | ≤ 22.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 17.5 | ≤ 21.5 |
|  | 10.08 ≤ B |  | ≤ 19.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 19.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 14.5 | ≤ 18.0 |
|  | 10.08 ≤ B |  | ≤ 16.0 |

Table 6.2.3.1.1-21 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 20.0 | ≤ 22.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 18.5 | ≤ 22.0 |
|  | 10.08 ≤ B |  | ≤ 20.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 15.0 | ≤ 18.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |

Table 6.2.3.1.1-22 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13A | SEfreq_-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 15.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 14.0 | ≤ 15.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 11.5 | ≤ 13.0 |
|  | 10.08 ≤ B |  | ≤ 12.0 |

6.2.3.2	A-MPR for PSFCH transmission

6.2.3.2.1	Simulation results from LG Electronics (R4-2503595)

For PSFCH A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 6.2.3.2.1-2 are considered.

Table 6.2.3.2.1-2: SL non-contiguous CA AMPR evaluation scenarios (PSFCH)

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| Conf#2, Conf#3 in Table 6.2.3.1.1-1 | 1 | 1RB51 | 1RB0 | SEMfreq_-13A | 15 |
|  | 2 | 1RB45 | 1RB6 | SEMfreq_-13A | 15 |
|  | 3 | 1RB39 | 1RB12 | SEMfreq_-13A | 15 |
|  | 4 | 1RB35 | 1RB16 | SEMfreq_-13A | 15 |
|  | 5 | 1RB26 | 1RB25 | SEfreq_-30A | 15 |
|  | 6 | 1RB21 | 1RB30 | SEfreq_-30A | 15 |
|  | 7 | 1RB13 | 1RB38 | SEfreq_-30A | 15 |
|  | 8 | 1RB0 | 1RB51 | SEfreq_-30A | 15 |
|  | 9 | 1RB23 | 1RB0 | SEMfreq_-13A | 30 |
|  | 10 | 1RB21 | 1RB2 | SEMfreq_-13A | 30 |
|  | 11 | 1RB19 | 1RB4 | SEMfreq_-13A | 30 |
|  | 12 | 1RB17 | 1RB6 | SEMfreq_-13A | 30 |
|  | 13 | 1RB12 | 1RB11 | SEfreq_-30A | 30 |
|  | 14 | 1RB8 | 1RB15 | SEfreq_-30A | 30 |
|  | 15 | 1RB4 | 1RB19 | SEfreq_-30A | 30 |
|  | 16 | 1RB0 | 1RB23 | SEfreq_-30A | 30 |
|  | 17 | 1RB10 | 1RB0 | SEMfreq_-13A | 60 |
|  | 18 | 1RB9 | 1RB1 | SEMfreq_-13A | 60 |
|  | 19 | 1RB8 | 1RB2 | SEMfreq_-13A | 60 |
|  | 20 | 1RB7 | 1RB3 | SEfreq_-30A | 60 |
|  | 21 | 1RB5 | 1RB5 | SEfreq_-30A | 60 |
|  | 22 | 1RB3 | 1RB7 | SEfreq_-30A | 60 |
|  | 23 | 1RB1 | 1RB9 | SEfreq_-30A | 60 |
|  | 24 | 1RB0 | 1RB10 | SEfreq_-30A | 60 |
| Conf#1, Conf#4 ~ Conf#9 in Table 6.2.3.1.1-1 | 1 | 1RB51 | 1RB0 | SEfreq_-30A | 15 |
|  | 2 | 1RB45 | 1RB6 | SEfreq_-30A | 15 |
|  | 3 | 1RB39 | 1RB12 | SEfreq_-30A | 15 |
|  | 4 | 1RB35 | 1RB16 | SEfreq_-30A | 15 |
|  | 5 | 1RB26 | 1RB25 | SEfreq_-30A | 15 |
|  | 6 | 1RB21 | 1RB30 | SEfreq_-30A | 15 |
|  | 7 | 1RB13 | 1RB38 | SEfreq_-30A | 15 |
|  | 8 | 1RB0 | 1RB51 | SEfreq_-30A | 15 |
|  | 9 | 1RB23 | 1RB0 | SEfreq_-30A | 30 |
|  | 10 | 1RB21 | 1RB2 | SEfreq_-30A | 30 |
|  | 11 | 1RB19 | 1RB4 | SEfreq_-30A | 30 |
|  | 12 | 1RB17 | 1RB6 | SEfreq_-30A | 30 |
|  | 13 | 1RB12 | 1RB11 | SEfreq_-30A | 30 |
|  | 14 | 1RB8 | 1RB15 | SEfreq_-30A | 30 |
|  | 15 | 1RB4 | 1RB19 | SEfreq_-30A | 30 |
|  | 16 | 1RB0 | 1RB23 | SEfreq_-30A | 30 |
|  | 17 | 1RB10 | 1RB0 | SEfreq_-30A | 60 |
|  | 18 | 1RB9 | 1RB1 | SEfreq_-30A | 60 |
|  | 19 | 1RB8 | 1RB2 | SEfreq_-30A | 60 |
|  | 20 | 1RB7 | 1RB3 | SEfreq_-30A | 60 |
|  | 21 | 1RB5 | 1RB5 | SEfreq_-30A | 60 |
|  | 22 | 1RB3 | 1RB7 | SEfreq_-30A | 60 |
|  | 23 | 1RB1 | 1RB9 | SEfreq_-30A | 60 |
|  | 24 | 1RB0 | 1RB10 | SEfreq_-30A | 60 |

- PC3 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-13 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.2.1-3, Table 6.2.3.2.1-4, and Table 6.2.3.2.1-5 show the AMPR simulation results.

Table 6.2.3.2.1-3: PSFCH AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 12.8 | 12.3 | 12.4 | 12.9 | 12.4 | 12.3 | 12.3 | 16.5 |
| S10_10_G10_10 | 9.1 | 8.6 | 9.7 | 12.9 | 12.4 | 12.3 | 12.3 | 12.4 |
| S20_10_G10_10 | 9.1 | 8.6 | 9.7 | 12.9 | 12.4 | 12.3 | 12.3 | 12.4 |
| S0_10_G20_10 | 12.4 | 12.3 | 12.3 | 12.4 | 12.4 | 12.3 | 12.2 | 16.5 |
| S10_10_G20_10 | 12.4 | 12.3 | 12.3 | 12.4 | 12.4 | 12.3 | 12.2 | 12.4 |
| S0_10_G30_10 | 12.4 | 12.4 | 12.4 | 12.4 | 12.3 | 12.3 | 12.3 | 16.5 |
| S10_10_G40_10 | 12.4 | 12.4 | 12.4 | 12.4 | 12.3 | 12.3 | 12.3 | 12.4 |
| S0_10_G40_10 | 12.5 | 12.4 | 12.4 | 12.4 | 12.3 | 12.2 | 12.3 | 16.5 |
| S0_10_G50_10 | 12.4 | 12.3 | 12.4 | 12.4 | 12.4 | 12.3 | 12.2 | 16.0 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.6 | 11.9 | 12.4 | 12.3 | 10.9 | 12.3 | 12.4 | 17.9 |
| S10_10_G10_10 | 8.3 | 7.8 | 8.7 | 10.0 | 10.9 | 12.3 | 12.4 | 11.0 |
| S20_10_G10_10 | 8.3 | 7.8 | 8.7 | 10.0 | 10.9 | 12.3 | 12.4 | 12.4 |
| S0_10_G20_10 | 12.1 | 12.4 | 12.4 | 11.9 | 11.3 | 12.3 | 12.0 | 17.9 |
| S10_10_G20_10 | 12.1 | 12.4 | 12.4 | 11.9 | 11.8 | 12.3 | 12.0 | 12.0 |
| S0_10_G30_10 | 12.5 | 11.9 | 11.9 | 12.3 | 12.2 | 12.3 | 12.4 | 16.5 |
| S10_10_G40_10 | 12.5 | 11.9 | 11.9 | 12.3 | 12.2 | 12.3 | 12.4 | 12.4 |
| S0_10_G40_10 | 12.4 | 12.3 | 12.2 | 12.2 | 12.3 | 12.4 | 12.4 | 17.4 |
| S0_10_G50_10 | 12.4 | 11.8 | 12.2 | 12.3 | 11.9 | 12.0 | 12.4 | 18.3 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 11.5 | 14.6 | 18.3 |
| S10_10_G10_10 | 6.2 | 6.2 | 9.1 | 11.0 | 11.5 | 11.1 | 11.4 | 11.4 |
| S20_10_G10_10 | 6.2 | 6.2 | 9.1 | 11.0 | 11.5 | 11.1 | 11.4 | 11.4 |
| S0_10_G20_10 | 11.5 | 11.3 | 11.3 | 11.3 | 10.9 | 11.5 | 14.7 | 16.6 |
| S10_10_G20_10 | 11.5 | 11.3 | 11.3 | 11.3 | 10.9 | 11.5 | 11.5 | 11.5 |
| S0_10_G30_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 12.0 | 14.6 | 16.5 |
| S10_10_G40_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 11.5 | 11.5 | 11.0 |
| S0_10_G40_10 | 11.5 | 11.4 | 11.4 | 11.0 | 11.5 | 12.0 | 14.6 | 16.4 |
| S0_10_G50_10 | 11.5 | 11.5 | 11.5 | 11.1 | 11.5 | 11.9 | 14.1 | 16.4 |

Table 6.2.3.2.1-4: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 13.0 | 12.5 | 12.6 | 13.1 | 12.6 | 12.5 | 12.5 | 17.2 |
| S10_10_G10_10 | 8.9 | 8.9 | 10.3 | 13.1 | 12.6 | 12.5 | 12.5 | 12.6 |
| S20_10_G10_10 | 8.9 | 8.9 | 10.3 | 13.1 | 12.6 | 12.5 | 12.5 | 12.6 |
| S0_10_G20_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.6 | 12.5 | 17.2 |
| S10_10_G20_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.6 | 12.5 | 12.6 |
| S0_10_G30_10 | 12.6 | 12.6 | 12.7 | 12.7 | 12.6 | 12.5 | 12.5 | 17.2 |
| S10_10_G40_10 | 12.6 | 12.6 | 12.7 | 12.7 | 12.6 | 12.5 | 12.5 | 12.6 |
| S0_10_G40_10 | 12.7 | 12.7 | 12.6 | 12.6 | 12.5 | 12.5 | 12.6 | 17.2 |
| S0_10_G50_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.5 | 12.5 | 16.7 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.8 | 12.2 | 12.6 | 12.5 | 11.1 | 12.6 | 12.7 | 18.6 |
| S10_10_G10_10 | 8.6 | 8.0 | 8.9 | 10.7 | 11.1 | 12.6 | 12.7 | 11.2 |
| S20_10_G10_10 | 8.6 | 8.0 | 8.9 | 10.7 | 11.1 | 12.6 | 12.7 | 12.6 |
| S0_10_G20_10 | 12.3 | 12.7 | 12.6 | 12.1 | 11.6 | 12.5 | 12.2 | 18.6 |
| S10_10_G20_10 | 12.3 | 12.7 | 12.6 | 12.1 | 12.0 | 12.5 | 12.2 | 12.2 |
| S0_10_G30_10 | 12.7 | 12.1 | 12.1 | 12.5 | 12.5 | 12.6 | 12.7 | 17.2 |
| S10_10_G40_10 | 12.7 | 12.1 | 12.1 | 12.5 | 12.5 | 12.6 | 12.7 | 12.6 |
| S0_10_G40_10 | 12.6 | 12.5 | 12.5 | 12.5 | 12.5 | 12.6 | 13.1 | 18.1 |
| S0_10_G50_10 | 12.6 | 12.0 | 12.5 | 12.5 | 12.1 | 12.2 | 12.6 | 18.6 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 11.6 | 11.6 | 11.6 | 11.7 | 11.7 | 12.2 | 15.3 | 19.0 |
| S10_10_G10_10 | 7.0 | 6.9 | 9.8 | 11.2 | 11.7 | 11.3 | 11.7 | 11.7 |
| S20_10_G10_10 | 7.0 | 6.9 | 9.8 | 11.2 | 11.7 | 11.3 | 11.7 | 11.7 |
| S0_10_G20_10 | 11.7 | 11.6 | 11.6 | 11.6 | 11.2 | 12.2 | 15.4 | 17.3 |
| S10_10_G20_10 | 11.7 | 11.6 | 11.6 | 11.6 | 11.2 | 11.7 | 11.7 | 11.8 |
| S0_10_G30_10 | 11.7 | 11.6 | 11.6 | 11.7 | 11.7 | 12.7 | 15.3 | 17.2 |
| S10_10_G40_10 | 11.7 | 11.6 | 11.6 | 11.7 | 11.7 | 11.8 | 11.7 | 11.3 |
| S0_10_G40_10 | 11.7 | 11.7 | 11.7 | 11.3 | 11.8 | 12.7 | 14.8 | 17.1 |
| S0_10_G50_10 | 11.7 | 11.7 | 11.7 | 11.3 | 11.8 | 12.6 | 14.8 | 17.1 |

Table 6.2.3.2.1-5: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 10.6 | 10.5 | 10.6 | 12.0 | 10.9 | 10.6 | 10.6 | 14.4 |
| S10_10_G10_10 | 5.5 | 5.4 | 9.2 | 12.5 | 11.4 | 11.5 | 11.6 | 11.8 |
| S20_10_G10_10 | 5.5 | 5.4 | 9.2 | 12.5 | 11.4 | 11.5 | 11.6 | 11.8 |
| S0_10_G20_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 15.3 |
| S10_10_G20_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 11.2 |
| S0_10_G30_10 | 10.6 | 11.0 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 12.1 |
| S10_10_G40_10 | 10.6 | 11.0 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 10.8 |
| S0_10_G40_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 13.1 |
| S0_10_G50_10 | 11.1 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 14.0 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 10.6 | 10.2 | 8.7 | 9.6 | 10.2 | 10.5 | 10.6 | 15.8 |
| S10_10_G10_10 | 5.5 | 5.6 | 5.4 | 10.5 | 11.2 | 11.5 | 10.6 | 11.3 |
| S20_10_G10_10 | 5.5 | 5.6 | 5.4 | 10.5 | 11.2 | 11.5 | 11.5 | 11.3 |
| S0_10_G20_10 | 10.6 | 10.7 | 10.1 | 9.1 | 10.2 | 10.5 | 11.0 | 16.7 |
| S10_10_G20_10 | 10.6 | 10.7 | 10.1 | 9.1 | 10.2 | 10.5 | 10.6 | 10.8 |
| S0_10_G30_10 | 10.2 | 10.2 | 10.6 | 10.5 | 10.7 | 10.5 | 10.6 | 14.9 |
| S10_10_G40_10 | 10.2 | 10.2 | 10.6 | 10.5 | 10.7 | 10.5 | 10.6 | 10.8 |
| S0_10_G40_10 | 10.6 | 10.7 | 10.1 | 10.5 | 10.7 | 10.5 | 11.5 | 17.2 |
| S0_10_G50_10 | 10.6 | 10.7 | 10.6 | 10.5 | 10.2 | 10.5 | 10.6 | 15.8 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 7.5 | 7.0 | 8.4 | 8.8 | 8.2 | 11.5 | 16.1 | 19.1 |
| S10_10_G10_10 | 5.6 | 5.6 | 5.6 | 10.2 | 9.6 | 9.7 | 9.8 | 10.5 |
| S20_10_G10_10 | 5.6 | 5.6 | 5.6 | 10.2 | 9.6 | 9.7 | 9.8 | 10.5 |
| S0_10_G20_10 | 8.4 | 8.8 | 8.4 | 8.3 | 8.2 | 10.1 | 14.3 | 16.8 |
| S10_10_G20_10 | 8.4 | 8.8 | 8.4 | 8.3 | 8.6 | 8.8 | 8.4 | 8.6 |
| S0_10_G30_10 | 8.9 | 8.4 | 8.4 | 8.3 | 8.2 | 10.6 | 14.8 | 17.2 |
| S10_10_G40_10 | 8.9 | 8.4 | 8.4 | 8.3 | 8.2 | 8.8 | 8.4 | 9.0 |
| S0_10_G40_10 | 8.9 | 8.4 | 8.8 | 8.8 | 7.7 | 10.6 | 14.8 | 17.2 |
| S0_10_G50_10 | 8.4 | 8.4 | 8.4 | 7.8 | 8.6 | 11.0 | 15.2 | 17.7 |

Table 6.2.3.2.1-6 shows the maximum value of simulation results for SL non-contiguous CA.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 6.2.3.2.1-6 : PC3 PSFCH AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | 12.9 | 12.4 | 11.4 | 16.5 | 18.3 | 18.3 |
|  | 2x20dBm + 1LO | 13.1 | 12.6 | 11.6 | 17.2 | 18.6 | 19.0 |
|  | 2x20dBm + 2LO | 12.0 | 10.6 | 8.4 | 15.3 | 17.2 | 19.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | 12.9 | 10.0 | 9.1 | 12.4 | 12.5 | 11.5 |
|  | 2x20dBm + 1LO | 13.1 | 10.7 | 9.8 | 12.7 | 12.7 | 11.8 |
|  | 2x20dBm + 2LO | 12.5 | 10.5 | 5.6 | 11.8 | 11.5 | 10.5 |

The AMPR can be proposed as Table 6.2.3.2.1-7 based on the simulation results when considering implementation margin.

Table 6.2.3.2.1-7 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 | ≤ 20.5 | ≤ 22.0 | ≤ 22.5 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 | ≤ 20.5 | ≤ 22.0 | ≤ 22.5 |
|  | 2x20dBm + 2LO | ≤ 16.5 | ≤ 15.5 | ≤ 14.0 | ≤ 19.0 | ≤ 20.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 18.0 | ≤ 15.0 | ≤ 13.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 15.0 | ≤ 13.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
|  | 2x20dBm + 2LO | ≤ 17.0 | ≤ 14.5 | ≤ 9.0 | ≤ 16.0 | ≤ 15.5 | ≤ 14.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |  |  |

From Table 6.2.3.2.1-6 and Table 6.2.3.2.1-7, the following is observed.

Observation: For PC3 PSFCH, A-MPR of higher SCS is larger than that of smaller SCS for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, PSFCH A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

PC3 PSFCH A-MPR can be proposed as follows.

For PC3 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 6.2.3.2.1-8.

For PC3 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 6.2.3.2.1-8.

Table 6.2.3.2.1-8 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 2LO | ≤ 16.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 2LO | ≤ 17.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

- PC2 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-1 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.2.1-9, Table 6.2.3.2.1-10, and Table 6.2.3.2.1-11 show the AMPR simulation results for the SL non-contiguous CA.

Table 6.2.3.2.1-9: PSFCH AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 14.3 | 14.3 | 14.4 | 14.9 | 14.4 | 14.3 | 14.3 | 19.9 |
| S10_10_G10_10 | 10.7 | 10.7 | 11.7 | 14.9 | 14.4 | 14.3 | 14.3 | 14.4 |
| S20_10_G10_10 | 10.7 | 10.7 | 11.7 | 14.9 | 14.4 | 14.3 | 14.3 | 14.4 |
| S0_10_G20_10 | 14.4 | 14.3 | 14.3 | 13.9 | 14.4 | 14.3 | 14.2 | 19.9 |
| S10_10_G20_10 | 14.4 | 14.3 | 14.3 | 13.9 | 14.4 | 14.3 | 14.2 | 14.4 |
| S0_10_G30_10 | 14.4 | 13.9 | 14.4 | 14.4 | 14.3 | 14.3 | 14.3 | 19.9 |
| S10_10_G40_10 | 14.4 | 13.9 | 14.4 | 14.4 | 14.3 | 14.3 | 14.3 | 14.4 |
| S0_10_G40_10 | 14.0 | 14.0 | 14.4 | 14.4 | 14.3 | 14.2 | 14.3 | 19.5 |
| S0_10_G50_10 | 14.4 | 14.3 | 14.4 | 14.4 | 14.4 | 14.3 | 14.2 | 19.4 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 13.6 | 13.9 | 14.3 | 14.3 | 12.9 | 13.9 | 14.4 | 20.8 |
| S10_10_G10_10 | 9.9 | 9.8 | 10.2 | 12.5 | 12.9 | 13.9 | 14.0 | 13.0 |
| S20_10_G10_10 | 9.9 | 9.8 | 10.2 | 12.5 | 12.9 | 13.9 | 14.0 | 13.9 |
| S0_10_G20_10 | 14.1 | 14.0 | 14.0 | 13.9 | 13.3 | 14.3 | 14.0 | 20.9 |
| S10_10_G20_10 | 14.1 | 14.0 | 14.0 | 13.9 | 13.8 | 14.3 | 14.0 | 14.0 |
| S0_10_G30_10 | 14.0 | 13.9 | 13.9 | 13.8 | 13.8 | 13.9 | 14.9 | 19.4 |
| S10_10_G40_10 | 14.0 | 13.9 | 13.9 | 13.8 | 13.8 | 13.9 | 14.0 | 13.9 |
| S0_10_G40_10 | 14.4 | 14.3 | 13.8 | 13.8 | 14.3 | 14.0 | 15.3 | 20.3 |
| S0_10_G50_10 | 13.9 | 13.8 | 14.2 | 14.3 | 13.9 | 14.0 | 13.9 | 21.3 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 12.9 | 13.3 | 13.4 | 13.4 | 13.5 | 14.4 | 18.0 | 21.3 |
| S10_10_G10_10 | 8.8 | 8.7 | 11.5 | 13.0 | 13.5 | 13.1 | 13.4 | 13.4 |
| S20_10_G10_10 | 8.8 | 8.7 | 11.5 | 13.0 | 13.5 | 13.1 | 13.4 | 13.4 |
| S0_10_G20_10 | 13.0 | 13.3 | 13.3 | 13.3 | 12.9 | 14.4 | 17.6 | 19.5 |
| S10_10_G20_10 | 13.0 | 13.3 | 13.3 | 13.3 | 12.9 | 13.5 | 13.5 | 13.1 |
| S0_10_G30_10 | 13.4 | 13.3 | 13.4 | 13.4 | 13.0 | 14.9 | 17.6 | 19.4 |
| S10_10_G40_10 | 13.4 | 13.3 | 13.4 | 13.4 | 13.0 | 13.5 | 13.4 | 13.0 |
| S0_10_G40_10 | 13.5 | 13.4 | 13.4 | 13.0 | 13.1 | 14.9 | 17.5 | 19.4 |
| S0_10_G50_10 | 13.0 | 13.5 | 13.5 | 13.1 | 13.5 | 14.8 | 17.0 | 19.4 |

Table 6.2.3.2.1-10: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 15.0 | 15.0 | 14.6 | 15.1 | 14.6 | 14.9 | 14.9 | 20.1 |
| S10_10_G10_10 | 11.3 | 10.9 | 12.3 | 15.1 | 14.6 | 14.9 | 14.9 | 15.1 |
| S20_10_G10_10 | 11.3 | 10.9 | 12.3 | 15.1 | 14.6 | 14.9 | 14.9 | 15.1 |
| S0_10_G20_10 | 15.0 | 14.9 | 15.0 | 14.6 | 14.6 | 15.0 | 14.9 | 20.6 |
| S10_10_G20_10 | 15.0 | 14.9 | 15.0 | 14.6 | 14.6 | 15.0 | 14.9 | 15.0 |
| S0_10_G30_10 | 14.6 | 14.6 | 14.6 | 14.6 | 14.5 | 14.5 | 14.9 | 20.6 |
| S10_10_G40_10 | 14.6 | 14.6 | 14.6 | 14.6 | 14.5 | 14.5 | 14.9 | 15.1 |
| S0_10_G40_10 | 14.7 | 14.6 | 14.6 | 14.6 | 14.9 | 14.4 | 15.0 | 20.1 |
| S0_10_G50_10 | 14.6 | 15.0 | 14.6 | 14.6 | 14.6 | 14.5 | 14.9 | 19.6 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 13.8 | 14.6 | 14.6 | 14.5 | 13.5 | 14.5 | 15.1 | 21.5 |
| S10_10_G10_10 | 10.6 | 10.0 | 10.9 | 13.1 | 13.5 | 14.5 | 14.6 | 13.7 |
| S20_10_G10_10 | 10.6 | 10.0 | 10.9 | 13.1 | 13.5 | 14.5 | 14.6 | 14.6 |
| S0_10_G20_10 | 14.3 | 14.7 | 14.6 | 14.6 | 14.0 | 14.5 | 14.6 | 21.6 |
| S10_10_G20_10 | 14.3 | 14.7 | 14.6 | 14.6 | 14.5 | 14.5 | 14.6 | 14.7 |
| S0_10_G30_10 | 14.7 | 14.6 | 14.1 | 14.5 | 14.5 | 14.6 | 15.1 | 20.1 |
| S10_10_G40_10 | 14.7 | 14.6 | 14.1 | 14.5 | 14.5 | 14.6 | 14.6 | 14.6 |
| S0_10_G40_10 | 14.6 | 14.5 | 14.5 | 14.4 | 14.5 | 14.6 | 16.0 | 21.0 |
| S0_10_G50_10 | 14.6 | 14.5 | 14.9 | 14.9 | 14.6 | 14.2 | 14.6 | 22.0 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 13.6 | 13.6 | 14.0 | 14.1 | 13.7 | 15.1 | 18.7 | 22.0 |
| S10_10_G10_10 | 9.5 | 9.4 | 12.2 | 13.6 | 13.7 | 13.7 | 13.7 | 14.1 |
| S20_10_G10_10 | 9.5 | 9.4 | 12.2 | 13.6 | 13.7 | 13.7 | 13.7 | 14.1 |
| S0_10_G20_10 | 13.7 | 13.6 | 14.0 | 14.0 | 13.2 | 15.1 | 18.3 | 20.2 |
| S10_10_G20_10 | 13.7 | 13.6 | 14.0 | 14.0 | 13.6 | 13.7 | 14.2 | 13.8 |
| S0_10_G30_10 | 14.1 | 13.6 | 14.1 | 14.1 | 13.7 | 15.6 | 18.3 | 20.1 |
| S10_10_G40_10 | 14.1 | 13.6 | 14.1 | 14.1 | 13.7 | 13.7 | 14.1 | 13.7 |
| S0_10_G40_10 | 14.1 | 13.6 | 14.1 | 13.7 | 13.7 | 15.5 | 17.7 | 20.1 |
| S0_10_G50_10 | 13.7 | 14.2 | 13.7 | 13.7 | 13.7 | 15.5 | 17.7 | 20.0 |

Table 6.2.3.2.1-11: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 11.6 | 11.5 | 11.5 | 13.0 | 11.9 | 11.5 | 11.6 | 17.2 |
| S10_10_G10_10 | 6.0 | 5.9 | 8.8 | 13.0 | 11.9 | 11.5 | 11.6 | 12.2 |
| S20_10_G10_10 | 6.0 | 5.9 | 8.8 | 13.0 | 11.9 | 11.5 | 11.6 | 12.2 |
| S0_10_G20_10 | 11.6 | 11.5 | 12.0 | 11.6 | 11.9 | 11.5 | 11.6 | 18.1 |
| S10_10_G20_10 | 11.6 | 11.5 | 12.0 | 11.6 | 11.9 | 11.5 | 11.6 | 12.2 |
| S0_10_G30_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.9 | 12.0 | 11.6 | 14.4 |
| S10_10_G40_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.9 | 12.0 | 11.6 | 11.7 |
| S0_10_G40_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.4 | 12.0 | 11.6 | 15.8 |
| S0_10_G50_10 | 12.1 | 11.5 | 11.5 | 11.6 | 11.9 | 11.5 | 11.6 | 16.7 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.6 | 11.2 | 9.7 | 10.5 | 11.6 | 11.5 | 13.3 | 18.6 |
| S10_10_G10_10 | 6.0 | 6.0 | 5.9 | 10.5 | 11.6 | 11.5 | 10.6 | 11.7 |
| S20_10_G10_10 | 6.0 | 6.0 | 5.9 | 10.5 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G20_10 | 11.6 | 11.6 | 11.1 | 10.5 | 11.6 | 11.5 | 13.8 | 19.0 |
| S10_10_G20_10 | 11.6 | 11.6 | 11.1 | 10.5 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G30_10 | 11.2 | 11.2 | 11.5 | 11.4 | 11.6 | 11.5 | 12.0 | 17.2 |
| S10_10_G40_10 | 11.2 | 11.2 | 11.5 | 11.4 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G40_10 | 11.6 | 11.6 | 11.1 | 11.4 | 11.6 | 11.5 | 14.2 | 20.0 |
| S0_10_G50_10 | 11.6 | 11.6 | 11.5 | 11.4 | 11.2 | 11.5 | 12.9 | 18.6 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 8.0 | 8.9 | 9.8 | 10.7 | 11.0 | 14.3 | 19.0 | 21.9 |
| S10_10_G10_10 | 6.0 | 6.0 | 6.0 | 10.2 | 9.2 | 9.3 | 9.8 | 9.9 |
| S20_10_G10_10 | 6.0 | 6.0 | 6.0 | 10.2 | 9.2 | 9.3 | 9.8 | 9.9 |
| S0_10_G20_10 | 9.4 | 9.8 | 9.3 | 9.7 | 9.6 | 12.9 | 17.1 | 19.5 |
| S10_10_G20_10 | 9.4 | 9.8 | 9.3 | 9.7 | 9.6 | 10.2 | 9.8 | 9.9 |
| S0_10_G30_10 | 9.8 | 9.3 | 9.8 | 9.7 | 10.1 | 13.4 | 17.6 | 20.0 |
| S10_10_G40_10 | 9.8 | 9.3 | 9.8 | 9.7 | 9.6 | 9.7 | 9.8 | 9.9 |
| S0_10_G40_10 | 9.8 | 9.8 | 9.8 | 10.2 | 10.1 | 13.4 | 17.6 | 20.0 |
| S0_10_G50_10 | 9.8 | 9.8 | 9.8 | 9.7 | 10.1 | 13.8 | 18.0 | 20.5 |

Table 6.2.3.2.1-12 shows the maximum value of simulation results for SL non-contiguous CA.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 6.2.3.2.1-12 : PC2 PSFCH AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A(SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | 14.9 | 14.3 | 13.4 | 19.9 | 21.3 | 21.3 |
|  | 2x23dBm + 1LO | 15.1 | 14.6 | 14.0 | 20.6 | 22.0 | 22.0 |
|  | 2x23dBm + 2LO | 13.0 | 11.6 | 9.8 | 18.1 | 20.0 | 21.9 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | 14.9 | 12.5 | 11.5 | 14.4 | 14.3 | 13.5 |
|  | 2x23dBm + 1LO | 15.1 | 13.1 | 12.2 | 15.1 | 14.7 | 14.2 |
|  | 2x23dBm + 2LO | 13.0 | 10.5 | 6.0 | 12.2 | 11.7 | 10.2 |

The AMPR can be proposed as Table 6.2.3.2.1-13 based on the simulation results when considering implementation margin.

Table 6.2.3.2.1-13 : PC2 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 | ≤ 24.0 | ≤ 25.0 | ≤ 25.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 | ≤ 24.0 | ≤ 25.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 16.0 | ≤ 15.5 | ≤ 21.5 | ≤ 23.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 20.0 | ≤ 17.5 | ≤ 15.5 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 17.5 | ≤ 15.5 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 15.0 | ≤ 9.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |  |  |

From Table 6.2.3.2.1-12 and Table 6.2.3.2.1-13, the following is observed.

Observation: For PC2 PSFCH, A-MPR of higher SCS is larger than that of smaller SCS for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, PSFCH A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

PC3 PSFCH A-MPR can be proposed as follows.

For PC2 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 6.2.3.2.1-14.

For PC2 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 6.2.3.2.1-14.

Table 6.2.3.2.1-14 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 16.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

6.2.3.3	A-MPR for S-SSB transmission

6.2.3.3.1	Simulation results from LG Electronics (R4-2503595)

For S-SSB A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 6.2.3.3.1-2 are considered.

Table 6.2.3.3.1-2: SL non-contiguous CA AMPR evaluation scenarios (S-SSB)

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| Conf#2, Conf#3 in Table 2.1-1 | 1 | 11RB41 | 11RB0 | SEMfreq_-13A | 15 |
|  | 2 | 11RB31 | 11RB10 | SEMfreq_-13A | 15 |
|  | 3 | 11RB20 | 11RB21 | SEfreq_-30A | 15 |
|  | 4 | 11RB0 | 11RB41 | SEfreq_-30A | 15 |
|  | 5 | 11RB26 | 11RB0 | SEfreq_-30A | 30 |
|  | 6 | 11RB21 | 11RB1 | SEfreq_-30A | 30 |
|  | 7 | 11RB13 | 11RB7 | SEfreq_-30A | 30 |
|  | 8 | 11RB0 | 11RB13 | SEfreq_-30A | 30 |
| Conf#1, Conf#4 ~ Conf#9 in Table 2.1-1 | 1 | 11RB41 | 11RB0 | SEfreq_-30A | 15 |
|  | 2 | 11RB31 | 11RB10 | SEfreq_-30A | 15 |
|  | 3 | 11RB20 | 11RB21 | SEfreq_-30A | 15 |
|  | 4 | 11RB0 | 11RB41 | SEfreq_-30A | 15 |
|  | 5 | 11RB26 | 11RB0 | SEfreq_-30A | 30 |
|  | 6 | 11RB21 | 11RB1 | SEfreq_-30A | 30 |
|  | 7 | 11RB13 | 11RB7 | SEfreq_-30A | 30 |
|  | 8 | 11RB0 | 11RB13 | SEfreq_-30A | 30 |

- PC3 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-13 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.3.1-3, Table 6.2.3.3.1-4, and Table 6.2.3.3.1-5 show the AMPR simulation results for the SL non-contiguous CA.

Table 6.2.3.3.1-3: SSSB AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 17.4 | 17.0 | 17.8 | 17.9 | 15.6 | 15.9 | 15.0 | 16.0 |
| S10_10_G10_10 | 8.5 | 14.3 | 17.8 | 17.5 | 11.4 | 14.1 | 15.1 | 15.1 |
| S20_10_G10_10 | 8.6 | 13.3 | 17.4 | 16.9 | 11.7 | 13.6 | 15.1 | 14.2 |
| S0_10_G20_10 | 17.8 | 17.0 | 17.9 | 17.8 | 15.6 | 15.4 | 15.5 | 15.6 |
| S10_10_G20_10 | 17.8 | 17.4 | 17.9 | 17.0 | 15.5 | 14.7 | 15.4 | 15.5 |
| S0_10_G30_10 | 17.8 | 17.9 | 17.8 | 17.0 | 15.9 | 15.9 | 15.5 | 15.6 |
| S10_10_G40_10 | 17.8 | 16.6 | 16.9 | 17.0 | 15.8 | 15.0 | 15.5 | 15.5 |
| S0_10_G40_10 | 17.1 | 17.4 | 18.6 | 17.5 | 15.0 | 15.9 | 15.0 | 15.6 |
| S0_10_G50_10 | 17.8 | 17.5 | 17.8 | 17.5 | 15.8 | 15.0 | 16.3 | 16.0 |

Table 6.2.3.3.1-4: SSSB AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 17.3 | 18.7 | 17.4 | 18.2 | 15.4 | 16.3 | 15.8 | 16.3 |
| S10_10_G10_10 | 8.4 | 15.0 | 18.1 | 18.2 | 11.7 | 13.5 | 16.7 | 15.9 |
| S20_10_G10_10 | 8.4 | 14.1 | 17.7 | 17.8 | 12.6 | 13.9 | 15.9 | 15.9 |
| S0_10_G20_10 | 19.0 | 17.8 | 17.8 | 18.2 | 15.4 | 15.3 | 15.8 | 15.4 |
| S10_10_G20_10 | 18.5 | 17.8 | 18.7 | 18.2 | 15.4 | 15.9 | 15.8 | 15.9 |
| S0_10_G30_10 | 18.6 | 17.8 | 18.6 | 18.2 | 16.6 | 14.9 | 14.9 | 15.5 |
| S10_10_G40_10 | 18.7 | 17.8 | 18.5 | 17.8 | 15.8 | 15.8 | 15.8 | 15.4 |
| S0_10_G40_10 | 17.4 | 18.2 | 17.7 | 18.6 | 15.0 | 15.4 | 15.5 | 17.3 |
| S0_10_G50_10 | 18.1 | 18.7 | 18.1 | 17.0 | 16.3 | 15.3 | 15.8 | 16.4 |

Table 6.2.3.3.1-5: SSSB AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 7.7 | 8.2 | 8.1 | 13.8 | 7.3 | 7.2 | 7.7 | 13.8 |
| S10_10_G10_10 | 3.6 | 5.4 | 8.6 | 8.2 | 4.4 | 5.4 | 5.8 | 5.9 |
| S20_10_G10_10 | 3.5 | 5.4 | 8.2 | 8.2 | 4.4 | 5.4 | 5.9 | 6.4 |
| S0_10_G20_10 | 8.2 | 8.3 | 8.2 | 15.2 | 7.2 | 7.2 | 8.6 | 15.2 |
| S10_10_G20_10 | 8.2 | 8.3 | 8.6 | 8.2 | 7.2 | 7.3 | 7.2 | 7.3 |
| S0_10_G30_10 | 9.1 | 8.2 | 8.6 | 12.4 | 7.3 | 7.2 | 7.2 | 13.3 |
| S10_10_G40_10 | 7.7 | 8.3 | 8.6 | 8.2 | 7.2 | 7.2 | 7.2 | 7.3 |
| S0_10_G40_10 | 8.2 | 8.2 | 8.1 | 13.8 | 7.2 | 7.2 | 9.2 | 15.6 |
| S0_10_G50_10 | 8.7 | 8.2 | 9.0 | 13.4 | 7.3 | 7.3 | 7.8 | 13.8 |

Table 6.2.3.3.1-6 shows the maximum value of simulation results for SL non-contiguous CA.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 6.2.3.3.1-6 : PC3 SSB AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | 17.4 | 15.9 | 18.6 | 16.3 |
|  | 2x20dBm + 1LO | 18.7 | 16.3 | 19.0 | 17.3 |
|  | 2x20dBm + 2LO | 8.2 | 7.3 | 15.2 | 15.6 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | 14.3 | 14.1 | 17.9 | 15.8 |
|  | 2x20dBm + 1LO | 15.0 | 13.9 | 18.7 | 16.7 |
|  | 2x20dBm + 2LO | 5.4 | 5.4 | 8.6 | 7.3 |

The AMPR can be proposed as Table 6.2.3.3.1-7 based on the simulation results when considering implementation margin.

Table 6.2.3.3.1-7 : PC3 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 20.0 | ≤ 18.5 | ≤ 21.0 | ≤ 21.0 |
|  | 2x20dBm + 1LO | ≤ 21.0 | ≤ 19.5 | ≤ 21.5 | ≤ 21.5 |
|  | 2x20dBm + 2LO | ≤ 14.5 | ≤ 14.0 | ≤ 17.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 17.0 | ≤ 16.5 | ≤ 20.5 | ≤ 18.5 |
|  | 2x20dBm + 1LO | ≤ 17.5 | ≤ 16.5 | ≤ 21.0 | ≤ 19.5 |
|  | 2x20dBm + 2LO | ≤ 12.0 | ≤ 12.0 | ≤ 15.0 | ≤ 14.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |

From Table 6.2.3.3.1-6 and Table 6.2.3.3.1-7, the following is observed.

Observation : For PC3 S-SSB, A-MPR of SCS 30kHz is larger than or equal to that of SCS 15kHz for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, S-SSB A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

PC3 S-SSB A-MPR can be proposed as follows.

For PC3 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 6.2.3.3.1-8.

For PC3 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 6.2.3.3.1-8.

Table 6.2.3.3.1-8 : PC3 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 20.0 | ≤ 21.0 |
|  | 2x20dBm + 1LO | ≤ 21.0 | ≤ 21.5 |
|  | 2x20dBm + 2LO | ≤ 14.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 17.0 | ≤ 20.5 |
|  | 2x20dBm + 1LO | ≤ 17.5 | ≤ 21.0 |
|  | 2x20dBm + 2LO | ≤ 12.0 | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

- PC2 A-MPR

< UE RF architecture >

UE RF architecture in Table 6.2.2.1.1-1 is considered for A-MPR evaluation.

< Simulation results >

Table 6.2.3.3.1-9, Table 6.2.3.3.1-10, and Table 6.2.3.3.1-11 show the AMPR simulation results for the SL non-contiguous CA.

Table 6.2.3.3.1-9: SSSB AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 20.8 | 20.4 | 21.2 | 21.3 | 18.0 | 18.0 | 17.1 | 18.6 |
| S10_10_G10_10 | 15.6 | 17.2 | 20.9 | 20.9 | 13.9 | 16.2 | 17.5 | 17.2 |
| S20_10_G10_10 | 13.8 | 16.3 | 21.3 | 20.9 | 13.8 | 15.7 | 17.5 | 18.1 |
| S0_10_G20_10 | 21.2 | 20.9 | 20.9 | 20.4 | 17.6 | 18.5 | 18.3 | 18.9 |
| S10_10_G20_10 | 20.7 | 20.8 | 20.4 | 21.7 | 18.9 | 17.5 | 17.0 | 17.5 |
| S0_10_G30_10 | 21.7 | 20.0 | 20.8 | 20.4 | 18.4 | 17.5 | 17.5 | 18.9 |
| S10_10_G40_10 | 20.9 | 20.4 | 20.3 | 20.9 | 18.0 | 18.9 | 17.5 | 18.1 |
| S0_10_G40_10 | 20.5 | 20.4 | 20.8 | 21.2 | 18.3 | 18.7 | 18.5 | 19.4 |
| S0_10_G50_10 | 22.1 | 20.4 | 20.9 | 21.3 | 17.4 | 17.4 | 18.5 | 17.7 |

Table 6.2.3.3.1-10: SSSB AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 21.1 | 20.7 | 21.1 | 21.6 | 17.9 | 18.3 | 18.2 | 19.3 |
| S10_10_G10_10 | 17.3 | 17.0 | 21.1 | 21.6 | 14.2 | 17.4 | 18.3 | 17.5 |
| S20_10_G10_10 | 14.6 | 17.1 | 21.2 | 22.1 | 14.2 | 16.0 | 18.7 | 18.4 |
| S0_10_G20_10 | 21.9 | 22.0 | 21.6 | 20.7 | 18.3 | 17.9 | 18.3 | 22.5 |
| S10_10_G20_10 | 22.3 | 21.6 | 21.7 | 22.0 | 17.5 | 18.3 | 19.5 | 17.9 |
| S0_10_G30_10 | 21.1 | 22.0 | 22.0 | 21.6 | 18.7 | 19.1 | 19.1 | 18.4 |
| S10_10_G40_10 | 21.6 | 21.7 | 21.5 | 20.7 | 18.3 | 18.2 | 18.7 | 18.8 |
| S0_10_G40_10 | 21.7 | 22.1 | 21.9 | 21.2 | 18.3 | 17.8 | 17.4 | 19.3 |
| S0_10_G50_10 | 21.5 | 20.3 | 21.6 | 21.2 | 18.3 | 18.7 | 19.3 | 20.7 |

Table 6.2.3.3.1-11: SSSB AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 11.6 | 12.1 | 12.0 | 16.3 | 7.9 | 8.3 | 10.2 | 16.3 |
| S10_10_G10_10 | 3.8 | 7.4 | 12.0 | 12.1 | 6.4 | 7.4 | 7.9 | 7.5 |
| S20_10_G10_10 | 3.8 | 7.4 | 12.5 | 12.1 | 6.0 | 6.9 | 8.3 | 7.9 |
| S0_10_G20_10 | 12.1 | 12.1 | 12.0 | 16.7 | 8.3 | 7.8 | 10.7 | 18.1 |
| S10_10_G20_10 | 12.1 | 12.1 | 12.5 | 12.1 | 7.9 | 7.8 | 7.9 | 8.3 |
| S0_10_G30_10 | 12.5 | 12.1 | 12.1 | 14.8 | 7.9 | 7.8 | 8.8 | 16.2 |
| S10_10_G40_10 | 12.2 | 12.1 | 12.0 | 12.6 | 7.8 | 7.9 | 8.8 | 7.8 |
| S0_10_G40_10 | 12.1 | 12.1 | 12.5 | 16.2 | 8.3 | 8.3 | 11.1 | 17.6 |
| S0_10_G50_10 | 12.1 | 12.1 | 12.0 | 16.7 | 7.8 | 8.4 | 10.7 | 16.7 |

Table 6.2.3.3.1-12 shows the maximum value of simulation results for SL non-contiguous CA.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 6.2.3.3.1-12 : PC2 SSB AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | 20.8 | 18.0 | 22.1 | 19.4 |
|  | 2x23dBm + 1LO | 21.1 | 18.3 | 22.1 | 22.5 |
|  | 2x23dBm + 2LO | 12.1 | 8.3 | 16.7 | 18.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | 17.2 | 16.2 | 21.7 | 18.9 |
|  | 2x23dBm + 1LO | 17.3 | 17.4 | 22.3 | 19.5 |
|  | 2x23dBm + 2LO | 7.4 | 7.4 | 12.6 | 8.8 |

The AMPR can be proposed as Table 6.2.3.3.1-13 based on the simulation results when considering implementation margin.

Table 6.2.3.3.1-13 : PC2 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 23.5 | ≤ 20.5 | ≤ 24.5 | ≤ 24.5 |
|  | 2x23dBm + 1LO | ≤ 23.5 | ≤ 21.5 | ≤ 24.5 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 15.5 | ≤ 15.0 | ≤ 19.0 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 19.5 | ≤ 18.5 | ≤ 24.0 | ≤ 21.5 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 | ≤ 25.0 | ≤ 22.5 |
|  | 2x23dBm + 2LO | ≤ 14.0 | ≤ 14.0 | ≤ 16.0 | ≤ 15.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |

From Table 6.2.3.3.1-12 and Table 6.2.3.3.1-13, the following is observed.

Observation : For PC2 S-SSB, A-MPR of SCS 30kHz is larger than or equal to that of SCS 15kHz for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, S-SSB A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

PC3 S-SSB A-MPR can be proposed as follows.

For PC2 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 6.2.3.3.1-14.

For PC2 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 6.2.3.3.1-14.

Table 6.2.3.3.1-14 : PC2 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 23.5 | ≤ 24.5 |
|  | 2x23dBm + 1LO | ≤ 23.5 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 15.5 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 19.5 | ≤ 24.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 14.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

<end of text proposal>