3GPP TSG-RAN WG4 Meeting #114-bis				                    					  R4-2503633 Wuhan, China 7th - 11th April 2025

Agenda item:			7.30.4

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [114bis][223] IoT_NTN_Ph3

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) phase 3, including the following topics covered

- Topic#1: RRM core requirements (AI 7.30.3)

Recommended issues for online discussion:

- Issue 1-1, 1-2, 1-4

# Topic #1: RRM core requirements (AI 7.30.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503132 | MediaTek inc. | Proposal 1: Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN, and the detailed behavior is pending further progress in R2. • Requirements for CB-Msg3 transmission • Correct behavior when receiving CB-Msg4 • Correct behavior when not receiving CB-Msg4 Proposal 2: CR work split based on Table 1. Table 1. CR work split for CB-Msg3 Core requirement; Volunteer; Comment. 6.6A Random Access for UE category NB-IoT for Satellite Access; New subclause for CB-Msg3. 6.2.4A Random Access Requirements for Cat-M1 UEs with CB-Msg3 for Satellite Access; New subclause for CB-Msg3 It appears that a separate new subclause is necessary.. 7.20A UE transmit timing for NB-IoT for Satellite Access; Revised to apply to CB-Msg3. 7.24A UE transmit timing for Category M1 for Satellite Access. |
| R4-2503388 | CMCC | Proposal 1: At least following UE procedural requirement need to be included, others are not precluded and further depends on RAN2’s input. - Correct behavior when transmitting CB-msg3. - UE shall transmit the CB-msg3 and select the CE level based on [CE level related RSRP threshold] if the RSRP is equal or higher than [RSRP threshold] . - UE shall fallback to Random access and transmit PRACH if the RSRP is lower than [RSRP threshold]. - Correct behavior when receiving CB-msg4. - UE stops the PDCCH monitoring window(s) and the remaining replicas transmission (if configured) once it receives a CB-msg4 containing a matching Contention Resolution Identity. Proposal 2: RAN4 to discuss whether to define measurement requirement for the RSRP used by UE to compare with RSRP thresholds. |
| R4-2503871 | Huawei, HiSilicon | Proposal 1: RAN4 to wait until next meeting with more progress on RAN2 MAC CR. Proposal 2: The procedural requirements for RA should be extracted and aligned with RAN2 MAC spec, and the number of the requirements should be limited. |
| R4-2504110 | Ericsson | Observation 1: RAN4 may be aware of UE’s possible behaviours, which still under RAN2’s discussions, regarding two scenarios: - If the UE receives Msg4 with contention resolution within monitoring window (s), - UE may send ACK if the contention resolution is successful. - If monitoring window(s) expires, - UE may attempt another CB-msg3 transmission (may be indicated by CB-Msg4) after waiting for a random back-off with an increased Tx power. - UE may fallback to 4-step random access EDT after a number of failed CB-msg3 transmission attempts. Proposal 1: RAN4 can update the procedural requirements for CB-MSG3 transmission after RAN2 achieves agreements. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1: CB-Msg3 EDT enhancement

Issue 1-1-1: Procedural requirements for CB-MSG3 transmission

Agreement:

- Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

Further discuss the content to include in the next meeting

Background: Agreement in R4#114

#### Issue 1-1: Subclauses in Random Access requirement for CB-Msg3

Proposals:

- Proposal 1 (MediaTek): Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN, and the detailed behavior is pending further progress in R2.

  - Requirements for CB-Msg3 transmission

  - Correct behavior when receiving CB-Msg4

  - Correct behavior when not receiving CB-Msg4

- Proposal 2 (CMCC):

  - Correct behavior when transmitting CB-msg3.

  - Correct behavior when receiving CB-msg4.

Recommended WF:

- Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN

  - For CB-Msg3 transmission, discuss the following options

    - Option 1: Requirements for CB-Msg3 transmission

    - Option 2: Correct behavior when transmitting CB-Msg3.

  - Correct behavior when receiving CB-Msg4

  - Correct behavior when not receiving CB-Msg4

- The exact titles of the subclauses can be further updated to align with RAN2 specifications.

#### Issue 1-2: CB-Msg3 transmission

Proposals:

- Proposal 1 (MediaTek, Huawei, Ericsson):

  - the detailed behavior is pending further progress in R2

  - Proposal 1a (MediaTek): The example TP based on PUR (removing TA validation) is provided in R4-2503132

- Proposal 2 (CMCC):

  - UE shall transmit the CB-msg3 and select the CE level based on [CE level related RSRP threshold] if the RSRP is equal or higher than [RSRP threshold] .

  - UE shall fallback to Random access and transmit PRACH if the RSRP is lower than [RSRP threshold].

Recommended WF:

- RAN4 to wait until next meeting for further progress in RAN2.

#### Issue 1-3: Correct behavior when receiving CB-Msg4

Proposals:

- Proposal 1 (MediaTek, Huawei, Ericsson):

  - the detailed behavior is pending further progress in R2

- Proposal 2 (CMCC):

  - UE stops the PDCCH monitoring window(s) and the remaining replicas transmission (if configured) once it receives a CB-msg4 containing a matching Contention Resolution Identity.

Recommended WF:

- RAN4 to wait until next meeting for further progress in RAN2.

#### Issue 1-4: CR work split

Proposals:

- Proposal 1 (MediaTek): CR work split based on Table 1.

- Table 1. CR work split for CB-Msg3

| Core requirement | Volunteer | Comment |
| --- | --- | --- |
| 6.6A Random Access for UE category NB-IoT for Satellite Access |  | New subclause for CB-Msg3 |
| 6.2.4A Random Access Requirements for Cat-M1 UEs with CB-Msg3 for Satellite Access |  | New subclause for CB-Msg3 It appears that a separate new subclause is necessary. |
| 7.20A UE transmit timing for NB-IoT for Satellite Access |  | Revised to apply to CB-Msg3 |
| 7.24A UE transmit timing for Category M1 for Satellite Access |  |  |

Recommended WF:

- Collect comments on the CR work split and assign volunteers during this meeting.