# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503646

# Wuhan, China, 7th – 11st April, 2025

Title: 	Discussion on performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Source: 	OPPO

Agenda item:	7.17.4

Document for:	Approval

Introduction

According to the existing RAN4 discussion, the core requirements for FR2-1 L3 measurement delay was almost completed. Based on the TU budget in [1], it is time to discuss the performance requirements form this meeting.

| leading WG | R4RD | R4RD | R4RF | R4RD |
| --- | --- | --- | --- | --- |
| RAN4 | 114bis | 115 | 116 | 116 |
| Core | 1 | 1 |  | 1 |
| Perf | 0.5 | 0.5 |  | 0.5 |

In this contribution, we will provide our views on performance requirements for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor and CSSF.

Discussion

Optimizing Rx beam sweeping factor

For Multi-Rx based L3 measurement, the following scenarios are defined in core requirements and the corresponding test cases need to be considered.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

For SSB based intra-frequency and inter-frequency measurement, there are many different cases (DRX/non-DRX, per-UE/per-FR gap, with/without SSB index reading etc.) defined in legacy test cases, but we do not need to test all the cases. For Multi-Rx based L3 measurement, we can choose one set of configurations for each of scenario. It is also fine to cover different configuration in different scenarios.

For handover and RRC re-establishment, intra-frequency and inter-frequency test cases can be defined. But to reduce the number of test case, it is also fine to choose one of intra- or inter-frequency for each scenario.

Proposal 1: For BSF optimization, RAN4 to define test cases for the following scenarios in FR2:

  - SSB based Intra-frequency measurement without MG

  - SSB based Inter-frequency measurement with MG

  - Intra-frequency Handover from FR2 to FR2

  - Inter-frequency Handover from FR2 to FR2

  - Intra-frequency RRC Re-establishment

  - Inter-frequency RRC Re-establishment

  - RRC Connection Release with Redirection

Regarding the test configuration, for scenario 5/7, since faster beam sweeping always applies during handover and RRC re-establishment, the existing configurations in A.7.3.1 and A.7.3.2 can be used as baseline.

For scenario 1/2/3/4, since the trigger condition of FBS based measurement is defined, besides taking the existing test configurations as baseline, the transmission power of serving cell from the start of T2 shall be set to meet the trigger condition of FBS based measurement. So the test consists of two successive time periods, with time duration of T1 and T2 respectively, and there are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2). During time duration T1, the UE shall not have any timing information of Cell 2. At the beginning of T2, Cell 2 starts transmission and the transmission power of Cell 1 is set to meet the trigger condition of FBS based measurement. UE perform FBS based measurement during T2.

Proposal 2: For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be used as baseline.

Proposal 3: For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. And the transmission power of serving cell from the start of T2 is set to meet the trigger condition of FBS based measurement.

Optimizing CSSF

For CSSF enhancement, the following two approaches are defined:

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

And the CSSF enhancement are considered for the following aspects and applied for the CA/DC mode including SA, EN-DC, NE-DC and NR-DC.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

But we think we do not need to define test case for all the CA/DC mode for each of intra-frequency, inter-frequency and inter-RAT scenarios to reduce the number of test cases.

Meanwhile, it was agreed to extend the technical solutions for optimizing CSSF to FR1 if applicable, and we think the test case for both FR1 and FR2 can be defined.

Proposal 4: For CSSF optimization, RAN4 to define test cases for the following scenarios in FR2:

  - For solution 1 (measure one serving CC per band):

    - Intra-frequency measurement without MG for FR1 only CA

    - Inter-frequency measurement without MG for FR2 only CA

    - Inter-RAT measurement without MG for FR1+FR2 CA (FR1 PCell)

  - For solution 3 (3 searchers for CSSF):

    - Intra-frequency measurement without MG for FR1 only CA

    - Inter-frequency measurement without MG for FR1+FR2 CA (FR1 PCell)

Conclusions

In this paper, we provided our views on the performance requirements for FR2-1 SSB based L3 measurement delay reduction for optimizing BSF and CSSF. And the following proposals are provided.

Proposal 1: For BSF optimization, RAN4 to define test cases for the following scenarios in FR2:

  - SSB based Intra-frequency measurement without MG

  - SSB based Inter-frequency measurement with MG

  - Intra-frequency Handover from FR2 to FR2

  - Inter-frequency Handover from FR2 to FR2

  - Intra-frequency RRC Re-establishment

  - Inter-frequency RRC Re-establishment

  - RRC Connection Release with Redirection

Proposal 2: For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be used as baseline.

Proposal 3: For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. And the transmission power of serving cell from the start of T2 is set to meet the trigger condition of FBS based measurement.

Proposal 4: For CSSF optimization, RAN4 to define test cases for the following scenarios in FR2:

  - For solution 1 (measure one serving CC per band):

    - Intra-frequency measurement without MG for FR1 only CA

    - Inter-frequency measurement without MG for FR2 only CA

    - Inter-RAT measurement without MG for FR1+FR2 CA (FR1 PCell)

  - For solution 3 (3 searchers for CSSF):

    - Intra-frequency measurement without MG for FR1 only CA

    - Inter-frequency measurement without MG for FR1+FR2 CA (FR1 PCell)

Reference

- RP-240830 New WID: NR Radio Resource Management (RRM) Phase 5