3GPP TSG-RAN WG4 Meeting #114bis	R4-2503648

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Discussion on UE RF requirements for FR2 LP-WUR

Source:	Samsung

Agenda item:	7.26.2.2

Document for:	Discussion

1.	Introduction

Previous RAN4 discussion was mainly focused on FR1, and the progress on FR2 LP-WUR is relatively slow. So far, we had a good starting on FR2 UE assumption for REFSENS evaluation. In this contribution, we share our views on the open issues for FR2 LP-WUR

2. 	Discussion

2.1 REFSENS

In RAN4#113 meeting, it was agreed that OFDM-based FR2 LPWUR architecture comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized [1]

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR Agreements: - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details - Multiple frequency conversion in RF domain is not precluded - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features: - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized - FFS on whether and how to combine two Rx chains - Antenna gain is that of a single element - Coverage target should be considered

And in RAN4#114 meeting it was further confirmed that 1Rx at based band for FR2 LP-WUR.

Issue 2-2-1: Combining strategy for FR2 LP-WUR Agreements: - For Rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation. - Confirm 1Rx at baseband for FR2 LP-WUR.

Then conclusion can be drawn that there will be no MRC combining for FR2 LP-WUR. Thus the best combining strategy of FR2 LP-WUR would be selection-based diversity for the two Rx chains with orthogonally polarized antennas.

Proposal 1:	the combining strategy of FR2 LP-WUR should be selection-based diversity for the two Rx chains with orthogonally polarized antennas

Regarding detailed impacts to REFSENS evaluation, the direct impact is the diversity gain in the REFSENS derivation equation.

For FR2 MR, 0dB diversity gain is assumed which was also reflected in the EIS metric which is an average of the two EIS components, as shown in Annex K 1.2 of TS 38.521-2

…… 9) Calculate the resulting “averaged EIS” as: averaged EIS = 2*[1/EIS(PolMeas= PolLink= +1/EIS(PolMeas= PolLink=]-1 The RX beam peak direction is where the minimum “averaged EIS” is found.

However, for LP-WUR, the metric for REFSENS is changed from EIS to MDR. If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric, because the legacy EIS metric only requires the averaged performance between different link polarization testing but the new MDR metric requires the worst case between the two polarizations testing has to pass.

Observation 1:	If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric.

Corresponding to the more stringent MDR metric than averaged EIS, the diversity gain for REFSENS derivation has to be assumed as -3dB so that the worst link polarization testing can pass the requirement.

Proposal 2:	-3dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation.

For spherical coverage of FR2 LP-WUR, the usual practice is to apply a gain drop from the peak performance. Note that the elements number does not affect the gain drop obviously but the panel number matters. For FR2 LP-WUR, single panel is assumed which is similar as FR2 RedCap in terms of panel number. So it is reasonable to reuse the same gain drop value from peak to 50%-tile as FR2 RedCap’s.

Proposal 3:	it is proposed to reuse the same gain drop value from peak to 50%-tile as FR2 RedCap’s, i.e., 10.9dB.

2.2 blocking and spurious response

In last meeting, whether to specify spurious response requirements is still an open issue [2]

Issue 3-2-4: spurious response requirements for FR2 Way Forward - Further check next meeting on whether need to specify spurious response requirements for FR2 LP-WUR.

It is common understanding that out-of-band blocking and spurious response are two correlated requirements. Given there is no out-of-band blocking for MR, so no spurious response requirements are specified accordingly. The same principle can be used for LR.

Proposal 4:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

3. 	Conclusion

Proposal 1:	the combining strategy of FR2 LP-WUR should be selection-based diversity for the two Rx chains with orthogonally polarized antennas

Observation 1:	If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric.

Proposal 2:	-3dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation.

Proposal 3:	it is proposed to reuse the same gain drop value from peak to 50%-tile as FR2 RedCap’s, i.e., 10.9dB.

Proposal 4:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

4. 	References

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo

- R4-2503028    “WF on UE RF requirements for LP-WUS”, vivo

- TR 38.869