3GPP TSG-RAN WG4 Meeting #114bis	R4-2503649

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	7.26.2.4

Document for:	Discussion

1.	Introduction

Testability is one of the scopes for this work item but no much progress yet, it is important to converge on high level framework and then move forward onto details.

In this contribution, we share our views on LP-WUR testability.

2. 	Discussion

There kinds of RRC states are involved for LP-WUS feature, i.e., RRC_Idle, RRC_inactive and RRC_Connected. For requirement and test, a high level agreement was achieved in RAN4#113 meeting [1]:

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing Agreements: - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Only one RRC state needs to be tested. Then it is next question which RRC state is to be selected or prioritized for testing. Progress on this aspect faces obstacle due to no clear consensus on UE capabilities, the way forward of last meeting is to further check whether all UE types can support IDLE and/or CONNECTED mode [2]:

Issue 4-1-2: Separate RF test case for idle and connection mode Way forward: - RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?

According to the UE feature list [3] attached in the RAN1 LS [4], UE has full flexibility in terms of its supported RRC states. UE can support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) separately, and UE can also support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) together, i.e., there could be 3 types of UE in terms of supported RRC states:

- UEs supporting IDLE/INACTIVE mode only

- UEs supporting CONNECTED mode only

- UEs supporting both IDLE/INACTIVE mode and CONNECTED mode

Observation  1:	According to RAN1 feature list, there could be UEs supporting IDLE/INACTIVE mode only, or CONNECTED mode only, or supporting both IDLE/INACTIVE mode and CONNECTED mode.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  |

For FR1, supporting LP-WUS in RRC_Idle states showed most obvious power saving benefits [5]. Though UE may support not all 3 RRC states, the large commercialization possibility is that the FR1 LP-WUS UE would either support RRC_Idle or support RRC_Idle & RRC_Connected states. Therefore it can be expected that test method based on RRC_Idle could cover most available UEs. It is reasonable to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 1:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

High level framework has been discussed for several meeting cycles, and the latest agreements are as following [1]:

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance Agreements: - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point: - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance. - FFS if the MR should only be woken up when data transfer the system simulator is needed. - Other options are not precluded.

There have been sufficient discussions on this issue and it can be concluded that waking up MR for each detected LP-WUS is not practical for conformance test. It is more practical to just counting the detected LP-WUS signal and not to wake up the MR before the test counting is over. It is proposed to agree on this high level framework and then move forward.

Proposal 2:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

3. 	Conclusion

Observation  1:	According to RAN1 feature list, there could be UEs supporting IDLE/INACTIVE mode only, or CONNECTED mode only, or supporting both IDLE/INACTIVE mode and CONNECTED mode.

Proposal 1:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 2:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

4. 	References

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo

- R4-2503028    “WF on UE RF requirements for LP-WUS”, vivo

- R1-2501397    “Initial RAN1 UE features list for Rel-19 NR after RAN1 #120”, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399    “LS on Rel-19 RAN1 UE features list for NR after RAN1#120”, RAN1

- TR 38.869