3GPP TSG-RAN WG4 Meeting #114bis	R4-2503659

Wuhan, Hubei, China, 07th – 11nd April, 2025

Agenda item:	7.16.2

Source:	Samsung

Title:	Simulation results on spatial channel model

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In RAN4#113 and RAN4#114 meetings, parameter assumptions, CDL methodologies and AAV configurations has been agreed for simulations [3][4].

In this contribution, we share our simulation results for CDL-based model, TDL extended model and baseline TDL channel model.

- Simulation assumptions

  - Agreed simualtion assumptions

For CDL based channel modeling, we combine the simulation assumptions argeed in RAN4#113 meeting [3] and RAN4#114 meeting [4] as below.

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | 40MHz |  |  |  |
| Subcarrier spacing |  | 30kHz |  |  |  |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 8 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed | Follow, Random |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=6 (L=4, pν=1/2, 1/4, β=1/2) ] |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) 4 CSI-RS Ports (2,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (5) |  |  |  |
| CQI/RI/PMI delay |  | N/A | 7 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

  - Other parameters and configurations

Channel Model Configurations

For the channel model configurations, we use option A1, TDL extended model option B1 and baseline TDLC300 as below.

- Option A1:  CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827

- Option B1: TDL extended model

- Baseline: TDLC300 defined in TS38.101-4

AAV Configurations

For AAV configurations of CDL based channel modeling, below options are used for initial simulation

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Doppler frequencies

- 10Hz

- 100Hz

Correlation matrix for baseline TDLC cases

- ULA Low

- X-Pol Medium

- X-Pol High

- ULA Medium

Extended TDL parameters

- Two clusters for rank4 with power [0, -3] dB, angles [0, π/2]

- Four clusters for rank8 with power [0, 0, -3, -3] dB, angles [0, π/6, π/2, 3π/2]

- Simulation results

  - PDSCH demodulation performance for 4Tx4Rx4layer Doppler 10Hz configuration

Legacy TDL and 38.827 based CDL comparison

Firstly, 38.827 based CDL is the starting point, hence it is important to compare the simulation results between legacy TDL channel models and 38.827 based CDL channel model in Figure 3-1. For legacy TDL, three correlation matrixes are deployed including Low, XP Medium, XP High. For 38.827 based CDL, AAV model option 3 and option 1Y are simulated.

Figure 3-1 PDSCH demodulation performance comparison between legacy TDL and 38.827 based CDL

From Figure 3-1, we have below observations:

- Peak throughput is hard to reach for legacy TDLC300-10 with XP High correlation matrix.

- 38.827 based CDL option 1Y has higher performance than option 3 because of the beamforming gain.

Hence, we need analysis the 38.827 based CDL results with SNR normalization factor as Figure 3-2, in which the SNR normalization factor obtained from simulation results as Table 3-1.

Table 3-1 SNR normalization factor for 4Tx4Rx 38.827 based CDL

| Number of antennas | Models | Normalization factor (linear) | Normalization factor (dB) |
| --- | --- | --- | --- |
| 4Tx4Rx | CDL827-option3 | 0.829431407 | -0.812195238 |
|  | CDL827-option1Y | 4.385069685 | 6.419764993 |
| 8Tx8Rx | CDL827-option3 | 0.890083597 | -0.505692023 |
|  | CDL827-option1Y | 4.384104304 | 6.418808781 |

Figure 3-2 PDSCH demodulation performance comparison with SNR normalization factors

From Figure 3-2, we have below observations:

- 38.827 based CDL option 3 and option 1Y have almost the same PDSCH demodulation performance when SNR normalization factors are considered.

- 38.827 based CDL options have similar PDSCH demodulation performance as legacy TDL300-10 XP Medium configuration.

38.827 based CDL and 38.901 based CDL comparison

According our discussion t-docs, the only difference between 38.827 based CDL and 38.901 based CDL is the AoA/AoD/ZoA/ZoD angles. the PDSCH demodulation performance curves are close as Figure 3-3 shows, and the SNR working points at 70% @ maximum TP are almost the same.

Note: results for 38.901 based CDL option 1Y need to be update later.

Figure 3-3 PDSCH demodulation performance comparison between 38.827 based CDL and 38.901 based CDL

Legacy TDL and extended TDL comparison

The simulation results for PDSCH demodulation performance comparison between legacy TDL and extended TDL models as Figure 3-4. From Figure 3-4, we have below observations:

- Extended TDL with XP high correlation could reach peak throughput.

- Extended TDL has better performance than legacy TDL cases.

- Extended TDL with XP high correlation has better performance than low correlation.

Figure 3-4 PDSCH demodulation performance comparison between legacy TDL and extended TDL

  - PDSCH demodulation performance for 8Tx8Rx8layer Doppler 10Hz configuration

Legacy TDL and 38.827 based CDL comparison

For 8Tx8Rx8Layer in Figure 3-5, we compare the PDSCH demodulation performance between legacy TDLC300-10 with low/XP Medium correlation matrixes and 38.827 based CDL option 3/option 1Y. from this comparison, the observations could be summarized as below

- For legacy TDL, the PDSCH demodulation performance of both codewords are almost the same.

- For 38.827 based CDL, the PDSCH demodulation performance of both codewords have some difference, especially for option 1Y.

- TDLC300-10 with XP Medium correlation matrix could not reach peak throughput.

Figure 3-5 PDSCH demodulation performance comparison between legacy TDL and 38.827 based CDL

  - PMI reporting performance for 8Tx4Rx2layer Doppler 10Hz configuration

Legacy TDL and 38.827 based CDL comparison

For 8Tx4Rx2Layer results in Figure 3-6, the PMI reporting performance are compared between legacy TDL and 38.827 based CDL models, for single panel Type I codebook and eType II codebook. The Gamma values are summarized as Table 3-2.

Figure 3-6 PMI reporting performance comparison between legacy TDL and 38.827 based CDL

Table 3-2 Gamma values for PMI reporting performance

| Number of antennas | Models | SNR @ 90% maximum TP (dB) | Gamma |
| --- | --- | --- | --- |
| 8Tx4Rx2Layer | TDLC TypeI | 4.6 | 2.73 |
|  | TDLC eTypeII | 8.1 | 1.97 |
|  | CDL827 option3 TypeI | 8.3 | 1.31 |
|  | CDL827 option3 eTypeII | 4.7 | 1.86 |
|  | CDL827 option1Y TypeI | 0.8 | 1.34 |
|  | CDL827 option1Y eTypeII | -2.5 | 1.88 |

From Figure 3-6 and Table 3-2, the observations could be summarized as below

- For legacy TDL300-10 with XP High correlation, the performance of eType II codebook is even worse than Type I codebook.

- For 38.827 based CDL, the performance of eType II codebook is between than Type I codebook.

- The Gamma values of 38.827 based CDL are less than legacy TDL as the performance of random PMI case has better performance.

PMI reporting performance of extended TDL

The PMI reporting performance of extended TDL model with XP High correlation is as Figure 3-7, and the Gamma values as Table 3-3. From Figure 3-7 and Table 3-3, the observations could be summarized as below

- For extended TDL300-10 with XP High correlation, the performance of eType II codebook is better than Type I codebook.

- The Gamma values of extended TDL300-10 model are larger than 38.827 based CDL since the performance of random PMI case has worse performance.

Figure 3-7 PMI reporting performance of extended TDL model

Table 3-3 Gamma values for PMI reporting performance of extended TDL model

| Number of antennas | Models | SNR @ 90% maximum TP (dB) | Gamma |
| --- | --- | --- | --- |
| 8Tx4Rx2Layer | Extended TDL TypeI | 4.2 | 1.87 |
|  | Extended TDL eTypeII | 2.4 | 2.33 |

  - PMI reporting performance for 8Tx4Rx4layer Doppler 10Hz configuration

For 8Tx4Rx4Layer results in Figure 3-8, the PMI reporting performance are compared between legacy TDL and 38.827 based CDL models, for single panel Type I codebook and eType II codebook. The Gamma values are summarized as Table 3-4.

Figure 3-8 PMI reporting performance comparison between legacy TDL and 38.827 based CDL

Table 3-4 Gamma values for PMI reporting performance

| Number of antennas | Models | SNR @ 90% maximum TP (dB) | Gamma |
| --- | --- | --- | --- |
| 8Tx4Rx4Layer | TDLC TypeI | NA | NA |
|  | TDLC eTypeII | NA | NA |
|  | CDL827 option3 TypeI | 20.1 | 1.07 |
|  | CDL827 option3 eTypeII | 17.6 | 1.30 |
|  | CDL827 option1Y TypeI | 12.5 | 1.08 |
|  | CDL827 option1Y eTypeII | 10.1 | 1.32 |

From Figure 3-8 and Table 3-4, the observations could be summarized as below

- For legacy TDL300-10 with XP High correlation, the performance of eType II codebook is even worse than Type I codebook.

- For 38.827 based CDL, the performance of eType II codebook is between than Type I codebook.

- For 38.827 based CDL, the performance of Type I codebook is worse than random PMI case at lower SNR range.

- The Gamma values of 38.827 based CDL are small as the performance of random PMI case has better performance than legacy TDL cases.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[3] R4-2419782, Way Forward for [113][320]NR_SCM, RAN4#113 meeting, Nokia

[4] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia