# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503697

# Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the NTN S band definition

Agenda Item:	6.13.1

Document for:	Approval

Introduction

In the RAN4#114 meeting, RAN4 reached some consensus in the way forward [1], but the out-of-band blocking requirements are still open. In this paper, we’d like to share our views on these issues in detail.

- Issue 1-2-4: UE Out-of-band Blocking - Agreement: - Further discuss the Out-of-Band Blocking requirements for band n252 based on the options following - Option 1, with no co-banding assumption with n256 and n65 Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -30; -15. n252; Finterferer (CW); MHz; -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750. n2542; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. n255; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. n2561; Finterferer (CW); MHz; -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void. - Option 2 with assumption of n252 co-banding with n256 and n65 Table 1: Out of-band blocking for n252 Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -30; -15. n2521; Finterferer (CW); MHz; -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750. n2561; Finterferer (CW); MHz; -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations. - Option3: reuse the OOB requirements of band B23

Out-of-band blocking requirement

In the contribution [2], companies proposed that one duplexer may cover all the bands n252, n256 and NR band n1/n65. The proposal is good enough to reuse the same component to cover more bands as many as possible. Thus, the out-of-band blocking requirements for band n252 can be similar to the band n256. The FDL_high is same between band n256 and n252, but there are 10MHz difference for the FDL_low. The Out-of-band blocking requirements for band n256 are defined below.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Thus, it’s necessary to reflect the difference of FDL_low in the spec, and there is no need to change the frequency range of FDL_high part.

Proposal 1: To consider Option 2 with assumption of n252 co-banding with n256 and n65.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Summary

Proposal 1: To consider Option 2 with assumption of n252 co-banding with n256 and n65.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

- References

[1] R4-2502850, WF on NR and IoT NTN bands, Moderator (Aalyria)

[2] R4-2419519, Discussion on NR-NTN n252 UE RF requirements, MediaTek Inc.