3GPP TSG-RAN WG4 Meeting #114bis	R4-2503706

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN

Agenda item:	7.11.6

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM requirements for Ku-band NTN based on approved WF [1] and revised WID [2].

Discussion

RRM scope for NR NTN on Ku bands

Agreement: - For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session. - Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE: - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

In the last RAN plenary, revised WID [2] was approved. 15kHz SCS for SSB and data channels was added for Ku-band,  and the requirements are only applicable for GSO and low mobility scenarios.

- Proposal 1: Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band,

  - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology with 15kHz and 30kHz

  - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology with 30kHz

  - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology with 120kHz

  - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology with 120kHz

Impact on RRM requirements from NR NTN on Ku bands

For RRM requirements of Ku-band NTN for both FR1-NTN and FR2-NTN numerologies, same assumptions for Rel-18 Ka-band RRM requirements should apply below:

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

- Proposal 2: The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

The UE type for Ku-band is mobile VSAT defined in Ka-band NTN. Based on the assumption mentioned above, the RRM requirements defined for Ka-band could be resued for Ku-band RRM requirements, except for timing requirements.

- Proposal 3: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN RRM requirements (not to introduce separate RRM requirements for Ku-band NTN), except for UL transmit timing requirements.

- Proposal 4: Add following applicability for Ku-band in Clause 3.6.12

- The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies.

UL transmit timing requirement

Agreement: - RAN4 to define timing accuracy requirements for case 1, 2: - For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - RAN4 to define timing accuracy requirements for case 3: - For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered. - RAN4 to define timing accuracy requirements for case 4: - For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

In RF session, frequency range designation was updated by including Ku-band frequency both FR1-NTN and FR2-NTN

Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

For FR1-NTN numerology, as mentioned above, since 15kHz SCS was added for GSO, timing error requiremients for 15kHz SCS should be considered for Ku-band. However, how to capture the timing error requirements for Ku-band should be discussed; introduce new Table, capture Note on applicability for Ku-band, or nothing to add (since RF specification will be describe it also).

- Proposal 5: Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band. 
          FFS how to capture it in RAN4 specification (also FR2-NTN numerology cases)

- Alt 1: introduce separate timing error requirements Table for Ku-band, e.g.,

Table 1. Te_NTN Timing Error Limit for Ku-band

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: 15kHz SCS is applicable to only GSO |  |  |  |  |

- Alt 2: add Note on applicability for Ku-band in existing Table, e.g.,

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: For Ku-band, 15kHz SCS of SSB and uplink signals are applicable to only GSO, and 30kHz SCS of SSB and uplink signals are applicable to GSO or NGSO |  |  |  |  |

- Alt 3: Nothing to add since RF specification would be describe Ku-band applicability

In the last meeting, for mobile VSAT served by GSO in FR2-NTN, following conditions were agreed to introduce, and RAN4 agreed to reuse the same timing error and the conditions for Ku-band case 3.

- Te_NTN for 120 kHz UL SCS applies under all the following conditions:

- LOS channel between UE and each GNSS satellite

- The GNSS signal power level greater than or equal to the applicable value specified in Table 6.2-1 of TS 38.171

- The GNSS satellite allocation is no less than the applicable value speicifed in Table 6.2-2 of TS 38.171

- The velocity change within one second does not result in the position difference with respect to the projected position, based on the position and velocity obtained one second before, exceeding 10 meters.

For NGSO as case 4, positioning error to derive transmit timing error would be larger for NGSO for GSO case. If the same transmit timing error as 7.5Ts is considered, additional condition might be needed to reduce the position error for NGSO as considered in case 3. To reduce position error for NGSO, the accuracy of ephemeris information for satellite should be improved, or the update periodicity of ephemeris information should be shortened or limited to a short periodicity. However, there is no accuracy of ephemeris information in RAN4, so shorter or limited ephemeris information update periodicity could be considered.

- Proposal 6: For NGSO as case 4, to reuse 7.5Ts timing requirements, shorter or limited ephemeris information update periodicity could be considered as an additional condition.

Conclusion

In this contribution, we provide our views on RRM requirements for Ku-band NTN, and we propose

- Proposal 1: Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band,

  - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology with 15kHz and 30kHz

  - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology with 30kHz

  - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology with 120kHz

  - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology with 120kHz

- Proposal 2: The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

- Single SAN Tx beam per radio cell in DL

- For inter satellite HO, only blind HO and no neighbor cell measurement

- Intra satellite neighbor cell measurement is needed

- Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

- Proposal 3: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN RRM requirements (not to introduce separate RRM requirements for Ku-band NTN), except for UL transmit timing requirements.

- Proposal 4: Add following applicability for Ku-band in Clause 3.6.12

- The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies.

- Proposal 5: Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band. 
          FFS how to capture it in RAN4 specification (also FR2-NTN numerology cases)

- Alt 1: introduce separate timing error requirements Table for Ku-band, e.g.,

Table 1. Te_NTN Timing Error Limit for Ku-band

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: 15kHz SCS is applicable to only GSO |  |  |  |  |

- Alt 2: add Note on applicability for Ku-band in existing Table, e.g.,

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: For Ku-band, 15kHz SCS of SSB and uplink signals are applicable to only GSO, and 30kHz SCS of SSB and uplink signals are applicable to GSO or NGSO |  |  |  |  |

- Alt 3: Nothing to add since RF specification would be describe Ku-band applicability

- Proposal 6: For NGSO as case 4, to reuse 7.5Ts timing requirements, shorter or limited ephemeris information update periodicity could be considered as an additional condition.

Reference

[1] R4-2500529, “WF on RRM requirements for NR_NTN_Ku_bands,” Apple

[2] RP-250799, “Revised WID: Introduction of Ku bands for NR NTN,” Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT