3GPP TSG-RAN WG4 Meeting #114bis	R4-2503707

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on some open issues on L3 measurement delay reduction with multi-Rx simultaneous reception based on [1].

Discussion

Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

RAN4 agreed not to introduce additional condition to activate FBS-based L3 measurement, and there some open discussion on related agreed triggering condition (cell quality based triggering). First issue is whether to limit the serving cell to FR2 serving cell for indication of triggering condition. Since RAN4 is focusing on FR2 single carrier and it is not clear use case for FR1 serving cell to trigger condition for FR2 FBS, in Rel-19, only FR2 serving cell should be considered as serving cell to indicate triggering condition.

- Proposal 1: Do not consider FR1 serving cell to activate FR2 FBS-based L3 measurement in Rel-19

Second issue is to add the hysteresis and TTT paramters. These parameters should be discussed and RAN4 can inform to RAN2 to consider these parameters to design the triggering condition signaling.

- Proposal 2: RAN4 informs to RAN2 to design the triggering condition signaling taking into account the hysteresis and timeToTrigger parameters.

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue.

For conditions to trigger UE to quit the FR2 FBS, there would be two kinds of cases; first, if UE cannot do perform FR2 FBS due to power saving or overheating issues; second, if UE completes the measurement based on FR2 FBS. For the first case, Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused, and for the second case, UE can deactivate FR2 FBS operation when UE reports valid measurement result after activating FR2 FBS for L3 measurement based on serving cell’s signal quality for scenario 1/2/3/4.

Therefore, RAN4 can consider two conditions to trigger UE to quit the FR2 FBS below;

- Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

- UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

In the last meeting, re-activation condition was discussed. Re-activation of FR2 FBS should be occur when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met. It is better to leave it to UE implementation whether to keep or re-active FR2 FBS, and RAN4 needs to define only conditions for activation/deactivation for FR2 FBS.

- Proposal 3: Two condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

  - UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

- Proposal 4: RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

According to the proposals from companies in the past few meetings, most proposals are to introduce the UE capability of redueced N value(s) for BSF. For scenario 5 and 7, it would be available to use reduced N value(s) for BSF as RAN4 has done in other WIs before. However, for scenario 1 to 4, existing M values as below have been defined by considering Rx BSF and the number of samples, and the number of sampls depends on Rx BSF; for example, 4 measurement samples with 6 Rx BSF or 3 measurement samples with 8 Rx BSF. It means that depending on UE implementation to perform L3 measurement, different Rx BSF was considered assuming single antenna panel.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

- Observation 1: Different Rx BSF has been considered for exsiting FR2 L3 measurement by UE implementation assuming single antenna panel.

If only reduced Rx BSF by N value(s) for L3 measurement is considered in Rel-19, it could impose limitation on the UE implementation for L3 measurement considered in previous release. Therefore, when UE activates FR2 FBS for L3 measurement, it is important to reduce the total M values. Introducing a scaling value < 1 is a right way that takes into account the exsiting UE impelemenataion rather than introducing reduced Rx BSF N value(s).

- Proposal 5: Introduce a scaling vlue < 1 considering exsiting UE implementation (Observation 1) at least for L3 measurements (scenario 1~4).

For delay requirements for scenario 1~4, based on the proposal 5, L3 measurement delay requirement using multi-Rx reception can be defined by adding Pmulti-Rx (< 1) with UE capability, e.g., below table.

Table 1 Example of proposed measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Pmulti-Rx x Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

- Proposal 6: L3 measurement delay requirement (scenario 1~4) using multi-Rx reception can be defined by adding Pmulti-Rx (< 1) with UE capability

Conclusion

In this contribution, we provide views on open issues for multi-Rx simultaneous reception, and we propose

- Proposal 1: Do not consider FR1 serving cell to activate FR2 FBS-based L3 measurement in Rel-19

- Proposal 2: RAN4 informs to RAN2 to design the triggering condition signaling taking into account the hysteresis and timeToTrigger parameters.

- Proposal 3: Two condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’

  - UE deactivates FR2 FBS based L3 measurements when UE has reported qualified measurement results

- Proposal 4: RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when seving cell’s signal quality is lower then the threshold configured by network and after quit conditions being discussed have been met.

- Observation 1: Different Rx BSF has been considered for exsiting FR2 L3 measurement by UE implementation assuming single antenna panel.

- Proposal 5: Introduce a scaling vlue < 1 considering exsiting UE implementation (Observation 1) at least for L3 measurements (scenario 1~4).

- Proposal 6: L3 measurement delay requirement (scenario 1~4) using multi-Rx reception can be defined by adding Pmulti-Rx (< 1) with UE capability

Reference

[1] R4-2502593, “WF on RRM requirements for NR_RRM_Ph5_Part2,” CATT.