# 3GPP TSG-RAN WG4 Meeting # 114bis		  R4-2503748

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on UE RF requirements for supporting new SDO band for LTE-based 5G Broadcast

# Agenda Item:	7.9.3

# Document for:	Approval

- Introduction

During RAN#107 meeting, a new Work Item [1] has been approved on specifying a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO. In this contribution, related UE RF core requirements for 10MHz PMCH are discussed. The objective of this WI is shown as below:

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO:

  - Specify related UE RF core requirements for 10MHz PMCH

    - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as  baseline (reuse assumption of 10 MHz UE Rx filter)

  - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements:

    - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline

    - Requirements based on Resolution 761 in WRC-19

    - Only consider QPSK and 16QAM modulation

Note: UE only needs to support 5G broadcast for this band

Note: Release independence applicability will be further discussed

- Discussion

For a SDO band UE RF, only receiver characteristics need to be considered. For REFSENS, we propose to use the following equation to derive.

REFSENS= -174+NF+10*log10(BW)+60-diversity gain+IM+SNR

BW(MHz) = 10*0.9

diversity gain = 3dB for 2Rx.

IM=2.5dB.

SNR= -1dB minimum SNR for detection

9dB NF can be assumed.

Therefore, REFSENS is -97 dBm for 10MHz.

The RF requirements are analyzed based on the system parameters in another contribution[4]. The spec impact on TS36.102 and our views on the UE RF requirements are listed in Table 2-1.

Table 2-1 UE RF requirements for the new SDO band (TS 36.102)

| Rx requirement | Proposals for new SDO band |
| --- | --- |
| 7.3 Reference sensitivity power level | Spec impact: Add a new chapter to capture Reference sensitivity power level for LTE-based 5G Broadcast, and REFSENS can be defined as -97 dBm for 10MHz. |
| 7.4 Maximum input level | Spec impact: Add a new chapter to capture Maximum input level for LTE-based 5G Broadcast. Maximum input level is channel bandwidth agnostic, so -40 dBm can be considered as specified in TS36.102. |
| 7.5 Adjacent Channel Selectivity (ACS) | Spec impact: Add a new chapter to capture Adjacent Channel Selectivity for LTE-based 5G Broadcast. |
| 7.6.2 In-band blocking | Spec impact: Add a new chapter to capture In-band blocking requirements for LTE-based 5G Broadcast. |
| 7.6.3 Out-of-band blocking | Spec impact: Add a new chapter to capture Out-of-band blocking requirements for LTE-based 5G Broadcast. |
| 7.6.4 Narrow band blocking | No spec impact. Narrow-band blocking requirements are not applicable to LTE based 5G terrestrial broadcast. |
| 7.7 Spurious response | Spec impact: Add a new chapter to capture Spurious response for LTE-based 5G Broadcast. |
| 7.8 Intermodulation characteristics | Spec impact: Add a new chapter to capture Intermodulation characteristics for LTE-based 5G Broadcast. |
| 7.9 Spurious emissions | No spec impact. |
| A.4 UL reference measurement channels | Spec impact: To introduce Fixed Reference Channel for 10MHz PMCH. |

Proposal1: To use the proposals in Table 1 for UE RF requirements for the new SDO band for LTE-based 5G Broadcast operating.

- Conclusion

In this contribution, some initial views on UE RF requirements are provided and the proposals ares made as following:

Proposal1: To use the proposals in Table 2-1 for UE RF requirements for the new SDO band for LTE-based 5G Broadcast operating.

- Reference

“RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.”

3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access"

R4-2503747 Discussion on band definition and system parameters for supporting new SDO band for LTE-based 5G Broadcast, ZTE, RAN4#114bis, Wuhan, China, April 2025.