# 3GPP TSG-RAN WG4 Meeting #114bis                               R4-2503761

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.23.3.2

Source: 	CMCC

Title: 			 Discussion on RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides discussion on RRM impacts for SBFD operation.

- Discussion

Issue 2-1-1a: CSI-RS L1 measurement – collision with UL - Proposals - Proposal 1: - For the collision handling between L1 periodic/semi-persistent CSI-RS resource and semi-statically configured/dynamic UL transmission, wait for RAN1 reply. This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements - Proposal 2 : - Dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

For CSI-RS colliding with UL transmission, it was agreed in RAN4 #113 meeting that for aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. For periodic and semi-persistent CSI-RS resource, RAN4 sent LS to RAN1 for clarification [6].

In last meeting, RAN1 discussed this this and the reply LS is agreed [7], and the details are duplicated as following.

RAN1 reply LS on CSI-RS measurement with SBFD operation (R1-2501560) Issue#1: Collision Case 2 that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements For collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, RAN1 would like to provide the following response: The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”. Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

According to RAN1 reply LS [7], RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. And the exsiting prioritization rules for serving-cell is to prioritize UL transmission and cancel CSI-RS reception.

Based on RAN1 feedback, for periodic and semi-persistent CSI-RS resource, dynamically scheduled UL transmission is prioritized over semi-statically configured CSI-RS. And UE will execute the dynamically scheduled UL transmission.

Proposal 1: for periodic and semi-persistent CSI-RS resource colliding with UL transmission, it is proposed to follow RAN1 reply LS that UL transmission is prioritized over semi-statically configured CSI-RS for serving cell.

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

For CSI-RS puncturing in frequency domain in SBFD symbols, there are two cases: DU configuration, DUD configuration. For both cases, if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold, no accuracy requirements apply. In detail, CSI-RS based RLM/BFD/CBD, the existing threshold to apply requirements is 24 RBS, and for CSI-RS based L1-RSRP/L1-SINR, the existing threshold to apply requirements is 48 RBS.

For DU configuration, it was agreed that existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs is no less than the existing threshold. How to handle DUD configuration is FFS. In detail, DUD configuration includes following cases:

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

This issue is about UE measurement behaviour. There is discussion about whether UE measures both DL subbands on each CSI-RS resource occasion or UE measures one DL subband on each CSI-RS resource occasion. In our understanding, it is possible that measures both DL subbands on each CSI-RS resource occasion In our understanding, combining two subbands is preferred. Firstly, this is for wideband reporting, it is reasonable to perform measurement and report by combining two DL subbands. The other consideration is that SBFD symbols originally are DL symbols. If SBFD is not configured, UE is capable to receive the whole DL PRBs. With SBFD, just the UL subband is punctured,but UE still can receive all DL usable PRBs. With the assumption that UE measures both DL subbands on each CSI-RS resource occasion, no need to extend measurement period, and legacy requirements can be applied.

Proposal 2: for CSI-RS based L1 measurement in SBFD symbols with DUD case, it is proposed to reuse existing measurement period for CSI-RS based L1 measurement, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

Considering that UE can measure all DL usable PRBs, no matter case 1 or case 2 or case 3, there is no difference, and existing accuracy requirements for CSI-RS based measurements apply as long as the all DL usable PRBs are larger than the minimum BW with which the existing accuracy requirements apply.

Proposal 3: for CSI-RS based L1 measurement in SBFD symbols with DUD case, including case 1, 2, 3, it is proposed to reuse existing accuracy requirements for CSI-RS based measurements if the number of usable CSI-RS PRBs is no less than the minimum BW with which the existing accuracy requirements apply.

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Proposals - Proposal 1: - The same principle, that only CSI-RS transmission occasions within one valid symbol type are used for CSI derivation, shall apply to at least CSI-RS based L1-RSRP and L1-SINR as these are one type of CSI report. - RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period depending on the valid symbol type (SBFD or non-SBFD symbols). - Proposal 2: - For one CSI-RS occasion, as long as the CSI-RS bandwidth is not below 48 PRBs, occasion on SBFD and non-SBFD symbols could be combined. This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements - Proposal 3 : - Wait for RAN1 reply to conclude the impact on CSI-RS measurement on SBFD and non-SBFD symbols.

For CSI-RS distributed over SBFD and non-SBFD symbols, according to RAN1 reply LS [7], the configuration of Configuration 1 or 2 are not applied to CSI-RS resources configuration. And RAN1 agreed that the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. And it is RAN1 common understanding that the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report can be SBFD symbols only or non-SBFD symbols only, no combination of SBFD symbols and non-SBFD symbols. From RAN4 requirements point of view, UE measure SBFD symbols only or non-SBFD symbols only based on the CSI report configuration.

- Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only.

- Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

Proposal 4: For CSI-RS distributed over SBFD and non-SBFD symbols, it is proposed that only SBFD symbols are measured or only non-SBFD symbols are measured based on the CSI report configuration.

Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

For the requirements for CSI-RS based L3 measurement, in our view, there is no difference between DU case DUD case. The agreements for DU case can be reused for DUD case. In detail, for DUD case, with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols, legacy requirements are applied.

Proposal 5: for DUD case, UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

Issue 2-1-3: Requirements for scheduling restriction - Proposals related to UL/DL transition - Proposal 1 : - The existing RAN4 scheduling restriction requirements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions. - Proposal 2 : - RAN4 scheduling restriction requirement is not needed for DL/UL transition. - Proposals related to inter-frequency SSB measurement without gap - Proposal 1 (Nokia, MTK): - There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps. - The existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation. - Proposal 2 (HW): - RAN4 to discuss whether scheduling restriction applies when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots. - Proposal 3 (vivo): - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

There is no scheduling restriciton for legacy L1 measurement, since there is guard period for the DL-to-UL transition. However, guard period is absent during the DL to UL transition. The potential issue is whether scheduling restriction is needed. RAN1 is discussing similar issue, as highlighted in blue as following. To avoid duplicated discussion, the issue can be left to RAN1.

Agreements in RAN1#116 meeting For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Proposal 6: it is proposed that RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: for periodic and semi-persistent CSI-RS resource colliding with UL transmission, it is proposed to follow RAN1 reply LS that UL transmission is prioritized over semi-statically configured CSI-RS for serving cell.

Proposal 2: for CSI-RS based L1 measurement in SBFD symbols with DUD case, it is proposed to reuse existing measurement period for CSI-RS based L1 measurement, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

Proposal 3: for CSI-RS based L1 measurement in SBFD symbols with DUD case, including case 1, 2, 3, it is proposed to reuse existing accuracy requirements for CSI-RS based measurements if the number of usable CSI-RS PRBs is no less than the minimum BW with which the existing accuracy requirements apply.

Proposal 4: For CSI-RS distributed over SBFD and non-SBFD symbols, it is proposed that only SBFD symbols are measured or only non-SBFD symbols are measured based on the CSI report configuration.

Proposal 5: for DUD case, UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

Proposal 6: it is proposed that RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2420165, LS on CSI-RS measurement with SBFD operation

- R1-2501560, Reply LS on CSI-RS measurement with SBFD operation