# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503770

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.1

Source: 	CMCC

Title: 			 Discussion on general aspects

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

RAN2 agreements in RAN2#129 Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

It can be seen that how applicable functionalities is activated is different for different report type. If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework. For semi-persistent CSI reporting, the applicable functionality is activated by MAC CE/DCI, for aperiodic CSI reporting, the applicable functionality is activated by DCI.

In our understanding, above RAN2 agreements are mainly for the switch from non-AI to AI. Sine there is different UE behaviour, the delay for the switch from non-AI to AI can be different, which need to be discussed case by case. For periodic CSI reporting, since UE can autonomously activate the applicable functionalities, the starting point of the delay for the switch from non-AI to AI could be that UE report applicable functionalities via RRCReconfigurationComplete. For semi-persistent and aperiodic CSI reporting, the starting point of the delay for the switch from non-AI to AI could be the point receiving activation indication by MAC CE/DCI.

Proposal 1: the switch delay from non-AI to AI is different for different report type:

- for periodic CSI reporting, since UE can autonomously activate the applicable functionalities, the starting point of switch delay from non-AI to AI is that UE report applicable functionalities via RRCReconfigurationComplete

- for semi-persistent and aperiodic CSI reporting, the starting point of the switch delay from non-AI to AI is the point receiving activation indication by MAC CE/DCI

RAN1 #120 Agreement For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric TR38.843 For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

Inference performance is eventully performance, which is impacted not only by the AI/ML model performance but may also be impacted by other factors. While model monitoring targets to evaluate whether a model works well under a centain configuation/scenario. Inference performance is good does not mean monitoring performance is good. From this point of view, intermediate KPI, at least cosine similarity (SGCS), ranging within [0, 1], can be considered as test metrics for LCM. RAN1 also agreed that for CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, calculation of monitoring metric based on intermediate KPI is supported

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS or NMSE, as requirements/tests metrics for LCM.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in last meeting, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the network deployment, which is not preferred.

Proposal 3: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: the switch delay from non-AI to AI is different for different report type:

- for periodic CSI reporting, since UE can autonomously activate the applicable functionalities, the starting point of switch delay from non-AI to AI is that UE report applicable functionalities via RRCReconfigurationComplete

- for semi-persistent and aperiodic CSI reporting, the starting point of the switch delay from non-AI to AI is the point receiving activation indication by MAC CE/DCI

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS or NMSE, as requirements/tests metrics for LCM.

Proposal 3: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement performance.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface