# 3GPP TSG-RAN WG4 Meeting #114                                  R4-2503793

# Wuhan, China, 7th – 11st  April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for RedCap NTN enhancement

Agenda item:	7.29.4.1

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

RAN4 shall define the RRM requirements based on the latest revised WID as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Discussion

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

  - 2.1 General consideration on RRM requirements

In above WID description, we know that RAN4 shall combine with R17/R18 RedCap WI and R17/R18 NR NTN WI to discuss this objective. Thus, we shall study the features introduced in RedCap which will impact the definition of requirements.

Issue 1-2-3-1: Feature 25-1 (parallelSMTC)

In the  last meeting, we spent several minutes on discussing feature 25-1 and the way forward could be seen as below:

<Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline mandatory for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

In legacy NTN study, NTN capable UE is mandatory to configured 2 SMTCs because of the time difference between gNB and UE among different NGSOs. However, some companies deem that one SMTC shall be only considered for Redcap UEs because of the bottleneck of the Redcap UE’s process, which is not related to 1Rx or 2Rx. However, the proponents of option 1 shall provide the strong reasons to explain which kind of processing capability will impact RedCap UE to use 2 SMTC for measurement. To my understanding, the only concern for option 1 is that the total measurement delay is severely extended. If option 1 is used, RedCap UE will complete all NTN neighbour cells via one SMTC, the case that RedCap UE shall spend more total time to complete all measurement. If the measurement delay is long enough and the moving speed of NGSOs is quick enough, the relative position between RedCap UEs and NGSOs will have a huge change. When the measurement report is sent to network, the report results may can not reflect the current radio condition, network would not feedback the downlink command timely. Thus, it may lead to delayed handover, resulting in brief service interruptions and noticeable decline in UE service quality. Consequently, option 1 seems not good for us. Furthermore, we could consider both the bottleneck of RedCap UE’s process (if proponents provide the concrete reasons) and total measurement delay, one compromised method could be optional support 2 parallel SMTCs, “parallel” means the SMTCs are TDM with proximity distance.

Observation 1: For option 1, the total measurement delay is severely extended.

Proposal 1: RAN4 could optional support 2 SMTCs in parallel, “parallel” means the STMC are TDM with proximity distance.

Issue 1-2-3-1a: The proximity distance between two SMTCs

In [2], RAN4 defines conditions on different SMTC collision for normal NR UEs as below:

When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

However, some companies deem that the conditions on SMTCs collision shall be changed for RedCap UEs compared to legacy NR normal UEs, and the wayforward could be seen as follows:

Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

There are two options for whether re-define the proximity distance or not. Some companies deem that the proximity distance shall be relaxed, and some companies think it is not reasonable to relax the proximity distance. From my perspective, the conditions for judging the collision between one more SMTCs refers to the conditions in concurrent gap. Since companies have concern that if more SMTCs have closed distance, the time for re-tuning and re-scheduling will be timeless. Thus, 4s shall also be one of conditions to judge the collision between one more SMTCs. However, the proponents shall provide concrete reasons for that RedCap UEs’ post-processing is worse than non-RedCap UEs, which will impact the time of re-tuning or re-scheduling. In (e)RedCap WI, there is no conclusion regarding RF re-tuning time and re-scheduling time for both RedCap UEs and non-RedCap UEs, which we can not say both two kinds of time must have significant differences easily. Consequently, we support option 1 firstly if there is no concrete reason to explain which part in RedCap UE will have impact on proximity distance relaxation.

Observation 2: In (e)RedCap WI, there is no conclusion regarding RF re-tuning time and re-scheduling time for both RedCap UEs and non-RedCap UEs, which we can not say both two kinds of time must have significant differences easily.

Proposal 2: RAN4 shall reuse the legacy collision conditions between two SMTCs.

Issue 1-2-3-3: The number of cells/SSBs/supported satellites

RAN4 shall clarify the number of cells, SSBs and supported satellites for RedCap UEs in this meeting. We could split two parts to consider: one for the number of supported satellites, the other one is the number of cells and SSBs.

- The number of supported satellites

- For feature 25-7, the concrete description could be as below:

maxNumber-NGSO-SatellitesPerCarrier-r17 - Indicates the number of target NGSO satellites the UE can monitor per carrier. For serving carrier, the number of target NGSO satellites also includes the serving satellite. If this field is not included, the number of target satellites UE can monitor per carrier is 2. The value shall be larger than or equal to the reported value on maxNumber-NGSO-SatellitesWithinOneSMTC-r17.

- From our perspective, UE is mandatorily to support measure 2 NGSOs per carrier which including satellite serving the PCell as described in [2]. The maximum value on feature25-5 is 4, and value on feature 25-7 could be 3 or 4 if UE supports feature 25-7. In [2], it says that cells from 3 or 4 satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17. Thus , we deem that the legacy number of supported satellites could be reused since there is no impact on types of UEs.

- Proposal 3: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

- The number of cells and SSBs

- From our perspective, considering the supported number of cells and SSBs, we deem that the number for normal NTN UE shall be reused for NTN RedCap UE. Since in TN network, the number of cells and SSBs are reused for RedCap UEs. Thus, it means this UE measurement capability would not consider the types of UE. Based on the clause 9 in [2], we can observed that the number of SSBs for NTN normal UEs is reduced to 8 for NGSO and 4 for GSO in legacy. Based on the above logic, legacy the number of cells and SSBs could be reused for RedCap UE in NTN scenario.

- Observation 3: In TN network, the number of cells and SSBs used for normal UEs are reused for RedCap UEs.

- Proposal 4: The number of cells and SSBs used for normal UE shall be reused for RedCap UEs in NTN scenario.

Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

- In previous meetings, we have the way forward on this issues as below:

<Way Forward>: - For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, - Proposal 1: Support to define. - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements. - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

- For feature 25-3, this feature is optional, so the requirements is up to UE implementation like the feature 25-5. We shall not preclude this feature for RedCap UEs.

- Proposal 5: The capability Feature 25-3(parallelMeasurementGap-r17) is valid for RedCap UE.

  - 2.2 Reduction in the number of UE Rx branches

- RAN4 has spent several meetings in discussing the reduction in the number of UE Rx branches. Some issues are still under discussion and we would like to provide our views on the following issues.

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

- In previous meetings, we discussed normal cell re-selection requirements for RedCap UEs. In [2], we find that the enhanced cell re-selection measurement requiremnets for 1Rx/2Rx RedCap UEs with FR1-NTN shall also be defined for RedCap UEs in NTN scenario. The following way forward could be seen as below:

<Way Forward>: - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN. - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused. - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE, - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples - Same delay requirements for 1Rx and 2Rx UEs. - Proposal 1c: - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements - For 1 Rx (e)RedCap NTN UE, RAN4 to - Option 1: Reuse the existing enhanced cell reselection measurement requirements - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined. - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

- From our understanding, when the NTN capable RedCap UEs supports measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16, the legacy enhanced cell re-selection requirements could be reused.

- Proposal 6: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements for RedCap UEs. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

- From our perspective, the cell re-selection measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs. However, RAN4 shall consider one point is the accuracy margin for 1Rx RedCap UE. Compared to 2Rx, 1Rx has lower transmission rate and the accuracy shift  is serious vary time, and RAN4 deems that the margin or threshold for 1Rx shall be higher than 2Rx and this can limit that two problems. Although the measurement requirements are same, the conditions for 1Rx and 2Rx shall be distinguished.

- Observation 4: Although the measurement requirements are same, the conditions for 1Rx and 2Rx shall be distinguished.

- Proposal 7: The cell re-selection inter-frequency and inter-RAT measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs considering TN to NTN and NTN to TN.

- Proposal 8: RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN.

Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- In the last meeting, the Way Forward of L1/L3 Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN is copied as follows [1]:

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

- The concrete L1/L3 measurement accuracy has been defined in clause 10.1A and 10.2A in [2] for RedCap UE in TN scenario. From our perspective, the legacy accuracy requirements shall be used for 2Rx RedCap UEs. For 1Rx RedCap UEs in TN network, 1dB margin on SS-RSRP/SS-RSRQ/SS-SINR accuracy requirements was added for L3 intra-frequency meausrement and inter-frequency measurement in FR1, and 3dB margin on L1-RSRP accuracy requirements was added for SSB based and CSI-RS based L1 measurement in FR1.

- For legacy NTN, we find that all of the values of accuracy requirements are the same as legacy.

- Consequently, for 2Rx RedCap UEs in NTN scenario, RAN4 shall reuse the legacy NTN accuracy requirements. For 1Rx RedCap UEs, RAN4 shall consider the same margin values as TN for 1Rx RedCap UEs.

- Proposal 9:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relaxed by 1dB.

- L1-RSRP: relaxed by 3dB.

  - 2.3 The impact of HD-FDD

- Compared to FD-FDD UE, HD-FDD operates by using separate frequency bands for uplink and downlink communication, but with a UE only being able to either transmit or receive at any given time – not both simultaneously. In RedCap WI, they discussed the several requirements for HD-FDD UE and define lots of applicable conditions not the new requirements. We can see the handover requirements as below:

Handover: Legacy RedCap HD-UE： For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

- We find that the legacy handover requirements can be met for HD-FDD UE, however, the measurement applicable conditions may be defined for HD-FDD operation. Thus, when we considering defining the requirements for RedCap HD-FDD UE in NTN scenario, the legacy requirements can be the baseline and the applicable condition can refer to the legacy conditions.

- Proposal 10: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE for HD-FDD in NTN scenario.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR RedCap NTN .

Observation 1: For option 1, the total measurement delay is severely extended.

Proposal 1: RAN4 could optional support 2 SMTCs in parallel, “parallel” means the STMC are TDM with proximity distance.

Observation 2: In (e)RedCap WI, there is no conclusion regarding RF re-tuning time and re-scheduling time for both RedCap UEs and non-RedCap UEs, which we can not say both two kinds of time must have significant differences easily.

Proposal 2: RAN4 shall reuse the legacy collision conditions between two SMTCs.

- Proposal 3: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

- Observation 3: In TN network, the number of cells and SSBs used for normal UEs are reused for RedCap UEs.

- Proposal 4: The number of cells and SSBs used for normal UE shall be reused for RedCap UEs in NTN scenario.

- Proposal 5: The capability Feature 25-3(parallelMeasurementGap-r17) is valid for RedCap UE.

- Proposal 6: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements for RedCap UEs. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

- Observation 4: Although the measurement requirements are same, the conditions for 1Rx and 2Rx shall be distinguished.

- Proposal 7: The cell re-selection inter-frequency and inter-RAT measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs considering TN to  NTN and NTN to TN.

- Proposal 8: RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN.

- Proposal 9:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relaxed by 1dB.

- L1-RSRP: relaxed by 3dB.

- Proposal 10: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE for HD-FDD in NTN scenario.

- References

[1] R4-2502604，WF on RRM requirements for NR_NTN_Ph3_Part1.

[2] TS38.133

[3] TS38.304

[4] TS38.331