# 3GPP TSG-RAN WG4 Meeting #114                                  R4-2503794

# Wuhan, China, 7th – 11st  April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Ku bands for NR NTN

Agenda item:	7.11.6

Document for:	Approval

Introduction

At RAN #107 meeting the revised WI “Introduction of Ku bands for NR NTN” [1] was approved.  In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #114 meeting we will specify the scenarios and general view on the topics. In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

Discussion

In the last meeting, RAN4 only discussed the general scope for NR NTN on Ku bands and the uplink transmit timing requirements. In this paper, we will provide the other potential RRM requirements for NR NTN on Ku bands. Furthermore, considering the BDaT session also had the agreements on SCS for downlink SSB signals and uplink data transmission, we will supplement the concrete requirements like uplink timing requirements etc.

  - 2.1 RRM scope for NR NTN on Ku bands

In the last meeting, we had the agreements as follows:

- For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session. - Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE: - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

The revised WID and BDaT session provide the applicable SCS for downlink SSB signals and uplink data transmission, the following description can be seen:

- Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE

Based on above table, we conclude that two points shall be noticed:

- For FR1-NTN supporting numerology, not only 30kHz but 15kHz shall be supported.

- For FR1-NTN and FR2-NTN supporting numerology, same SCS is assumed for SSB and data channels.

Thus, RAN4 shall define the RRM requirements including timing requirements shall follow above two points.

Observation 1: Based on description of revised WID and BDaT session, the following two points shall be noticed:

- For FR1-NTN supporting numerology, not only 30kHz but 15kHz shall be supported.

- For FR1-NTN and FR2-NTN supporting numerology, same SCS is assumed for SSB and data channels.

Proposal 1: For applicable SCS, RAN4 shall also consider SCS=15kHz in FR1-NTN numerology.

Proposal 2: RAN4 shall notice that the same SCS is assumed for SSB and data channels for FR1-NTN and FR2-NTN numerology respectively.

  - 2.2 Uplink transmit timing requirement

In the last meeting , RAN4 firstly confirmed four cases from RRM requirements perspective. Based on four cases, we defined the related uplink timing requirements, and there still have some issues shall be confirmed in this meeting:

Issue 1-3: UL transmit timing requirement Agreement: - RAN4 to define timing accuracy requirements for case 1, 2: - For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - RAN4 to define timing accuracy requirements for case 3: - For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered. - RAN4 to define timing accuracy requirements for case 4: - For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used. - For the applicable SCS, follow the agreement in BDaT session. - The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

RAN4 to define timing accuracy requirements for case 1, 2

We have agreed that the existing requirements in Table 7.1C.2-1 of TS 38.133 are reused for FR1-NTN numerology. However, for applicable SCS, only SCS=30kHz and 15kHz shall be considered and the SCS for downlink SSB signal and uplink data transmission is same. The concrete requirement could be seen as below:

Table 1: Te_NTN timing error limit for mobile VSAT is served by GSO and NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 3: The concrete timing requirements for case 1 and case 2 should be:

Table 1: Te_NTN timing error limit for mobile VSAT is served by GSO and NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

RAN4 to define timing accuracy requirements for case 3

We have agreed that the existing requirements in Table 7.1C.2-3 of TS 38.133 are reused for case3 with FR2-NTN numerology. The concrete timing requirements could be seen as below:

Table 2: Te_NTN timing error limit for mobile VSAT is served by GSO

| frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 4: The concrete timing requirements for case 3 should be:

Table 2: Te_NTN timing error limit for mobile VSAT is served by GSO

| frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

RAN4 to define timing accuracy requirements for case 4

For the uplink timing requirements for case 4, the majority companies agree with the requirements for mobile VSAT served by GSO could be reused. As the uplink timing requirements for fixed VSAT UE served by GSO and NGSO is same in legacy, the same logic could be considered for case 4 and mobile VSAT UE served by GSO. A few companies think the UE type is different for fixed VSAT and mobile VSAT. To our understanding, the legacy requirements for mobile VSAT UE served by GSO could be reused, and the side conditions could be considered modifying.

In legacy discussion, three side conditions are discussed:

- GNSS signal power (table 6.2-1 in TS 38.171)

- Satellite allocation (table 6.2-2 in TS 38.171)

- UE movement (UE velocity change within one second and UE moving direction change within one second)

From our perspective, GNSS signal power and Satellite allocation shall still follow the legacy conditions since the Te has already limited to 7.5Ts. The side condition for UE movement could be re-considered. We don’t have strong view on this and the concrete values could be discussed in this meeting.

Proposal 5: The concrete timing requirements for case 4 should be:

Table 3: Te_NTN timing error limit for mobile VSAT is served by NGSO

| frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 6: The side condition could be re-considered compared to the legacy one,especially on the UE movement.

  - 2.3 Potential RRM requirements

In the last meeting, the company just list the potential RRM requirements. In this chapter, RAN4 shall firstly recognize the potential RRM requirements based on the following list and then determine

- Which RRM requirements could be reused;

- Which need to be revised for editorial modification;

- Which need the new technical work.

- First of all, the potential RRM requirements for Ku band could be studied as below:

Table 1 Summary of RRM requirements which are possibly impacted by NTN operation in the NTN-Ku bands

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- To our understanding, R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands including timing requirements served by NGSO.

- Proposal 7: The following potential RRM impacts as follows:

Table 1 Summary of RRM requirements which are possibly impacted by NTN operation in the NTN-Ku bands

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- Proposal 8: R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Observation 1: Based on description of revised WID and BDaT session, the following two points shall be noticed:

- For FR1-NTN supporting numerology, not only 30kHz but 15kHz shall be supported.

- For FR1-NTN and FR2-NTN supporting numerology, same SCS is assumed for SSB and data channels.

Proposal 1: For applicable SCS, RAN4 shall also consider SCS=15kHz in FR1-NTN numerology.

Proposal 2: RAN4 shall notice that the same SCS is assumed for SSB and data channels for FR1-NTN and FR2-NTN numerology.

Proposal 3: The concrete timing requirements for case 1 and case 2 should be:

Table 1: Te_NTN timing error limit for mobile VSAT is served by GSO and NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 4: The concrete timing requirements for case 3 should be:

Table 2: Te_NTN timing error limit for mobile VSAT is served by GSO

| frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 5: The concrete timing requirements for case 4 should be:

Table 3: Te_NTN timing error limit for mobile VSAT is served by NGSO

| frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 120 | 7.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 6: The side condition could be re-considered compared to the legacy one,especially on the UE movement.

- Proposal 7: The following potential RRM impacts as follows:

Table 1 Summary of RRM requirements which are possibly impacted by NTN operation in the NTN-Ku bands

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- Proposal 8: R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands.

- References

[1] RP-243314, Revised WID: Introduction of Ku bands for NR NTN.