3GPP TSG-RAN WG4 Meeting #114bis                                         R4-2503808

Wuhan, P.R.China, 07th – 11th April, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF for Rel-19 LP-WUS

Agenda Item:	7.26.3

Document for:	Approval

Introduction

BS RF requirements have been further discussed for LP-WUS and some aspects to be further analysed are captured in WF [1].

This contribution provides further analysis for the issues identified in the WF.

Discussion

Dynamic range/power boosting

The most outstanding issue for BS RF part is the requirement of dynamic range/power boosting for LP-WUS. Some progresses were made in last meeting and the WF is listed as below:

- Adopt following content as the concept of LP-WUS power boosting.

  - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

- Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate.

- The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement.

  - FFS on the detail of the minimum power boosting level, e.g. Whether it can be different for different BS CBW

Based on the approved Way Forward, and similar to the NB-IoT approach, the following power boosting declarations could be considered for LP-WUS:

Table 1: Declarations for LP-WUS

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of [+3dB] for channel bandwidth >= [20MHz]. | x |  |

While defining a minimum power boost level is not required as a core requirement, at least one boost level should be included in the declaration.

Given that the NB-IoT-like power boost definition shows minimal sensitivity to larger channel bandwidths, a single boost level would be sufficient for wider channels (e.g., ≥20MHz). For smaller CBWs, different boost levels could still be declared.

Proposal 1: It is proposed to adopt power boost declarations similar to those specified in Table 1. A single minimum value should be declared for certain larger CBWs, which is not sensitive to CBW according to the boosting definition. Different boosting levels could still be declared for smaller CBWs.

Other requirements other than dynamic range

Besides the dynamic range requirement, other requirements have not been discussed in the SI and the first meeting of Rel-19 for LP-WUS. It is understandable that the introduction of LP-WUS should have minimal impact on BS implementation, as legacy BS must also accommodate this feature. In last meeting, we have the following WF:

  - Discuss on whether transmit signal quality should be defined

    - FFS whether equivalent EVM requirement could be considered,

      - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier

  - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

During the previous discussion, regardless whether requirements should be defined for spectrum related requirements, e.g. unwanted emissions, at least the common understanding among companies is: At least current regulation relevant spectrum requirements, e.g. SEM, SE should also be applied for LP-WUS.

Regarding transmitted signal quality, due to the inherent characteristics of low-order modulation in LP-WUS OOK signals, EVM metrics are not expected to be a bottleneck. Furthermore, frequency error can be ensured through testing with NR signals only. However, preliminary simulation results indicate that the embedded LP-WUS signal may affect the transmitted quality of the NR signal. The combined signals exhibit a slightly increased PAPR compared to NR signals alone, which could cause some performance degradation for NR, especially with higher modulation orders.

Two options are considered for the transmitter signal quality requirement:

- Option 1: Transmitted signal quality requirements should be defined for LP-WUS, but depending on further RAN1 progress on waveform design.

- Option 2: No need to introduce transmitted signal quality requirement for LP-WUS.

The choice of which option to consider for the LP-WUS EVM requirement also depends on the testing methodology. It is noted that for in-band operation, except the dynamic range requirement, no other requirements are specified for NB-IoT. Additionally, though LP-WUS draws from IEEE 802.11ba, there is also no EVM requirement for the OOK waveform there. Thus, we prefer to specifying only a dynamic range requirement, similar to NB-IoT in-band operation, leaving compliance to regulatory requirements up to implementation choices.

Specifically, transmitted signal quality of LP-WUS could be verified with 64QAM NR signal. Though the test is not directly performed for LP-WUS signal, if there is no obvious EVM degradation for NR signal with high order modulation scheme, the signal quality of LP-WUS can be ensured as well in the simultaneous transmission of both NR and LP-WUS.

Proposal 2: No specific transmit signal quality requirement is defined for LP-WUS. Instead, a higher-order EVM test for NR with embedded LP-WUS is utilized as the verification metric.

Conclusion

This contribution provides further consideration for BS RF of LP-WUS. With above analysis, we have the following proposals:

Proposal 1: It is proposed to adopt power boost declarations similar to those specified in Table 1. A single minimum value should be declared for certain larger CBWs, which is not sensitive to CBW according to the boosting definition. Different boosting levels could still be declared for smaller CBWs.

Table 1: Declarations for LP-WUS

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of [+3dB] for channel bandwidth >= [20MHz]. | x |  |

Proposal 2: No specific transmit signal quality requirement is defined for LP-WUS. Instead, a higher-order EVM test for NR with embedded LP-WUS is utilized as the verification metric.

- References

[1] R4-2502305, WF for [114][308] NR_LPWUS, Huawei, HiSilicon