3GPP TSG-RAN WG4 Meeting #114bis	R4-2503809

Wuhan, China,  7th April – 11th April 2025

Agenda item:	7.14.2.3

Source:	Apple

Title:	On FR1 MIMO OTA Dynamic Channel Model

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1  Introduction

# Discussion

The draft TP includes the following aspects:

- In Clause 8.2.1, update the measurement setup diagrams

- In Clause 8.2.3.6, define the PDP target values for UMa and UMi routes

- In Clause 8.2.4, update TCF targets for UMa route

- In Clause 8.2.5, clarify the offset distances

- In Annex A, correct ‘UE DoT Az’ for UMi route segments 7 through 10

- Misc editorial corrections

It should be noted that the UMa channel model targets are aligned with those from CTIA which underwent an extensive alignment between Keysight and Spirent.

It is proposed to approve the TP with the understanding that additional changes and modifications can be made.

Proposal 1: approve the TP with the understanding that additional changes and modifications can be made

A TP to TR 38.762 was submitted in [1] proposing definitions/updates for PDP/TCF/SCF targets for FR1 MIMO OTA Dynamic Channel model, i.e.:

The TP to TR [1] was submitted without a corresponding discussion TD. Despite the statement (highlighted in yellow) above, a close inspection against the cited CTIA UMa channel model targets indicated that most of the validation targets were not aligned with CTIA. The proponents in [1] were notified, and a contribution with technical justifications was requested to justify the discrepancy on those validation targets between CTIA and 3GPP.

# Discussion

The draft TP includes the following aspects:

- In Clause 8.2.1, update the measurement setup diagrams

- Correct the normalized PL values for UMi route in Table 8.2.2.4-2

- In Clause 8.2.3.6, define the PDP target values for UMa and UMi routes

- In Clause 8.2.4, update TCF targets for UMa route

- In Clause 8.2.5, clarify the offset distances

- In Annex A, correct ‘UE DoT Az’ for UMi route segments 7 through 10

- Misc editorial corrections

It should be noted that the UMa channel model targets from CTIA underwent an extensive alignment between Keysight and Spirent.

It is proposed to approve the TP with the understanding that additional changes and modifications can be made.

Proposal 1: approve the TP with the understanding that additional changes and modifications can be made

During the same RAN4#113 meeting, the TP to TR was revised in [2] with the same channel model validation targets, the revision on the statement (highlighted in yellow) below was considered insufficient justification, i.e.:

There was no consensus on the new channel model validation targets proposed in [2] and the TP to TR was postponed.

This contribution summarizes the discussion related to the channel model discrepancy between CTIA and 3GPP and proposes a resolution, including the potential specification impact.

# 2 Discussion

After RAN4#113, and during the CTIA MOSG December 2024 meeting, a contribution in [3] was approved further modifying the CTIA FR1 MIMO Dynamic Channel Model PDP targets. Despite these modifications, most of the discrepancy between CTIA and 3GPP Channel Model validation targets remains the same.

The summary of the discrepancy between CTIA and 3GPP FR1 MIMO Dynamic Channel Model targets was submitted during RAN4#114 in [4].

In the same meeting (RAN4#114), following requests made during RAN4#113 a contribution in [5] with technical justifications was submitted. Clarifying the technical reasons for the discrepancy on those validation targets between CTIA and 3GPP.

The technical justifications were accepted, and the RAN4#113 postponed TP to TR 38.762 was approved in [6].

As stated on RAN4#114 WF in [7]. It was agreed to consult different SDOs on the alignment feasibility with Channel Model validation targets approved in [6], i.e.:

# 1.Topic #1: FR1 dynamic MIMO OTA test methodology

### 1.1.1 Sub-topic 1-1 Dynamic channel model emulation and validation

Issue 1-1-1: Dynamic channel model validation targets in different SDOs

Agreement:

Pending the alignment of path loss models with different SDOs, the channel models and their applicability might need to be further clarified and/or renamed by RAN4#116.

After the RAN4#114 meeting, and during the March 2025 MOSG meeting. The contribution in [8] was submitted, providing the feedback information regarding the RAN4 inquiry on the alignment feasibility with CTIA Channel Model validation targets. From this contribution and similar observations found in [5], the proponents provided the following analysis based on the SIR model for interference on UMa:

Figure 1 [8]. SIR model for interference model for the UMa route adopting full path loss in [9]. Blue curves are the Rx power of the intended signal and two interfering signals as observed by an omni-directional UE. The red curve denotes the resulting SIR.

Observation 1: 		The dynamic range required for signal and interference while adopting full path loss in 38.901 is around 70dB.

During the preliminary studies presented in CTIA [10], a system integrator presented a data analysis questioning the feasibility of a commercial MPAC system capability to achieve the required large dynamic range for signal and interference.

Observation 2: 		System integrator questioned the feasibility to have a commercial MPAC system capable to generate the dynamic range for signal and interference following the adoption of full path loss defined in 38.901.

In response to this early concern raised by a system integrator, a CE vendor proposed an alternate path loss model based on dynamic Ricean K-factor, as demonstrated in Figure 2, i.e.:

Figure 2 [8]. SIR model for interference model for the UMa route adopting alternate path loss based on K-factor [9]. Blue curves are the Rx power of the intended signal and two interfering signals as observed by an omni-directional UE. The red curve denotes the resulting SIR.

Observation 3: 		The dynamic range required for signal and interference while adopting alternate path loss based on K-factor in 38.901 is around 35dB.

Based on these Observations, CTIA agreed to continue the development and deployment of its FR1 MIMO OTA Dynamic Channel Model test plan, based on the previously agreed alternate Ricean K-factor based path loss model.

Observation 4: 		CTIA agreed to continue the development of channel models with path loss variations. Ultimately adopting the Ricean K-factor based path loss which is defined on CTIA 01.42 test plan.

Observation 5: 		3GPP TR 38.762 adopts the full path loss defined on TR 38.901 [9] as a reference.

Based on the feedback received from CTIA Operators, Channel Emulator vendors and System Integrator engaged on the development of the first commercially available FR1 MIMO OTA with Dynamic Channel Model in [5, 8]. It can be observed that besides different environmental conditions, CTIA and 3GPP will also diverge on Path-Loss definitions and consequently will have different channel models validation targets.

Observation 6: 		Based on the analysis provided in [5, 8, and 10], CE vendors and system integrators consider deem unfeasible to have the same channel model validation targets for both environmental conditions, i.e.: SIR limited and Noise limited.

Observation 7: 		CTIA Operators, Channel Emulator vendors and system integrator propose in [8], to continue the development and deployment of CTIA FR1 MIMO OTA Dynamic channel model based on the alternate path loss based on Ricean F-factor.

Considering the decision made in CTIA. In response to the agreement captured in [7] issue 1-1-1, a clear differentiation between CTIA and 3GPP FR1 MIMO OTA Channel model nomenclature will be necessary to avoid future industry dubious interpretation. Therefore, clarification proposals are provided, i.e.:

Proposal 1:		RAN4 to rename the FR1 MIMO OTA Dynamic Channel Model as follows: FR1 UMa Dynamic Noise-limited and FR1 UMi Dynamic Noise-limited, these changes are applicable to TS 38.151 or any TS to be defined for dynamic channel models.

Proposal 2:	In TR38.762, update the channel model generation chapter, update references to channel models to clearly indicate their applicability only to the noise-limited environment, and update the applicability of validation targets to only the noise-limited channel models.

Proposal 3:	In TS38.151, make the applicability of the specification only to the noise-limited environment explicit.

# 3 Observations/Proposals

Observation 1: 		The dynamic range required for signal and interference while adopting full path loss in 38.901 is around 70dB.

Observation 2: 		System integrator questioned the feasibility to have a commercial MPAC system capable to generate the dynamic range for signal and interference following the adoption of full path loss defined in 38.901.

Observation 3: 		The dynamic range required for signal and interference while adopting alternate path loss based on K-factor in 38.901 is around 35dB.

Observation 4: 		CTIA agreed to continue the development of channel models with path loss variations. Ultimately adopting the Ricean K-factor based path loss which is defined on CTIA 01.42 test plan.

Observation 5: 		3GPP TR 38.762 adopts the full path loss defined on TR 38.901 [9] as a reference.

Observation 6: 		Based on the analysis provided in [5, 8, and 10], CE vendors and system integrators consider deem unfeasible to have the same channel model validation targets for both environmental conditions, i.e.: SIR limited and Noise limited.

Observation 7: 		CTIA Operators, Channel Emulator vendors and system integrator propose in [8], to continue the development and deployment of CTIA FR1 MIMO OTA Dynamic channel model based on the alternate path loss based on Ricean F-factor.

Proposal 1:		RAN4 to rename the FR1 MIMO OTA Dynamic Channel Model as follows: FR1 UMa Dynamic Noise-limited and FR1 UMi Dynamic Noise-limited, these changes are applicable to TS 38.151 or any TS to be defined for dynamic channel models.

Proposal 2:	In TR38.762, update the channel model generation chapter, update references to channel models to clearly indicate their applicability only to the noise-limited environment, and update the applicability of validation targets to only the noise-limited channel models.

Proposal 3:	In TS38.151, make the applicability of the specification only to the noise-limited environment explicit.

# 4  References

- R4-2419397, “On Dynamic MIMO OTA Testing”, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN4 Meeting #113, November 2024.

- R4-2419811, “TP for TR 38.762: On Dynamic MIMO OTA Testing”, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN4 Meeting #113, November 2024.

- OTAMIMO_2024_010_005 V1, “PDP Targets”, Keysight Technologies, Spirent Communications, December 2024

This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

- R4-2500481, “On FR1 MIMO OTA Dynamic Channel Model”, Apple, 3GPP RAN WG4 Meeting #114, February 2025.

- R4-2502175, “On Channel Models in CTIA and 3GPP for Dynamic FR1 MIMO OTA”, Keysight Technologies, Spirent Communications, CAICT, Huawei, AT&T, Verizon Wireless, 3GPP TSG-RAN4 Meeting #114, February 2025.

- R4-2502176, “TP for TR 38.762: DynMIMO CM Validation Details and Corrections”, Keysight Technologies, Spirent Communications, 3GPP TSG-RAN4 Meeting #114, February 2025.

- R4-252296, “WF for [114][328] MIMO_OTA_Ph3”, CAICT, 3GPP RAN WG4 Meeting #114, February 2025

- OTAMIMO_2025_003_003 V1, “On Path Loss Models”, Keysight Technologies, ETS-Lindgren, Spirent Communications, AT&T, and Verizon Wireless, March 2025

This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group member

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

- OTAMIMO_2021_009_005, “MIMO OTA Dynamic Range Considerations”, ETS-Lindgren, September 2021

This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group member