3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503841

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	NR NTN HPUE Rx Requirements

Agenda Item:	7.10.2.2.2

Document for:	Approval

Introduction

In the previous meeting, progress was made on PC2 NR-NTN RSD as following in the WF in [2]:

Issue 3-1-0: General

Agreement:

- For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5.

Issue 3-1-1: RSD requirements for PC2 on n256

Agreement:

- For PC2 on n256: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx.

Issue 3-1-2: RSD requirements for PC2 on n254

Agreement:

- For PC2 on n254: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx.

There are still open issues for all the power classes. In this paper, some of the proposals and analysis are provided.

HPUE Impact for NR-NTN RSD

General

Similar to [3] which submitted in last meeting, it should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined, and here is a table from [4] in Tx part:

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

For the differentiation of handheld and non-handheld, the main parameter in SD derivation are antenna isolation. However, as discussed before, more simplified requirements would bring a lot of merits, and unified requirements for handheld and non-handheld is preferred.

Observation 2: Unified requirements for handheld and non-handheld are preferred as much as possible.

PC2

As already been agreed in previous WF, the RSD is 0dB for n254/n256 for both 1Tx/2Tx, since there are sufficient duplex gap.

However, for n255, the situation is different. The range for n255 is same to n24, with less restriction in use. Currently the n24 do not have a MSD definition.

In [1], some other proposals are provided for n255, and proposals are listed:

Issue 3-1-3: RSD requirements for PC2 on n255

- Proposals

  - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek)

| Operating Band n255 | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| 1Tx | [0.4] | [0.4] | [0.5] | [0.5] |
| 2Tx | [0.8] | [0.8] | [0.9] | [0.9] |

  - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei)

    - PA noise as -117 dBm/Hz.

  - Option 3: Assuming IIP2 54dBm (Qualcomm)

| Operating Band n255 | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| 1Tx | 1.3 | 1.3 | 1.3 | 1.3 |
| 2Tx | 2.4 | 2.4 | 2.4 | 2.4 |

- Recommended WF

  - Clarify assumptions between different companies and agree on assumptions first.

It seems that some RSD looks reasonable for this case, but he proposed values still diverse, and the assumptions and ways of derivation may also different. It is proposed that more assumptions and derivation are considered.

In addition, up till now, no differentiation of handheld and non-handheld requirements is made. It is also quite aligned with the current way.

Proposal 2a: For NR-NTN power class 2, RSD for n255 need to be further studied, and RSD is likely to be defined for 1Tx/2Tx respectively.

As already been agreed, power class 2 is confirmed to be feasible for both handheld and non-handheld UE. Currently there is no differentiation of handheld / non-handheld UE for the RSD requirements, and the understanding is these requirements are based on common components and architectures.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1.5

Currently, 2Tx for non-handheld is confirmed for PC1.5, and the requirements need to be defined for all the bands.

Most of the assumptions If PC1.5 for 2Tx could be agreed to be defined, then it should not be too difficult for PC1.5 2Tx RSD derivation, since its similarity to PC2 2Tx in the derivation process.

Observation 2a: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined.

Furthermore, although only non-handheld for 2Tx PC1.5 considered currently, it should be noted that it may still be possible to extend the use case of 2Tx PC1.5 to handheld UE type in some later stage as long as implementation is possible. In this case, keep more aligned assumptions may bring some simplicity in the future and thus preferred.

Observation 2b: For PC1.5, although only non-handheld and 2Tx is considered currently, try to use assumptions also applicable for handheld may also meaningful.

PC1

According to early discussion in [3], following observations are submitted:

Observation 3a: Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 3b: For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

However, still non-zero value is proposed. We are not sure what kind of assumption is made for the duplexer and other components, and detailed analysis seems would be different compared to PC2/PC1.5. So if non-zero value proposed, then more analysis are needed.

In addition, since PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

Observation 3c: PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Any non-zero value proposal should be accompanied with more detailed assumptions of the reference components.

Proposal 4b: FFS for PC1 with 4Tx cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Observation 2: Unified requirements for handheld and non-handheld are preferred as much as possible.

PC2

Proposal 2a: For NR-NTN power class 2, RSD for n255 need to be further studied, and RSD is likely to be defined for 1Tx/2Tx respectively.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1.5

Observation 2a: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined.

Observation 2b: For PC1.5, although only non-handheld and 2Tx is considered currently, try to use assumptions also applicable for handheld may also meaningful.

PC1

Observation 3a: Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 3b: For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

Observation 3c: PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Any non-zero value proposal should be accompanied with more detailed assumptions of the reference components.

Proposal 4b: FFS for PC1 with 4Tx cases.

- References

[1] R4-2500680, Topic summary for [113][128] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#114

[2] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114

[3] R4-2500763, NR NTN HPUE Rx Requirements, vivo, RAN4#114

[4] R4-2503840, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114bis