3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503846

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on SRS IL imbalance reporting

Agenda Item:	7.1.1.3.4

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC.

In RAN4#110bis, the 6Rx related issue has been discussed. The topic summary is in [2] and a WF is agreed in [3]. A discussion paper in [4] is also provided for 6Rx related topics. Among this WF, the SRS IL imbalance is started. This has been discussed for a long time in REl-18 but no conclusion has been reached.

In RAN4#111, a dedicated agenda is set up for the SRS IL imbalance issue. A WF is agreed in [9] and summary is in [10], in which the common understandings were proposed to be discussed as the base for tentative solutions.

In RAN4#112, the common understandings were discussed. The summary is in [13] and WF in [14].

In RAN4#112bis, slight progress has been made and in WF [16], in that a new proposal 10 was introduced:

  - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

In RAN4#113, the WF[18] was updated mainly for Issue 3-2-1, to have some more candidate solution framework, rather than a list of proposals from different companies.

In RAN4#114, a WF [20] was updated. In that the IL imbalance issue was also updated for issue 3-2-1. More complete candidate solution framework was updated. However, the whole set of frame work is still as “candidate” and no conclusion made, it is apparent that anything more than implementation would be difficult to be agreed anymore.

In this contribution, most of previous discussion were not kept. We only kept the key proposals. Other background can reference earlier contribution [21].

Discussion

Based on current condition, it is apparent that complicated reporting scheme is unlikely to be passed in Rel-19.

Proposal 1: In Rel-19, it is proposed to consider self-compensation as the option and do not introduce any more reporting.

In RAN4#112bis, a proposal was formulated in the ad hoc as following:

  - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

Although some negative view was provided in [17] on similar proposal, this is by far the most simplified reporting scheme that could be considered. In this proposal, the UE is only indicate the capability that it would do self-compensation of SRS IL up to its capabilities, but not promising to what degree it can do. It provides the network some basic confidence that as long as the Tx power is well controlled and a bit far from maximum output power, the SRS IL would not pose any imbalance issue.

Observation: Even only indicating the capability of do compensation may provide some information on the balancing when still there is room for tx power.

Compared to the case of more information is dynamically reported, this proposal is simple enough to avoid the complexity issue. The performance gain would not necessarily much degraded compared to those more complicated solutions.

Proposal 2: If really needed, the single digit optional capability to indicates self-compensation of SRS IL up to its maximum Tx power capabilities could be considered as a compromise.

Conclusion

In this paper, the following proposals were provided:

Proposal 1: In Rel-19, it is proposed to consider self-compensation as the option and do not introduce any more reporting.

Proposal 2: If really needed, the single digit optional capability to indicates self-compensation of SRS IL up to its maximum Tx power capabilities could be considered as a compromise.

- References

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, RAN#103

[2] R4-2405282, Topic summary for [110bis][129] NR_ENDC_RF_Ph4, Moderator (Huawei, HiSilicon), RAN4 #110bis

[3] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[4] R4-2404666, Discussion on 6Rx for handheld and FWA UE, vivo, RAN4 #110bis

[5] R4-2303519, [Draft] LS on the UE SRS IL imbalance issue, RAN4#106

[6] R1-2306200, FL summary on UE SRS IL imbalance issue, Moderator (Huawei), RAN1#112bis-e

[7] R4-2321793, [Draft] LS on the UE SRS IL imbalance issue, RAN4#109

[8] R4-2405933, Initial discussion on SRS insertion loss imbalance reporting and 6Rx requirements, Ericsson, RAN4 #110bis

[9] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[10] R4-2408937, Topic summary for [111][126] NR_ENDC_RF_Ph4_part3, Moderator(AT&T), RAN4 #110bis

[11] R4-2408122, Discussion on SRS IL imbalance reporting, vivo, RAN4#111;

[12] R4-2305082, Discussion on 8Rx UE RF requirements, vivo, RAN4#106bis

[13] R4-2412820, Topic summary for [112][118] NR_ENDC_RF_Ph4_part3, Moderator (AT&T), RAN4#112

[14] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112

[15] R4-2412094, Discussion on SRS IL imbalance reporting, vivo, RAN4#112

[16] R4-2417209, WF on 6Rx UE RF requirements, AT&T, RAN4#112bis

[17] R4-2415787, Discussion on SRS IL imbalance reporting, vivo, RAN4#112bis

[18] R4-2420376, WF on 6Rx UE RF requirements, AT&T, RAN4#112bis

[19] R4-2418199, Discussion on SRS IL imbalance reporting, vivo, RAN4#113

[20] R4-2503014, WF on 6Rx requirements, AT&T, RAN4#114

[21] R4-2500761, Discussion on SRS IL imbalance reporting, vivo, RAN4#114