3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503871

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.30.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 IoT NTN

Document for:	Discussion

# Introduction

The new WID of NTN IoT was approved in [1] and revised [2][3]. In this contribution, we provide our views based on the latest progress in last RAN4 meeting [4].

# Discussion

In last meeting, RAN4 discussed and agreed to include procedural requirements for CB-MSG3.

Issue 1-1-1: Procedural requirements for CB-MSG3 transmission Agreement: - Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133. - Further discuss the content to include in the next meeting.

Taking existing section for RA requirements, which is shown as following, we could find that that are exactly extracted from RAN2 MAC spec.

### 6.6A.2 Requirements The UE shall have capability to calculate NPRACH transmission power according to the NPRACH power formula defined in TS 36.213[3] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in TS 36.102[60]. The relative power applied to additional preambles shall have an accuracy as specified in 36.102[60]. The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on target cell as specified in clause 5.1.4 in TS 36.321 [17]. #### 6.6A.2.1 Correct behaviour when receiving Random Access Response reception The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. #### 6.6A.2.2 Correct behaviour when not receiving Random Access Response reception The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 TS 36.321. #### 6.6A.2.3 Correct behaviour when receiving a NACK on msg3 The UE shall re-transmit the msg3 upon the reception of a NACK on msg3. #### 6.6A.2.4 Correct behaviour when receiving a message over Temporary C-RNTI The UE shall send ACK if the Contention Resolution is successful. The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message. #### 6.6A.2.5 Correct behaviour when contention Resolution timer expires The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

Based on the progress in last RAN2 meeting, the MAC Running CR is scheduled for post meeting discussion, which is intended to be endorsed in next RAN2 meeting. Thus, for this RAN4 meeting, we could not have solid MAC procedures to define procedural requirements. Thus, it is proposed to comeback in next meeting after the RAN2 progress on MAC Running CR is stable. And another thing should be drawn attention is that, we should avoid coping too much pure procedure requirements which are already clearly defined in RAN2 spec, which is also commented during last RAN4 meetings by several companies.

Proposal 1: RAN4 to wait until next meeting with more progress on RAN2 MAC CR.

Proposal 2: The procedural requirements for RA should be extracted and aligned with RAN2 MAC spec, and the number of the requirements should be limited.

# Conclusions

Proposal 1: RAN4 to wait until next meeting with more progress on RAN2 MAC CR.

Proposal 2: The procedural requirements for RA should be extracted and aligned with RAN2 MAC spec, and the number of the requirements should be limited.

# References

[1] RP-234077 New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] RP-241624 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[3] RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[4] R4-2502606 WF on RRM requirements for IoT_NTN_Ph3