3GPP TSG-RAN WG4 Meeting #114bis	R4-2503891

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on RLM/BFD test cases for 6Rx UE

Source: 	Huawei, HiSilicon

Agenda item:	7.1.2

Document for:	Discussion

Introduction

For R19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is to enable 6Rx in FR1 [1]. In this contribution, we will provide the discussion on RLM/BFD test cases for 6Rx capable UEs.

Discussion

R19 NR_ENDC_RF_Ph4 work item includes three objectives: High power UE (HPUE) for CA in terrestrial network (TN), Power boosting and/or MPR reduction, 6Rx for handheld and FWA UE. It is agreed in RAN4#112 meeting [2], the first two objectives have no impact on RRM requirements.

< Agreement > Issue 1-1: support for the higher power UE for CA - No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA. < Agreement > Issue 1-2: Power boosting and/or MPR reduction - No RRM work is expected due to the WI objective on power domain enhancements.

The third objective about 6Rx are duplicated as below [1].

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

As we know, the RRM core requirements for 6Rx capable UE have been completed in RAN4#113 meeting. The achieved agreements are summarized:

Agreements in RAN4#112 meeting [R4-2413972] < Agreement > Issue 1-3: Cell identification delay, measurement delay, mobility requirements for 6Rx capable UE Agreement: Agree on the following and no core spec impact. - The legacy core RRM requirements for identification delay, measurement delay, mobility requirements, scheduling restriction are reused for 6 Rx capable UEs. - Note: Interruption requirements at SRS antenna switching for 6Rx capable UE is discussed in a separate issue. Agreements in RAN4#113 meeting [R4-2420088] Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE Agreement: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no CR work is needed for core requirements.

As per work plan, the performance work will be carried out at this meeting.

RLM/BFD test cases for 6Rx capable UEs

It is agreed in RAN4#112 that RAN4 would introduce antenna connection for 6Rx capable UEs in performance part [2].

< Agreement > Issue 1-5: RRM performance requirements for 6Rx capable UE Agreement: - RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified: - Testing principles for 6Rx capable UEs - RLM and BFD testing

In current specification, all the RRM test cases defined in section A.4 and A.6 for FR1 are configured with two Rx antenna ports for UEs supporting 2RX. For RLM and BFD test cases, most of the test parameters except SNR level are the same for different Rx capable UEs. As the SNR levels configured for RLM and BFD tests are derived from the assumed values of Qin/Qout/Qout_LR, it may be impacted by the number of Rx antenna ports.

As we know the antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified in section A.3.6.1. In section A.3.6.1, the testing principles and the modified SNR levels for RLM and BFD tests are specified for 4Rx capable UEs. 4RX capable UE supporting 4RX on some but not all bands is only required to be tested on 2RX band with 2 RX antenna connection. 4RX capable UEs supporting 4RX on all bands is required to be tested on any one of 4RX band with 4 RX antenna connection, and the SNR levels of RLM and BFD test cases need to be modified according to table A.3.6.1.1.2.1-1, table A.3.6.1.1.2.1-2 and table A.3.6.1.1.2.1-3.

Table A.3.6.1.1.2.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| A.4.5.1.1 | -18 | N/A | N/A | N/A |
| A.4.5.1.3 | -18 | N/A | N/A | N/A |
| … | … | … | … | … |

Table A.3.6.1.1.2.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| A.4.5.1.2 | -18 | N/A | -8 | N/A |
| A.4.5.1.4 | -18 | N/A | -8 | N/A |
| … | … | … | … | … |

Table A.3.6.1.1.2.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5 (dB) |
| --- | --- |
|  | Test 1 |
| A.4.5.5.1 | -15 |
| A.4.5.5.2 | -15 |
| … | … |

In R18 NR_ENDC_RF_FR1_enh2, the test principle and SNR levels of RLM/BFD tests for 8 RX capable UEs follow that of 4 Rx UEs because only 4Rx is used for PDCCH reception even if UE has more RX antennas. Likewise, the RLM and BFD testing for 6Rx capable UE shall reuse SNR levels specified for 4Rx capable UE as well.

Proposal 1: RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE.

Regarding the antenna connection for 6Rx capable UE, a new clause would be created. The test principle of antenna connection for 8 Rx capable UE can be reused. To be more specific, the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall apply with “8RX” being replaced with “6RX”.

Proposal 2: A new clause of antenna connection for 6 Rx capable UEs is to be introduced, the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall be applied, with “8RX” being replaced with “6RX”.

Conclusions

In this paper we provided our analysis on RLM/BFD test cases for 6Rx capable UEs. The following proposals are provided.

Proposal 1: RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE.

Proposal 2: A new clause of antenna connection for 6 Rx capable UEs is to be introduced, the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall be applied, with “8RX” being replaced with “6RX”.

Reference

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon

- R4-2413972, “WF on RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC”, Huawei, HiSilicon