# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503941

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			NTN Ku bands – system parameters

Agenda item:		7.11.3

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In last RAN4#114 meeting, RAN4 agreed on the frequency range definition of the NTN Ku-bands in both FR1-NTN and FR2-NTN and their corresponding system parameters ([4]).

In this contribution, we are analyzing the impacts of the further update of the NTN WI.

# Discussion

## Transmission bandwidth configuration and minimum guard band

In last RAN#107 meeting, the NTN Ku-band WI ([1]) was further updated to include support for 15 kHz SCS in the 10, 15, 25, 35 and 50 MHz channel bandwidths and in FR1-NTN only.

To address this WI update, the RAN4#114 agreements shall be updated as proposed in the following.

Proposal1: Add the specification of the following transmission bandwidth configuration for FR1-NTN:

| SCS (kHz) | 10 MHz | 15 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 52 | 79 | 133 | 188 | 270 | N/A | N/A |
| 30 | N/A | N/A | 65 | 92 | 133 | 189 | 273 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Proposal2: Add the specification of the following minimum guard band for FR1-NTN:

| SCS (kHz) | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 312.5 | 382.5 | 452.5 | 522.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 665 | N/A | 805 | 785 | 925 | 1045 | 965 | 845 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

## Channel bandwidths and SCS for the NTN Ku-bands

Based on the agreements made in RAN4#114 on the NTN Ku bands ([4]), the following channel bandwidth and SCS should be specified for those bands.

Proposal3: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR1-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

The following proposal was not captured in the agreed WF ([4]) from last RAN4#114 meeting.

Proposal4: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR2-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

## NR ARFCN

Based on the agreements made in RAN4#114 ([4]) and the revised WI ([1]), the following NR ARFCN shall be further updated.

Proposal5: Update the FR1-NTN ARFCN according to the following table.

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <1> – 1333332 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

## NR GSCN

Based on the agreements made in RAN4#114 ([4]) and the revised WI ([1]), the following NR GSCN shall be further updated. As the WI clearly states that the same subcarrier spacing is assumed for SSB and data channels, RAN4 shall also specify 15 kHz SCS based SSB.

Proposal6: Update the FR1-NTN GSCN according to the following table.

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

# Conclusion

In this contribution, we analyzed the system parameters for the new NTN Ku-bands considering the last WID update. We made the following observations and proposals:

Proposal1: Add the specification of the following transmission bandwidth configuration for FR1-NTN:

| SCS (kHz) | 10 MHz | 15 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 52 | 79 | 133 | 188 | 270 | N/A | N/A |
| 30 | N/A | N/A | 65 | 92 | 133 | 189 | 273 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Proposal2: Add the specification of the following minimum guard band for FR1-NTN:

| SCS (kHz) | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 312.5 | 382.5 | 452.5 | 522.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 665 | N/A | 805 | 785 | 925 | 1045 | 965 | 845 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Proposal3: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR1-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

The following proposal was not captured in the agreed WF ([4]) from last RAN4#114 meeting.

Proposal4: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR2-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

Proposal5: Update the FR1-NTN ARFCN according to the following table.

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <1> – 1333332 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Proposal6: Update the FR1-NTN GSCN according to the following table.

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Eutelsat