3GPP TSG-RAN WG4 Meeting #114bis							        	R4-2503989

Wuhan, CN, 7th-11st April 2025

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			7.27.2.3

Document for:	Discussion

- Introduction

In RAN2#129 meeting, some conclusions on conditional intra-CU LTM have been duplicated as follows [1]. From this meeting, we will continue to discuss the RRM Core requirements on conditional intra-CU LTM.

RAN2#129 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. 2. The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. 3. For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. 7. Do not support DG-based first UL transmission for RACH-less conditional LTM. 8. For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. 9. For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. 10. Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

- Discussion

Issue 1-2-1: UE based TA measurement

< Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

In RAN2, both UE based TA measurement and PDCCH ordered RACH are introduced for early UL Synchronization in CLTM. And in R18 LTM, UE based TA measurement is also agreed in RAN2, however RAN4 did not define the relate requirements due to time limit.

What is more, the benefactions of CLTM are shorter latency, less signaling and higher robustness, in such a condition, it seems that UE based TA measurement is better than PDCCH ordered RACH. So, we think it is necessary to define RRM requirements for UE based TA measurement.

Proposal 1: Define RRM requirements for UE based TA measurement.

Conditional Intra-CU LTM delay

< Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

In last RAN2 meeting, some conclusions on conditional intra-CU LTM delay have been duplicated as follows.

2. The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

So, we think the starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Base on the definition of the conditional intra-CU LTM delay as shown above. The components of Conditional intra-CU LTM delay can contain these as follows

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +Tearly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM, and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Tearly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Proposal 2:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Due to this is the first meeting on conditional intra-CU LTM in all RAN WGs. We should revisit this issue when other WGs achieve further conclusions.

Proposal 3：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

CR skeleton

In previous meetings, we have discussed the specification structure of CLTM. And we could find that most specifications of LTM can be reused. The main ongoing issue is the CLTM delay. So, according to the work plan and the progress, CR split should be done in this meeting. After examining the agreements and ongoing open issues so far, maybe the following CR skeleton and notes could be a starting point for drafting the CRs.?

| Expected impact | Notes | New or impacted section | Companies |
| --- | --- | --- | --- |
| Definitions and abbreviations |  | 3.1 3.3 |  |
| CLTM delay requirements | Depending on RAN2 conclusion | (new) 6.x | China Telecom |
| UL transmit timing requirements | TBD: UE based TA measurement | 7.1 7.3 |  |
| PDCCH-order RACH on neighbor cell | Reuse the related R18 RAN4 requirements | 6.2.2C |  |
| TCI state activation | Reuse the related R18 RAN4 requirements | 8.25 |  |
| L1-RSRP measurement | Reuse the related R18 RAN4 requirements | 9.14 9.15 |  |
| L3 measurement based CLTM |  |  |  |

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: Define RRM requirements for UE based TA measurement.

Proposal 2:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 3：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

Proposal 4：CR skeleton

| Expected impact | Notes | New or impacted section | Companies |
| --- | --- | --- | --- |
| Definitions and abbreviations |  | 3.1 3.3 |  |
| CLTM delay requirements | Depending on RAN2 conclusion | (new) 6.x | China Telecom |
| UL transmit timing requirements | TBD: UE based TA measurement | 7.1 7.3 |  |
| PDCCH-order RACH on neighbor cell | Reuse the related R18 RAN4 requirements | 6.2.2C |  |
| TCI state activation | Reuse the related R18 RAN4 requirements | 8.25 |  |
| L1-RSRP measurement | Reuse the related R18 RAN4 requirements | 9.14 9.15 |  |
| L3 measurement based CLTM |  |  |  |

- References

[1] RP-250338, Status Report to TSG, Apple Inc.

[2] R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom.