3GPP TSG RAN WG4 Meeting #114-bis	R4-2504030

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.27.2.2

Source:	vivo

Title:	On RRM requirements for LTM CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

In the RAN4#114 meeting, RAN4 discussed RRM requirements for LTM CSI-RS based L1 measurement. Following agreements were made and captured in the WF [1].

#### Issue 4-1: Known cell definition in CSI-RS based L1 measurement Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. #### Issue 4-10: Restriction on CSI-RS resource configuration Agreement: - In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration. - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction: - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

In this contribution, we further provide our views on RRM requirements for LTM CSI-RS based L1 measurement.

# Discussion

## 2.1 Measurement procedure

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The following is concluded in RAN4#113 meeting.

For step 2, it was not concluded whether it is needed for FR2-1.

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

LTM is mainly for SpCell switch. In R18, RAN4 agreed that L1 measurement is pre-requisite to LTM procedures in FR2-1. In our understanding, for FR2-1, that means the main advantage of LTM is that UE can perform measurement for CSI-RS with fine Rx beam before cell switch, so that UE may perform faster cell switch and may be scheduled as early as possible.

From UE measurement perspective, the UE should use fine Rx beam to perform L1 measurement. For CSI-RS based measurement, it is under discussion whether a fixed fine Rx beam is used, which is determined from previous SSB-based L1 measurement, or Rx beam sweeping is used during the measurement. In general, with Rx beam sweeping during the measurement the UE can get better measurement results for optimized Tx and Rx beam pair, though the UE needs to perform more measurements compared to 2-step measurement. On the other hand, with 2-step measurement, the measurement complexity is smaller but the reported useful beams may be less due to a fixed Rx beam is used.

For LTM, we believe the two approaches could work well. Therefore, it is reasonable to introduce a UE capability so that the choice is left to UE implementation.

Proposal 1: For FR2-1, Whether step 2 can be skipped or not is based on UE capability reporting.

From signalling design perspective, the UE capability can indicate to network whether Rx beam sweeping is supported for measurements on CSI-RS resources with repetition OFF during LTM cell switch. If it is supported, the NW can configure CSI-RS based L1 measurement for LTM after SSB-based L3 measurement report. If not, then the NW needs to configured SSB-based L1 measurement firstly before configuring CSI-RS based L1 measurement for LTM.

Proposal 2: UE capability is to indicate whether Rx beam sweeping is supported for measurements on CSI-RS resources with repetition OFF during LTM cell switch.

## 2.2 Measurement definition related

There has been long discussion on definition of intra-frequency/inter-frequency measurement for CSI-RS based L1 measurement. It converged to following two options in the last meeting.

#### Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

When specifying definition of intra-frequency/inter-frequency, some general principles would be taken into account. For intra-frequency measurement, one aspect is that neighbour cell and serving cell can be measured simultaneously, and another aspect is that measurement gap is not supposed to be needed if the measurement resources are in the active BWP. For L3 measurement, the serving cell SSB is used as reference to define intra-frequency and inter-frequency measurement. It is better that similar principle is used for consistency of the definition. It means a reference signal of serving cell for L1 measurement is needed if it is intra-frequency measurement. It depends on the relationship between CSI-RS resource and active BWP whether gap is needed or not. So, it is also an important factor to be considered in the definition of intra-frequency measurement.

With above analysis, we would like to propose a bit change to option 2.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

Another issue would be how to define the case of within/outside active BWP of the UE.

#### Issue 4-5: Definition of “inside/outside active BWP” Candidate options: - Option 1: (Apple) The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP. - Option 2: (CATT) RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’. - Option 3: (vivo) From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

As discussed above, for serving cell, different CSI-RSs can be configured to different UE BWPs. However, this may not be the case for LTM candidate cell. As in Figure 1, Based on legacy definition, all cells in Figure 1 shall be intra-frequency cells, since their SSB frequency are the same. However, if UE connects to Cell C in Figure 1, based on legacy definition, CSI-RSs of Cell A and Cell B shall be out of active BWP.

Figure 1. Intra-frequency cells with different channel BW, and different CSI-RS w.r.t RBs and center frequency

Therefore, for gNBs that supports more than one active BWP, it is important for gNB to share one CSI-RS configuration that is more ‘common’ for different kinds of active BWPs on the same SSB frequency layer. In this case, it could be a typical case that the neighbor cell CSI-RS is outside UE active BWP. However, In TS 38.214 v 15.16.0, the following is captured in clause 5.2.2.3.1.

TS 38.214 v15.16.0 Clause 5.2.2.3.1 The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

Figure 2 has provided an illustration for this truncation. According to this truncation, only Case 5 is not measured. However, in our understanding, R15 RRM requirements did not consider this truncation.

Figure 2. Truncation of NZP CSI-RS based on R15 TS 38.214

Observation 1: If a NZP CSI-RS is configured to occupy a bunch of RBs, which are derived based nrofRBs and startingRB, and if the bandwidth of this bunch of RBs is wider than active BWP, UE may perform truncation on the CSI-RS first, based on bandwidth of active BWP, and then perform L1 measurement with the truncated CSI-RS, according to R15 TS 38.214.

Therefore, no matter how to interpret R15 RAN4 requirements, for R19 CSI-RS based LTM, in our view RAN4 shall first clarify the definition of CSI-RS based L1 measurements that is outside active BWP.

Proposal 4: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

- Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

- Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

- Option 3 (preferred): a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

One another issue related to measurement is how to define a frequency layer for CSI-RS based L1 measurement for LTM.

#### Issue 4-11: Definition of frequency layer Candidate options: - Option 1: (Apple) Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded. - Option 2: (MTK) If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”. - Option 3: (Huawei) If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer. - Option 4: (Nokia) Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity) - Option 5: (Ericsson) Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

In our understanding, it is quite challenging for the network to ensure both of the below points:

- All the cells having the same SSB frequency shares the same CSI-RS configuration in frequency domain.

- Any two cells having different SSB frequency do not share the same CSI-RS configuration in frequency domain.

For example, in Figure 1, Cell 3 shares the same SSB frequency as Cell 1 and Cell 2. However, the center-frequency and the number of RBs are different between CSI-RSs from different cells.

We can also consider another cell, e.g., Cell 4 in the scenario of Figure 1. It configures exactly the same CSI-RS as Cell 1, but SSB frequency of Cell 4 is different from Cell 1. From SSB perspective, Cell 4 and Cell 1 could be inter-frequency cells.

Note that for NZP CSI-RSs, the QCL relation is configured at a per-resource level, not at per-layer level. This is different from CSI-RS based L3 measurements. In legacy CSI-RS based L3 measurements, all resources configured in the same MO shall have the same center frequency, and associated to the same SSB frequency, which is restricted by RAN1/2 specs. However, for R19, RAN1 has already agreed not to have any restriction on the frequency location of CSI-RS.

Therefore, we have the following observation:

Observation 2: If a new definition of CSI-RS frequency layer is to be introduced, it is necessary to restrict that, the corresponding SSBs, which provides timing reference for the CSI-RSs on the same layer, also have the same SSB frequency, so that UE may obtain the time-frequency sync at the same time.

For the definition of intra-frequency measurement, one potential condition is that at least 48 RBs of the CSI-RS resource are confined within the active DL BWP. This may also be applicable to frequency layer definition that two at least 48 RBs of the CSI-RS resources among candidate cells are overlapping, then these CSI-RS resources can be considered as one frequency layer.

Proposal 5: From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement.

- If the CSI-RS resources among candidate cells are overlapping with at least 48RBs, and have the same SCS and CP type, it is considered as one frequency layer.

## 2.3 Requirements impact

It is not concluded yet whether to define requirements for gap-based CSI-RS measurement for LTM, and whether to define requirements for RTD > CP.

#### Issue 4-7: RTD assumption Candidate options: Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: Conclude this issue in the next meeting.

In last meeting, whether to also support the case when RTD>CP was futher discussed. The main concern from network side is that gNBs may not know the actual RTD at UE side, especially for the mobility scenarios.

Therefore, it would be OK to follow the same methodology as R18. RAN4 may define requirements for both RTD>CP and RTD<CP. An optional UE capability shall be introduced for the requirements when RTD<CP. To move forward, it is also okay not to define requirements for RTD > CP.

Proposal 6. Define requirements for both RTD<CP and RTD>CP and introduce an optional UE capability for RTD>CP, or not to define requirements for RTD > CP.

For the scenario of CSI-RS based L1 measurement performed outside active BWP, e.g., in another band that is different from current serving cells, we still doubt the benefit of such huge specification works. Here is our analysis based existing R19 RAN1/2 agreements.

Agreements in RAN1 #118 - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell Agreements in RAN1 #118bis - The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported - From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement Agreements in RAN2 #127bis - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

In R18 LTM, NZP CSI-RS resources can be configured for candidate cell. Existing TS 38.331 [3] is as follows.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

It is clear that R18 NZP CSI-RS can be configured for LTM candidate cells, while the point A of the NZP CSI-RS resource is configured per-resource. In other word, UE may not be able to know the actual channel bandwidth or the BWP where the NZP CSI-RS for LTM locates, based on the candidate cell configuration outside ltm-CandidateConfig-r18, i.e. configurations that UE can obtain before ASN.1 decoding of candidate cell. Since configuration signalling of the frequency domain position and RBs of the NZP CSI-RS for LTM follows R15 methods, UE may obtain the frequency location of candidate cell CSI-RS directly based on point A, SCS, startingRB and nrofRBs, without knowing the channel bandwidth or the BWP location.

R18 NZP CSI-RS for LTM is configured for the purpose of providing candidate cell TCI configuration. Normally the QCL-D source CSI-RS of PDCCH TCI is used for L1-RSRP measurements. Hence, it is straight forward that R18 NZP CSI-RS for LTM is re-used for the configuration of R19 CSI-RS for beam management, which is already agreed in RAN1.

Observation 3: RAN1 and RAN2 have agreed to re-use R18 CSI resource configuration as much as possible for R19 LTM, and candidate cell CSI-RS for beam management shall be used, at least, for CSI-RS based L1 measurement. In legacy, CSI-RS for beam management is also configured based on NZP CSI-RS configuration, and R18 NZP CSI-RS configuration already supports the case of candidate cell in LTM.

Observation 4: Frequency location of R18 NZP CSI-RS for LTM candidate cell is derived based on pointA, SCS, startingRB and nrofRBs that are per-resource configured, and RAN1/2 have agreed not to restrict frequency location of the CSI-RS from their perspective.

In legacy, UE can be configured with more than one BWPs for any active serving cells. In each BWP, the CSI-RSs for TRS, and the CSI-RS for L1-RSRP reporting can be specifically configured. Each CSI resource configuration is associated to a BWP-Id. Therefore, for serving cell CSI-RS based L1 measurement the used CSI-RS resource sets may change due to active BWP change, and it is much easier to ensure all configured CSI-RSs within the active BWP.

On the other hand, if the CSI-RS for BM configured in each cell is also used for LTM purpose, i.e. configured to neighbour cell UE, and if such information is shared between gNBs on different NR bands, e.g. an gNB on band n1 configures UE to perform CSI-RS based L1 measurement on band n78, the UE may have no any active BWP that can cover such CSI-RS. It is unclear what kind of channel bandwidth UE shall assume for such kind of L1 measurement. Without a worst-case assumption on RF filtering for the ‘inter-band’ case, it is difficult to discuss how UE can meet the legacy accuracy requirements. Note that such issue is not only for the ‘inter-band’ case. At least if the CSI-RS from an LTM candidate cell is completely outside any configured BWP(s) of the UE, such issue would happen.

The similar issue was also discussed in R18, and companies thought that UE can set up an ‘initial BWP’ for itself to perform L1 measurements on SSBs outside active BWPs. However, for CSI-RS, it is unclear how wide such BWP would be. Note that as discussed in previous proposal, to ensure mobility performance, a BWP shall not be too wide for multi-FFT based approach, and UE would need to ensure mobility performance under RTD>CP case based on multi-FFT

Observation 5: For CSI-RS based L1 measurement, if the CSI-RS is completely outside all configured BWPs of the UE, and if there is no any restriction in frequency domain configuration on such CSI-RS, UE may not know how wide UE shall set its RF filtering bandwidth, so as to efficiently perform mobility measurement based on these CSI-RS.

In our view, it would be more difficult for RAN4 to solve this issue in R19 CSI-RS based LTM. This would have quite a lot impact to the spec, e.g., gap scheduling and CSSF for the within gap case. It is quite risky that RAN4 specifies a lot of things but not implemented by any vendors. Hence, we prefer to deprioritize the CSI-RS based LTM L1 measurement outside active BWPs.

Proposal 7: Do not define requirements for the case where CSI-RS for LTM L1 measurements are outside active BWP(s).

CSI-RS based L1 measurement with gap includes two scenarios. One is the case that the CSI-RS is outside active BWP as discussed above. The other case is that CSI-RS is inside active BWP, but fully overlapped with measurement gap. In our view, this case shall also be de-prioritized.

Proposal 8: RAN4 do not support CSI-RS based L1 measurement with gap in Rel-19.

#### Issue 4-12: Whether to consider CSI-RS based L1-SINR measurement Candidate options: - Option 1: (CATT, Apple) RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. - Option 2: (ZTE, CT) Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

In RAN1, there is no consensus on the support L1-SINR measurement based on CSI-RS for candidate cells. So, L1-SINR measurement is not supported for LTM. RAN4, therefore, doesn’t need to define corresponding requirements.

Proposal 9: Do not define RRM requirements for CSI-RS based L1-SINR measurement for candidate cells.

In the past meetings, RAN4 discussed whether repetition = ‘ON’ shall be supported for LTM candidate cells.

#### Issue 4-6: Measurement on CSI-RS with repetition ON/OFF Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

There were similar discussions in RAN1. Actually, this discussion shall be in RAN1 scope. The key points are:

- What are the additional spec impacts if repetition = ‘ON’ is supported for LTM candidate cell, on top existing R18 LTM L1 reporting UCI definition? Note that in this case SSBRI or CRI shall not be reported

- What is the benefit of having such feature? For example, how many UE can be able to support repetition = ‘ON’?

For the 2nd key point, according to TS 38.133, UE needs to try as many as ‘maxNumberRxBeam’, on the number of resources in the resource set. For LTM L1 measurements, this set can contain much more resources than serving cell. Therefore, it would be difficult for LTM L1 measurement to support repetition on.

Proposal 10: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1.

RAN4 discussed CSI-RS resource type in the past meetings.

#### Issue 4-8: CSI-RS resource type Candidate options: - Option 1: (Apple, OPPO, Nokia, vivo) RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision. - Option 2: (CMCC, Ericsson) Semi-persistent CSI-RS resource for LTM candidate cells is supported. - Option 3: (CATT) RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

RAN1 made following agreements on the CSI-RS resource type.

Agreements in RAN1 #118 - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell Agreements in RAN1 #118bis Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability.

In the RAN1#120 meeting, RAN1 confirmed the working assumption.

Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

Since RAN1 has confirmed to support semi-persistent CSI-RS, it is better to define requirements for this either. Considering only three meetings left, simplified requirements are expected to support SP CSI-RS for LTM.

Proposal 11: CSI-RS based L1 measurement requirements are specified for both periodic CSI-RS and semi-persistent CSI-RS.

## 2.4 Conditions

The pre-requisite measurement for CSI-RS based L1 measurement on LTM candidate cell was discussed.

Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

For SSB-based L1 measurement for LTM, the SINR side conditions is -3dB. To support CSI-RS based L1 measurement for LTM, the UE should support SSB-based LTM as pre-requisite feature. It doesn’t make sense that the two measurements are conditioned on different SINRs.

Proposal 12: SSB based L1 measurement is the pre-requisite for CSI-RS based L1 measurement on an LTM candidate cell. Same side condition of SINR=-3dB is reused.

Proposal 13: RRM requirements for CSI-RS based LTM are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

# Summary

In this contribution, we further provided our views on RRM requirements for LTM CSI-RS based L1 measurement. Following proposals and observations are present.

Proposal 1: For FR2-1, Whether step 2 can be skipped or not is based on UE capability reporting.

Proposal 2: UE capability is to indicate whether Rx beam sweeping is supported for measurements on CSI-RS resources with repetition OFF during LTM cell switch.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

Proposal 4: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

- Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

- Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

- Option 3 (preferred): a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Proposal 5: From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement.

- If the CSI-RS resources among candidate cells are overlapping with at least 48RBs, and have the same SCS and CP type, it is considered as one frequency layer.

Proposal 6. Define requirements for both RTD<CP and RTD>CP and introduce an optional UE capability for RTD>CP, or not to define requirements for RTD > CP.

Proposal 7: Do not define requirements for the case where CSI-RS for LTM L1 measurements are outside active BWP(s).

Proposal 8: RAN4 do not support CSI-RS based L1 measurement with gap in Rel-19.

Proposal 9: Do not define RRM requirements for CSI-RS based L1-SINR measurement for candidate cells.

Proposal 10: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1.

Proposal 11: CSI-RS based L1 measurement requirements are specified for both periodic CSI-RS and semi-persistent CSI-RS.

Proposal 12: SSB based L1 measurement is the pre-requisite for CSI-RS based L1 measurement on an LTM candidate cell. Same side condition of SINR=-3dB is reused.

Proposal 13: RRM requirements for CSI-RS based LTM are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

Observation 1: If a NZP CSI-RS is configured to occupy a bunch of RBs, which are derived based nrofRBs and startingRB, and if the bandwidth of this bunch of RBs is wider than active BWP, UE may perform truncation on the CSI-RS first, based on bandwidth of active BWP, and then perform L1 measurement with the truncated CSI-RS, according to R15 TS 38.214.

Observation 2: If a new definition of CSI-RS frequency layer is to be introduced, it is necessary to restrict that, the corresponding SSBs, which provides timing reference for the CSI-RSs on the same layer, also have the same SSB frequency, so that UE may obtain the time-frequency sync at the same time.

Observation 3: RAN1 and RAN2 have agreed to re-use R18 CSI resource configuration as much as possible for R19 LTM, and candidate cell CSI-RS for beam management shall be used, at least, for CSI-RS based L1 measurement. In legacy, CSI-RS for beam management is also configured based on NZP CSI-RS configuration, and R18 NZP CSI-RS configuration already supports the case of candidate cell in LTM.

Observation 4: Frequency location of R18 NZP CSI-RS for LTM candidate cell is derived based on pointA, SCS, startingRB and nrofRBs that are per-resource configured, and RAN1/2 have agreed not to restrict frequency location of the CSI-RS from their perspective.

Observation 5: For CSI-RS based L1 measurement, if the CSI-RS is completely outside all configured BWPs of the UE, and if there is no any restriction in frequency domain configuration on such CSI-RS, UE may not know how wide UE shall set its RF filtering bandwidth, so as to efficiently perform mobility measurement based on these CSI-RS.

- References

- R4-2502601	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple