3GPP TSG RAN WG4 Meeting #114bis	R4-2504048

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.21.2.2

Source:	vivo

Title:	Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

According to the latest working plan agreed in RAN4#114 meeting, the discussion on potential RRM impacts for measurement event prediction will be initiated in this meeting.

RAN4 agreements in RAN4#114 Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

According to the above schedule, the initial discussion is allowed for the measurement event prediction use-case in this meeting. Next, we will make some preliminary analysis of measurement event prediction use-case based on the latest RAN2 progress.

- Discussion

As shown below, we have taken some of the conclusions reached at previous RAN2 meetings for reference.

RAN2#127bis - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (e.g., event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference. RAN2#128 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’ RAN2#129 Agreements on Measurement event case A 1 Most of the results show that the F1 score for prediction of measurement event is very good. 2 For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. Agreements on Measurement event case B 1 Measurement event case B can have very good F1 score depending on filtering approach/PW 2 F1 score is good for the PW window for case B (i.e. low PW) 3 F1 score decreases with an increasing MRRT value 4 We will focus on finding cases that bring benefits to the system rather than trying to compare 5 With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism. Other agreements Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

In the box above, we have highlighted a few key conclusions about the progress achieved at the previous meetings of RAN2. The main summary is shown as follows:

- Indirect measurement event prediction and direct measurement event prediction are both within the scope of RAN2 research. In particular, the difference between the two scenarios is mainly reflected in: for the scenario of indirect measurement event prediction, in addition to the expected occurrence time of a certain measurement event as final output, there is also a further RSRP of serving/neighboring cells as intermediate output.

- Specifically, For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

- For indirect measurement event prediction, based on consideration and studied scenarios, there will be the following four combinations:

  - Indirect measurement event prediction for temporal domain case A

  - Indirect measurement event prediction for temporal domain case B

  - Indirect measurement event prediction for frequency domain

  - Indirect measurement event prediction for spatial domain

- For KPIs for measurement event prediction, RAN2 to adopt the following KPIs for measurement event prediction use case:

  - for indirect: the RSRP difference, missed event detection, false event detection, F1 score, time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction.

  - for direct: missed event detection, false event detection, F1 score.

- On the evalution under the F1 score，the measurement event prediction for both Case A and Case B have good performance.

Combined with the above conclusions, from RAN4 perspective, for indirect scenario, the intermediate result reported is the predicted value of the service cell/neighborhood (A3 event). This is the same as the current RRM measurement prediction use case for temporal domain. The discussion and conclusion on RSRP difference prediction accuracy in RRM measurement prediction use case can be directly reused without extra work effort.

For additional KPIs proposed by RAN2, for example, time difference of true time event reporting triggered and predicted time event reporting triggered, since RAN2 is also conducting further performance evaluations on these KPIs, RAN4 can wait for further evaluation results of RAN2, and then discuss whether to define relevant KPIs in RAN4.

Observation 1: For indirect measurement event prediction, the intermediate result reported is the predicted value of the service cell/neighborhood (A3 event), which is the same as the current RRM measurement prediction use case for temporal domain

Proposal 1: On the study of RRM impact on measurement event prediction, RAN4 can leverage of the progress made in RRM measurement prediction

In particular, for the studied scenario in RAN4, since we already discussed the priorities of scenarios in general aspect, we can correspondingly deprioritize scenario 6 first and study scenario 2,3, and 4 first in measurement event prediction.

Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

- Summary

Observation 1: For indirect measurement event prediction, the intermediate result reported is the predicted value of the service cell/neighborhood (A3 event), which is the same as the current RRM measurement prediction use case for temporal domain

Proposal 1: On the study of RRM impact on measurement event prediction, RAN4 can leverage of the progress made in RRM measurement prediction

Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2502594, WF on AI/ML Mobility, Nokia