3GPP TSG RAN WG4 Meeting #114bis	R4-2504051

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.29.4.2

Source:	vivo

Title:	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on the objectives of downlink coverage enhancements. The considerations from companies and the latest progress were summarized in WF [1]. In this contribution, we would like to provide further analysis of the potential RRM impacts on DL coverage.

- 2.	Discussion

  - Downlink coverage enhancement

    - For the requirements that include SSB periodicity > 20ms

- RRC re-establishment:

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

First, we would like to recall the identification of Tidentify_inter_NR, i in the R15 discussion requirement, where:

The 800ms for unknown cell case came from the current NR measurement requirements the minimum cell search times for FR1 is 600 ms + 200 ms (800 ms). For the case specified in note 1 when TSMTC,i > 20 ms and serving cell SSB Es /Iot < -8 dB, this is because when SNR is < -8, UE may be almost losing track of the serving cell timing. In this case, it is infeasible to search all frequency layers, especially with SMTC periodicity larger than 20ms. Therefore, considering the TSMTC is always less than 20ms, max (800, 13TSMTC) should be simplified as 800ms in FR1 for Es /Iot < -8 dB unknown case.

Now if we consider the SSB periodicity is extended to 160ms in DL coverage enhancement, the SMTC assumption will be accordingly adopted as 160ms to consider the worst case.

Regarding the FFS part on whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined, we think this update shall apply for all Rel-19 UEs. One reason is that, if this update is only applied for DL coverage capable UE, it may occur that DL coverage capable UE will assume 160ms periodicity for blind search while for DL coverage not-capable UE will still try 20ms for cell search. When NW transmit SSB with 80ms periodicity, DL coverage not-capable UE will fail to detect nothing. Another reason is that, from specification perspective, if we differentiate two default values of SMTC by UE capability, two sets of requirements may be needed respectively for TSMTC ≤20ms and TSMTC >20ms.

And for the FFS whether the above is for only inter-frequency or for both intra-frequency and inter-frequency, considering for intra-frequency, SMTC periodicity is always configured and it is specified that Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. From this perspective, no additional SMTC assumption is needed.

Proposal 1: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. Specifically, it will apply for all Rel-19 UEs and only for inter-frequency

On how to update the spec with consideration of the case SSB Ês/Iot < -8dB and SSB periodicity >20 ms, it mainly includes two alternatives. One is to accordingly update the Tidentify_inter_NR, i for Es /Iot < -8 dB unknown case to adapt TSMTC,i=160ms, which is like MAX(k * 800 ms, [X] x TSMTC, i). The X value can be further determined. Another way is to follow the Note 1 that UE is not required to meet the requirement when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. We are open to discuss which of the above methods to use. Specifically, for the first method, it may result in an excessively long re-establishment time due to 160ms periodicity.

Proposal 2: On whether/how to update the RRC Re-establishment in case of SSB Ês/Iot < -8dB and SSB periodicity >20 ms, RAN4 to

- Option 1: Update Tidentify_inter_NR, i for SSB Ês/Iot < -8dB unknown cell as MAX (k * 800 ms, [X] x TSMTC, i)

- Option 2: Still assume that the UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB and no update is needed

- Summary

Proposal 1: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. Specifically, it will apply for all Rel-19 UEs and only for inter-frequency

Proposal 2: On whether/how to update the RRC Re-establishment in case of SSB Ês/Iot < -8dB and SSB periodicity >20 ms, RAN4 to

- Option 1: Update Tidentify_inter_NR, i for SSB Ês/Iot < -8dB unknown cell as MAX (k * 800 ms, [X] x TSMTC, i)

- Option 2: Still assume that the UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB and no update is needed

- References

- R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated