3GPP TSG RAN WG4 Meeting #114bis	R4-2504052

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.27.2.3

Source:	vivo

Title:	Further discussion on Conditional Intra-CU LTM

Document for:	Discussion

- Introduction

After the discussion of the past three meetings, the general aspect on defining delay requirements on CLTM is determined. In this paper, we will further provide our thinking on the details of the definition of delay requirements for supporting conditional Intra-CU LTM based on the latest WF [1].

- Discussion

In the last meeting, RAN2 further reached some constructive agreements in terms of inter-CU LTM and C-LTM as follows. Next, we will further analyze the RRM impacts on requirements for conditional LTM delay based on the following latest progress.

RAN2 Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

<conditional LTM>

Issue 1-2-1: UE based TA measurement RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK). < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

In the last meeting, we discussed the requirements for defining L1 measurement on deactivated Scell and UE-based TA measurement in RAN4. Although, in our view, both features are necessary for the CLTM mechanism, we tend to prioritize the definition of UE-based TA. The main considerations are as follows:

Firstly, for conditional LTM，since configured events are evaluated by the UE itself, the network is not able to know in advance when triggering conditions of conditional LTM will be met and to which candidate cell the UE will switch. Therefore, UE may not have TA of the target cell derived from the source cell in advance before the triggering condition is satisfied. In this case, the UE-based TA can be used as a supplementary method for providing valid TA so that the UE can execute the cell switch in time once it realizes that the trigger condition is fulfilled. From this point of view, we see that UE-based TA has more benefits for CLTM features than LTM.

Secondly, regarding the UL timing error introduced by the UE-derived TA for neighboring cells, compared to the existing timing error, the main additional introduction is the UE downlink synchronization error for the target cell. When the synchronization between the network side and the serving cell/target cell is well-handled, the resulting error is minimal and acceptable in most SCS scenarios.

Therefore, we do see the benefit and feasibility of introducing UE-based TA in CLTM. Accordingly, the corresponding requirements should be considered in RAN4.

Proposal 1: RAN4 to define RRM requirements for UE-based TA measurement in CLTM

Issue 1-3-3: The components of conditional LTM delay < Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Based on the discussions in previous meetings, there is a consensus that the CLTM delay framework should use the existing CHO delay framework as a baseline. The main divergence lies in whether to optimize within the current LTM and CHO framework by performing early time/frequency (T/F) tracking before Tprocessing. Our understanding on this issue is as follows:

According to R16 CHO discussions, after the UE determines that a neighboring cell meets the relevant evaluation conditions, it will disconnect from the source cell—for example, by releasing UL data compression configurations and leaving the source cell. Only after this step can the UE proceed with software processing (e.g., RF warm-up). In our view, fine time/frequency tracking with the target cell should occur after completing UE processing related to the target cell (e.g., software processing including RF warm up). The approach of performing fine synchronization while the UE is still camped on the source cell may not be applicable in all scenarios. Therefore, we prefer to align with the existing CHO delay structure, placing fine tracking within Tinterruption time—i.e., after UE processing is completed.

Additionally, regarding TLTM-RRC-processing, based on LTM discussions, this component is included in TCLTM_interrupt. We do not see a strong necessity to separate it from TCLTM_interrupt and treat it as an independent part of the delay.

Therefore, in our view, we prefer Option 3, which adopts the existing CHO delay structure and specificlly for TCLTM_interrupt, the legacy delay components within TLTM_interrupt can be used.

Proposal 2: RAN4 to define the timeline of CLTM cell delay requirement as follows. The definition and corresponding value of each delay component can be further determined.

DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Issue 1-3-4: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay. - Proposal 2: Too early to agree whether TRRC is needed. Issue 1-3-5: TEvent_DU < Way forward > FFS the following proposals: - Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Proposal 1a: It can cover both cases: - UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation - Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met. - Proposal 2: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. - Proposal 3: Wait for more RAN2 agreements. Issue 1-3-6: Tmeasure < Way forward > FFS the following proposals: - Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - Proposal 1a: - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on CSI-RS based L3-RSRP measurement. - Proposal 1b: - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure. - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO. Issue 1-3-7: TCLTM_execution / TLTM_execution < Way forward > FFS the following proposals: - Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Proposal 1a: TLTM_execution can be up to 10ms. - Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay. Issue 1-3-8: Tinterrupt < Way forward > FFS the following proposals: - Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL transmission on the target cell. - Proposal 2: The term Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure. - Proposal 3: Wait for more RAN2 agreements. Issue 1-3-9: The components of Tinterrupt < Way forward > FFS the following proposals: - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. - Proposal 2: Excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU. - Proposal 3: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Regarding TRRC, based on the conclusion from the last RAN2 meeting, the UE starts execution condition evaluation upon receiving the CLTM configuration, including the C-LTM execution condition. This behavior aligns with the existing CHO UE procedure, where the corresponding RRC processing time can reuse the current processing time defined for RRC reconfiguration messages containing CHO configuration.

Proposal 3: For CLTM, the definition and corresponding value of exsisting TRRC in CHO can be reused

For TEvent_DU, in addition to the existing CHO consideration—where it simply waits for "a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM"—we need to further evaluate whether this definition remains applicable for the early UL/DL synchronization procedure in CLTM.

Regarding PDCCH ordered RACH, According to the RAN2 conclusions reached in the previous meeting, we have the following analysis:

- The early TA is signaled to the UE from the source cell and the candidate cell's TA information to UE via new MAC CE by NW. Unlike the Rel-18 LTM, when UE receives the conditional LTM configuration in the RRC message since configured events are evaluated by the UE itself, the network is not able to know in advance when triggering conditions of conditional LTM will be met and to which candidate cell the UE will switch. Therefore, it is more appropriate to send the candidate cell's TA information to the UE as early as possible and complete UL sync to ensure that the TA of the target cell has been obtained by the time the UE executes the cell switch.

- The candidate cell's TA information to UE via new MAC CE by NW will be maintained by a new timer: The TA value can be regarded as valid when the timer is not expired when the UE triggers the cell switch.

From the above two points, we can see that to support PDCCH ordered early TA acquisition mechanism for CLTM, the process needs to be completed before UE starts evaluation. That is, in addition to the evaluation criteria configured by the network, whether the candidate cell's TA information is received and whether the TA value is valid should also be considered as a condition for performing CLTM with early UL sync. From this point of view, time duration specifically for UL sync can be considered involved in TEvent_DU.

Considering RAN2 has not yet made a clear conclusion as to when NW will send MAC CE to inform the candidate cell's TA information to UE (it may depend on the network implementation), We can wait for RAN2's further conclusions before deciding the specific delay definition.

For DL sync procedure, there may be UE involvement to decide when and which to perform TCI state activation/deactivation, e.g., totally decided by UE itself based on L1 measurement or which TCI state is preferred according to some priority rules. The details will be further discussed in RAN2 design. Thus, similar to the UL sync analyzed above, for Early TCI state activation for CLTM to be supported, the process needs to be completed before UE begins its evaluation. Therefore, this time specifically for DL sync can also be considered involved in TEvent_DU.

Proposal 4: For CLTM, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. Specifically, it can cover both cases:

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

For measurement time, based on the current RAN2 agreements as follows, it can be observed that:

- candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

For L3 measurements, the existing CHO evaluation events (A3 and A5) will be reused. Accordingly, the current definition of Tmeasure and its corresponding L3 measurement requirements can also be reused in CLTM.

conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation of measurement events is per candidate beam. CLTM will be triggered for execution as long as one beam meets the event condition. If multiple beams satisfy the condition, it is up to UE implementation to select a beam and perform C-LTM. For L1 measurements, the relevant L1 measurement period for neighbor cells has been defined in R18 LTM. This can serve as a starting point for defining the CLTM measurement time.

Besides, CSI-RS based L1 measurement is introduced in Rel-19 LTM, and the detailed requirements are currently being discussed in RAN4 in parallel. From a feature completeness perspective, we prefer to define the corresponding requirements for CSI-RS based L1 measurement for CLTM in Rel-19, provided that the requirements on CSI-RS based L1 measurement can be fully defined.

Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- When the condition is L1-based:

  - For SSB based L1-RSRP measurement, RAN4 to use the Rel-18 L1 measurement period for neighbor cell to define Tmeasure.

  - If CSI-RS based L1-RSRP measurement requirements is defined in Rel-19 LTM, it can be reused in Tmeasure

- When the condition is L3-based: Tmeasure to follow the definition of Tmeasure for CHO.

<subsequent conditional LTM>

Issue 1-4-1: General aspects of Subsequent Conditional LTM delay RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.. < Way forward > FFS the following proposals: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 2: Wait for more RAN2 agreements before defining the requirements for subsequent CLTM. Issue 1-4-2: The starting point of Subsequent conditional LTM delay < Agreement > ad hoc - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM]. - Revisit if RAN2 has more progress. Issue 1-4-3: The components of subsequent conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

From delay requirement point of view, compared with conditional LTM, the only difference for subsequent conditional LTM is the starting point. The subsequent conditional LTM starts from the completion of LTM procedure for the previous PCell. Apart from this, the delay requirement on conditional LTM can be reused to define the remaining components.

Proposal 6: For subsequent conditional LTM, Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM.

- Summary

Proposal 1: RAN4 to define RRM requirements for UE-based TA measurement in CLTM

Proposal 2: RAN4 to define the timeline of CLTM cell delay requirement as follows. The definition and corresponding value of each delay component can be further determined.

DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Proposal 3: For CLTM, the definition and corresponding value of exsisting TRRC in CHO can be reused

Proposal 4: For CLTM, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. Specifically, it can cover both cases:

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- When the condition is L1-based:

  - For SSB based L1-RSRP measurement, RAN4 to use the Rel-18 L1 measurement period for neighbor cell to define Tmeasure.

  - If CSI-RS based L1-RSRP measurement requirements is defined in Rel-19 LTM, it can be reused in Tmeasure

- When the condition is L3-based: Tmeasure to follow the definition of Tmeasure for CHO.

Proposal 6: For subsequent conditional LTM, Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM.

- References

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom