# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2504056

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.24.4

Source:	Qualcomm Incorporated

Title:	Views on Ambient IoT Testability

Document for:	Approval

# 1  Introduction

In RAN#106, the new WI on solutions for Ambient IoT (A-IoT) in NR was approved in [1]. RAN4 is tasked to study and develop the test methodology for A-IoT device 1 according to D1T1-B scenario.

- Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

In RAN4#114, the agreements for A-IoT device OTA test method were captured in [2]. In this paper, we provide our further views on the OTA test methodology for A-IoT device.

# 2  OTA test methdology for A-IoT device

## 2.1 General test setup for A-IoT device testing

As agreed in [2], anechoic chamber is considered as the baseline for device 1 testing in Rel-19. Therefore, the traditional far-field test system of AC for handheld UE TRP and TRS testing specified in section A.3 of TS 38.161 [3] can be considered as the starting point for A-IoT device testing.

### 2.1.1 Chamber setup

Since the target scenario for Rel-19 A-IoT WI is D1T1-B in which CW node is outside, the possible test setup shown in Figure 1 with separate CW/RF energy source and measurement antenna descripted in [2, 4] can be used as the starting point of test setup for A-IoT device OTA testing. Different from traditional AC for handheld UE TRP and TRS testing, at least the following test setup parameters shall be defined for A-IoT device testing with the specific criteria.

Figure 1: An example of test setup for A-IoT device OTA testing

Proposal 1: The following test setup parameters shall be defined for A-IoT device OTA testing with the specific criteria:

- DMA2D: distance between measurement antenna and DUT

  - To follow the minimum range length for TRP and TRS OTA testing with 30cm quiet zone size defined in Table A.3.4-1 of TS 38.161

- DCW2D: distance between CW node and DUT

  - To follow the minimum range length for TRP and TRS OTA testing with 30cm quiet zone size defined in Table A.3.4-1 of TS 38.161

- IsoCW2M: Isolation between CW node and measurement antenna

  - To ensure that the received D2R signal in measurement antenna is X dB higher than CW signal leakage to measurement. FFS on X value

Proposal 2: Ripple Test for Quiet Zone specified in section 7.6 of TR 38.870 can be reused for AIoT device testing.

### 2.1.2 Position of Device

For the position of Device, the centre of the reference coordinate system shall be aligned with the geometric centre of the DUT in order to minimize the offset between antenna and the centre of the quiet zone. The free space DUT orientation 1 specified in Table J.2-1 of TS 38101-2 can be assumed as the default position of device.

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space DUT Orientation 1 (default) | α = 0º; β = 0º; γ = 0º | θLink; ϕLink with polarization reference PolLink = θ or ϕ | θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ |  |

Proposal 3: The free space DUT orientation 1 specified in Table J.2-1 of TS 38101-2 can be assumed as the default position of device.

### 2.1.3 Energy source and energy harvesting

In [1], it was agreed to further discuss the energy source issue. First of all, energy harvesting (EH) is one of key device behavior for AIoT device 1. Even though EH procedure is not to be specified and verified, EH procedure should not be skipped in the testing.

Since we cannot always ensure that the device’s battery is large enough to maintain Tx power and send signals throughout the entire testing process after charging. Therefore, it is necessary to place an energy source in the chamber. Either CW node or reader node or a separate node/antenna can be used for energy signal provider.

Proposal 4:  It is necessary to place an energy source in the chamber. Either CW node or reader node or a separate node/antenna can be used for energy signal provider. The power level of energy source shall be specified.

As captured in TR 38.769, the ON-OFF pattern for the device should be known during the testing as there is no specified device behavior regarding when device wakes up after fully charging and how long can the device be in ON status after starting transmission. Even though device can be charged from reader in R2D synbol or during the gap between R2D packages, it is not possible to be known by TE during the testing. Therefore, it is necessary to know the ON-OFF pattern for the device and then energy source can provide the energy signal when device is in OFF symbol and energy source will have to be muted when device is in ON symbol.

Proposal 5: It is necessary to know the ON-OFF pattern for the device and energy source can provide the energy signal when device is in OFF symbol and energy source will have to be muted when device is in ON symbol. ON-OFF pattern can be based on the declaration.

## 2.2 Test methodology for Tx RF requirements of A-IoT device

RAN4 will specify power backscattering loss and occupied bandwidth for device 1 as the Tx RF requirements. The test procedure for backscattering loss could be given by following steps:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device.

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next testing point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

Proposal 6: Take the following test procedure as the baseline for backscattering losing testing.

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device.

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

## 2.3 Test methodology for Rx RF requirements of A-IoT device

It was agreed that reference sensitivity shall be specified for A-IoT device. Reference sensitivity is to verify the received performance of R2D link. While different from legacy NR device testing, it is not feasible to measure the reference sensitivity for R2D link via throughput metric, e.g., >95% of maximum throughput. One feasible way is to verify the device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna, i.e., reference sensitivity. The following test procedure is given as an example.

Figure 2: Example of timing relationship between R2D and D2R

Proposal 7: The reference sensitivity of device could be verified by measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. FFS on how to set the side condition.

With proposal 4, we have the following proposal for the test procedure of reference sensitivity.

Proposal 8: The following test procedure of reference sensitivity for A-IoT device 1 can be considered:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the number of times the DUT can correctly sent the response. FFS on how to specify the performance metric.

# 3  Conclusion

In summary, we have following proposals.

Proposal 1: The following test setup parameters shall be defined for A-IoT device OTA testing with the specific criteria:

- DMA2D: distance between measurement antenna and DUT

  - To follow the minimum range length for TRP and TRS OTA testing with 30cm quiet zone size defined in Table A.3.4-1 of TS 38.161

- DCW2D: distance between CW node and DUT

  - To follow the minimum range length for TRP and TRS OTA testing with 30cm quiet zone size defined in Table A.3.4-1 of TS 38.161

- IsoCW2M: Isolation between CW node and measurement antenna

  - To ensure that the received D2R signal in measurement antenna is X dB higher than CW signal leakage to measurement. FFS on X value

Proposal 2: Ripple Test for Quiet Zone specified in section 7.6 of TR 38.870 can be reused for AIoT device testing.

Proposal 3: The free space DUT orientation 1 specified in Table J.2-1 of TS 38101-2 can be assumed as the default position of device.

Proposal 4:  It is necessary to place an energy source in the chamber. Either CW node or reader node or a separate node/antenna can be used for energy signal provider. The power level of energy source shall be specified.

Proposal 5: It is necessary to know the ON-OFF pattern for the device and energy source can provide the energy signal when device is in OFF symbol and energy source will have to be muted when device is in ON symbol. ON-OFF pattern can be based on the declaration.

Proposal 6: Take the following test procedure as the baseline for backscattering losing testing.

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device.

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

Proposal 7: The reference sensitivity of device could be verified by measuring device’s respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. FFS on how to set the side condition.

Proposal 8: The following test procedure of reference sensitivity for A-IoT device 1 can be considered:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Energy source transmit the energy signal until energy storage is full and then turn off the energy source based on the known ON-OFF pattern.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the number of times the DUT can correctly sent the response. FFS on how to specify the performance metric.

# 4  References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

[2] R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC

[3] 3GPP TS 38.161 V18.1.0

[4] 3GPP TR 38.769 V2.0.1