3GPP TSG-RAN WG4 Meeting #114bis		               R4-2504066

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	Discussion on GNSS IDC issue for NR NTN

Agenda item:	7.29.2.1

Document for:	Approval

# Background

In response to RAN4's original LS in R4-2419898, RAN2 has replied with further questions on asking the periodicity and duration of IDC free time to help the UE read the GNSS. In this contribution, we provide our analysis of information needed to reply to RAN2.

# Discussion

In the LS replied from RAN2, it states [1]:

RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

Meanwhile, the current IDC-TDM solution is specified as below in 38.331:

Based on the information above, it is our understanding that RAN2 requires information on how often the GNSS measurement event would happen and how long the measurement gap would require.

## GNSS validity duration

GNSS validity duration was specified for IoT NTN since Rel-17, which is UE autonomously reported value that indicates the remaining GNSS validity duration in the UE.

In our view, the large range of validity duration specified for IoT NTN can also be valid for NR NTN as well, as it can accommodate the scenario under different speeds, etc. Therefore, it is proposed that those numbers of NR NTN be reused.

Proposal 1: the GNSS validity duration values of IoT NTN can be re-used for NR NTN.

## GNSS measurement duration

Measurement gap on GNSS has been introduced for IoT NTN since Rel-17, which ranges up to 31s (see table below). Per our understanding, such a long measurement gap was defined to accommodate a cold start of the GNSS receiver with IoT NTN devices, as the GNSS receiver will be totally shut off during the NTN transmission for the sake of power consumption. However, it is unclear if such a limitation would also be valid for NR NTN. Therefore, RAN4 may further discuss the reasonable value range for the measurement gap of GNSS.

Proposal 2: Take the value of the GNSS measurement gap defined for IoT NTN as a starting point and further discuss if large values like 31s would be needed.

# Conclusion

In this paper, we presented our view on the reply to RAN2 on GNSS IDC issue, where the following observations and proposals are made:

Proposal 1: the GNSS validity duration values of IoT NTN can be re-used for NR NTN.

Proposal 2: Take the value of the GNSS measurement gap defined for IoT NTN as a starting point and further discuss if large values like 31s would be needed.

# References

- R2-2501568 Response LS on simultaneous operation between GNSS and NR NTN